3GPP TSG-RAN WG2 Meeting #125	R2-2400807

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.4.1.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On the cell switch aspects in LTM

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This document discusses some of the remaining issues related to LTM cell switch procedure. Specifically, the paper focuses on the remaining MAC aspects, such as enabling RACH-based LTM, design of MAC CE CSC command and handling of the CFRA resources. We also discuss remaining open issues related to the LTM recovery procedure, semi-persistent CSI reporting on PUCCH and deactivation of active LTM TCI states. Additionally, the TA and UL grant validity at the time of the cell switch is also discussed.

# 2	Discussion

## 2.1	MAC aspects of LTM cell switch

### 2.1.1	LTM cell switch over RACH

In RAN2#124 we have agreed on the following:

Procedure assumptions: At LTM cell switch: UE uses TA from the network if it is provided (target TA or TA=0 or TA=same as src). If not provided and the UE is configured for UE based TA, then UE based TA is used. If the UE does not have/cannot derive the TA for target, the cell switch uses RACH. (FFS if more details need to be considered).

Current LTM cell switch procedure prevents NW from triggering RA procedure towards target cell when the UE-based TA estimation is configured. We have already agreed that the exact time when the UE performs TA measurements is left to UE implementation. This implies that the UE-based TA estimate may not always be accurate – furthermore, NW may have other reasons to issue access towards target cell over RACH (e.g., to acquire proper SSB for the target cell). It is even not possible to enforce RA with including CFRA preamble into the LTM CSC MAC CE. In the current TS 38.321  [1], the following procedure and structure for LTM CSC MAC CE is captured.

### 5.18.35 LTM Cell Switch Command The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

Fig. 1 LTM Cell Switch command MAC CE

Observation 1: Current LTM cell switch procedure does not allow NW to trigger LTM cell switch over RACH when UE-based TA estimation is configured.

This modelling does not only limit the usefulness of the UE-based TA estimation feature but effectively kills the whole feature since NW would not configure a feature which limits its options for decision making for the handover. Obviously, NW should have the call of enforcing target cell access through RACH in any scenario – and this would increase the usability of the UE based TA-estimation feature as well. This can be enabled by an indication in the LTM CSC MAC CE.

Proposal 1: NW can trigger LTM cell switch over RACH regardless of if the UE-based TA estimation is configured.

### 2.1.2	TAC in LTM CSC MAC CE

As can be seen from Fig. 1 above, the Timing Advance Command reserves 2 bytes from the LTM CSC MAC CE always regardless of whether LTM is performed over RACH. This makes little sense as the need for RACH can be determined from the first byte already (e.g., through C bit). When RACH is not indicated by the NW, the TAC can be present and the value FFF indicates the usage of the UE-based TA estimate (when configured) as in the current procedure. Furthermore, TAC field should be placed after the TCI state ID fields in the MAC CE.

Proposal 2: The TAC field in the LTM CSC MAC CE is only present when NW does not indicate LTM cell switch over RACH, otherwise it is absent. TAC field is also placed after the TCI state ID fields in the MAC CE.

### 2.1.3	TCI state ID(s) in LTM CSC MAC CE

As can be seen from Fig. 1 above, separate TCI state ID fields are present in the current LTM CSC MAC CE where the other is 7 bits long (DL/joint) and the other 8 bits long (UL) with the following statement: “The most significant bits of UL TCI state ID are considered as reserved bits and the remainder 6 bits indicate the TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5].”. This makes no sense and is not how MAC CEs are modelled, instead, reserved bit(s) shall be used.

Proposal 3: Two R bits are included as the rightmost bits in the byte of UL TCI state ID.

Another thing with respect to TCI state ID(s) is that while the MAC procedure includes an option the TCI state ID(s) are not included in the LTM CSC MAC CE, the actual MAC CE part of the specification does not allow this:

### 5.18.35 LTM Cell Switch Command The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: (…) 3> if TCI state information is included: 4> consider the SSB corresponding to the indicated TCI state as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 4> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

It seems beneficial to allow NW from not including the TCI state information in the LTM CSC MAC CE, but the UE will apply the TCI state information activated, e.g., through the Candidate Cell TCI States Activation/Deactivation MAC CE. Or if there is no TCI information available for the indicated target, the UE applies the SSB acquired through the RA procedure as long as NW activates TCI states in the target cell (similarly to legacy procedure).

Proposal 4: Explicit indication is introduced for NW to indicate if TCI State ID field(s) are present in the LTM CSC MAC CE.

### 2.1.4	MAC reset

Currently, the MAC procedure for LTM cell switch states the following:

### 5.18.35 LTM Cell Switch Command The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

It is hard to understand what the “as requested by upper layers” refers to here. Naturally, the MAC reset is triggered by the RRC procedure, but one cannot write specification text such that an operation is known to take place before an action. Hence, this condition needs to be changed to such that the operation is performed when upper layers request the MAC reset.

Proposal 5: Change the MAC reset operation in the LTM cell switch procedure to “upon request by upper layers, perform the MAC reset as specified in clause 5.12:”.

### 2.1.5	RACH-less LTM cell switch and CFRA resources

In the current handover procedure over RACH, when the UE completes the RA procedure, it discards any CFRA resources (except CFRA for BFR) so that the CFRA resources are not hold up by the UE and used, e.g., for scheduling request purposes in the target cell:

### 5.1.6 Completion of the Random Access procedure Upon completion of the Random Access procedure, the MAC entity shall: 1> discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any; 1> flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer. Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall: 1> indicate the successful completion of the Random Access procedure to the upper layers.

Since the reconfigurationWithSync may include RACH-ConfigDedicated with CFRA resources for the LTM target cell, these CFRA resources would not be discarded in case of RACH-less LTM cell switch since no RA procedure is enforced. Hence, the CFRA resources would remain in UE configuration and UE would utilize those CFRA resources for any RA procedure trigger in the target cell. This is not intentional and not expected by the NW.

Proposal 6: UE discards the CFRA resources configured for the LTM target cell if RACH-less cell switch is performed.

## 2.2	LTM Recovery Procedure

In RAN2#123 we have agreed the following:

Upon an LTM cell switch failure (i.e., supervision timer expiry) or RLF, fast recovery similar to CHO: a) UE performs cell selection. b) If selected cell is an LTM candidate cell, UE performs RACH-based LTM cell switch on the selected cell (network-controlled). c) If selected cell is not an LTM candidate cell, UE transmits RRC re-establishment request.

The LTM recovery procedure is triggered when the NW triggers LTM cell switch with the LTM CSC MAC CE and the UE fails in connecting to the target cell. As described also by the above agreement, in this case, the UE would start with performing the cell selection procedure. Then, if the selected cell is an LTM candidate, the UE would perform RACH-based LTM cell switch procedure towards that cell. If the LTM recovery process is successful, the UE keeps the LTM configurations that were initially given by the NW.

The UE may have been given pre-provisioned CFRA resources for the selected cell used for LTM recovery. Hence, when RA procedure is triggered upon the cell switch procedure, UE applies the CFRA resources in the RA procedure which may be problematic and unexpected by the NW since the NW indicated LTM cell switch procedure towards different target cell. NW may have already initiated reassigning the CFRA resources for another UE. Furthermore, it is also questionable what happens with the CFRA resource possibly given in the LTM CSC MAC CE upon LTM cell switch failure and LTM recovery procedure trigger.

Observation 2: Further specification effort is needed to indicate to the UE which resources it should use to perform random access to the target cell.

A simplified illustration of the problem is depicted in the below figure.

Fig. 2 CFRA preamble collision during LTM recovery procedure.

When the UE is configured with LTM recovery procedure, the UE should know which resources it needs to use for this purpose. From this viewpoint, two options are possible: i) UE uses indicated CFRA resources; ii) UE uses CBRA resources. To enable CFRA-based recovery the UE neds to know which preambles it can use for this purpose. In case, the simplest approach would be for the NW to indicate to the UE which CFRA resources can be used for recovery.

Proposal 7: The NW indicates to the UE which CFRA resources can be used for LTM recovery procedure.

## 2.3 Semi Persistent CSI Reporting on PUCCH

In [2], RAN1 identified an issue concerning the lack of support for MAC-CE signaling for semi-persistent L1 reporting on PUCCH for LTM. Consequently, RAN1 has requested RAN2 to address and resolve this issue.

Semi-persistent reporting on PUCCH is supported for both LTM and legacy L1 reporting. For the legacy semi-persistent CSI reporting on PUCCH, a MAC-CE is used for activation/deactivation of one or more semi-persistent reports on PUCCH configurations by indicating CSI report Config ID or IDs to be activated or/and deactivated [see 6.1.3.16 in 38.321]. With an independent configuration of LTM CSI reporting, the MAC-CE command should indicate  whether the activation/deactivation is  for LTM semi-persistent reporting on PUCCH, legacy SP CSI reporting. Therefore, it is needed to provide signalling support to indicate the UE whether the activation/deactivation of one or more SP reporting on PUCCH configuration applies for LTM reporting configuration or serving cell reporting configuration (e.g., legacy reporting used for serving cell BM and CSI).

Observation 3: With an independent configuration of LTM CSI reporting, the MAC-CE command should indicate  whether the activation/deactivation is  for LTM semi-persistent reporting on PUCCH, legacy SP CSI reporting.

To indicate whether the activation/deactivation is for the legacy SP CSI reporting or for the LTM SP CSI reporting a reserved bit in the activation/activation MAC CE could be used.

Proposal 8: Utilize a Reserved bit in the SP CSI reporting on PUCCH Activation/Deactivation MAC CE to indicate whether the MAC CE is for the legacy SP CSI reporting or for the LTM SP CSI reporting. Adopt the text proposal from Appendix A.1 in TS 38.321.

## 2.4 Deactivation of active LTM TCI state(s)

In Rel-17 unified TCI activation/deactivation, once the (unified) TCI state has been activated, the deactivation of the previously activated TCI state is done by activating another TCI state. Thus, the deactivation is implicit if the latest MAC CE providing the activation command does not include the previously activated TCI state.

For LTM, RAN1 has agreed to allow TCI state activation before cell switch for one or more than one candidate cell.

RAN1# 114 Agreement

TCI state activation by MAC CE before cell switch command for one or more than one candidate cells is allowed.

The Candidate Cell TCI States Activation/Deactivation MAC CE can activate/deactivate TCI states of only one candidate cell at a time. With such a design, the TCI activation for one candidate cell cannot be used for TCI deactivation of another candidate cell unless Candidate Cell TCI States Activation/Deactivation MAC CE is allowed to activate TCI states from multiple candidate cells with the same command.

Observation 4: With the current Candidate Cell TCI States Activation/Deactivation MAC CE, it is not possible to deactivate activated TCI states for candidate cells not indicated in the MAC CE.

A UE may have a limited capability in terms of number of active LTM TCI states could be managed simultaneously across multiple candidate cells. Especially when TCI activation is triggered for more than one candidate cell, in some scenarios, it may cause the UE unnecessarily monitor TCI states for candidate cells which may no longer relevant for cell switch purpose.

Observation 5: Keep the UE tracking the timing of the TCI states of candidate cells which will not be selected as the target cell may cause the UE unnecessarily monitor TCI states that are no longer relevant for cell switch purposes.

In order minimize the monitoring of unnecessary activated TCI states, TCI deactivation should be supported. There can be different approaches to achieve that. One method is to modify the current MAC CE to allow activation/deactivation for more than one candidate cells and use the legacy method of deactivating TCI states which are not included in the latest MAC CE. Another method may include defining a separate deactivation MAC CE to deactivate one or more TCI states of a set of candidate cells or a timer-based approach, where a timer can be configured for the supervision of already activated TCI states (for LTM).

Proposal 9: Support of deactivation of LTM TCI states for a candidate cell (or set of candidate cells).

## 2.5 	TA Value and UL grant validity at the time of the cell switch

In previous meetings it was agreed that the TA value will be provided to the UE at the time of the cell switch in the MAC CE to trigger cell change. TA validity, till the moment that it is shared with the UE is maintained by the network and the serving DU is responsible to ensure that the TA s shared with the UE is valid. Upon receiving the cell switch command, the UE would need to maintain the TAT. Additionally, the UE (possibly) will perform RTD measurements for UE based TA calculation before the reception of the cell switch command. In such cases the UE needs to maintain the TA validity locally.

Proposal 10: The UE can start a timer to maintain the validity of TA at the time of the RTD calculation for the UE based TA estimation.

Following the above approaches T304 timer and TAT run independently. This is because T304 is a timer that is used to detect radio link related issues during the handover procedure whereas TAT is a timer that is used to identify if the acquired TA is still valid or not.

Observation 6: It is possible that TA alignment timer will expire before the T304 expiration.

Observation 7: If TAT expires before the T304 the UE has to wait till T304 will expire as well in order to act.

For the UE to act in time, the following options exist:

- Option 1: UE fallback to source cell after timer to maintain the validity of TA expiry of the target cell of handover (if source TAT has not expired but target TAT has expired). The uplink resource for fallback is pre-configured at source for the UE to indicate the return. In this case the UE does not wait until T304 expiry and connects back to source cell to avoid failure or longer interruption time.

- Option 2: UE fallback to RACH based target cell access after timer to maintain the validity of TA expiry of the target cell of handover or based on radio-condition of the Reference Signal (SSB index) used as Quasi-co location (QCL) reference for PUSCH transmission (for using RACH config given in LTM RRC Reconfiguration). The UE does not wait until T304 expiry and initiates the RACH procedure that may have better coverage, e.g., switch to a better SSB/beam.

- Option 3: UE declares early failure and initiates re-establishment/recovery after the timer to maintain the validity of TA expiry of the target cell of handover. Thus, the UE does not wait until T304 expiry and initiates the re-establishment (or recovery if configured). Such approach may be a good approach in case fallback mechanisms are not applicable.

Proposal 11: RAN2 should discuss on the above options and decide which is the preferred way to be followed.

Similarly, to the case with the TA validity expiration, the UE should be able to receive the UL grant with PDCCH of the target cell. However, the amount of time the UE should wait for the UL grant should be guarded. This can be done either using a timer, or using a certain number of PDCCH occasions, for not waiting for the T304 to expire and perform recovery. In case the condition for receiving UL grant (i.e., timer or counter) expires, the UE should not perform re-establishment/recovery, but it should fallback to RACH based access to the target cell.

Proposal 12: The UE should be configured with a timer or a counter of PDCCH occasions to guard the time to access the target cell using LTM RACH-less access. This configuration can be part of the RRC.

Proposal 13: The UE should perform RACH based access to the target cell, once the conditions to guard the cell change process for the reception of the UL grant expires.

# 4	Conclusion

This document has made the following observations:

Observation 1: Current LTM cell switch procedure does not allow NW to trigger LTM cell switch over RACH when UE-based TA estimation is configured.

Observation 2: Further specification effort is needed to indicate to the UE which resources it should use to perform random access to the target cell.

Observation 3: With an independent configuration of LTM CSI reporting, the MAC-CE command should indicate whether the activation/deactivation is for LTM semi-persistent reporting on PUCCH, legacy SP CSI reporting.

Observation 4: With the current Candidate Cell TCI States Activation/Deactivation MAC CE, it is not possible to deactivate activated TCI states for candidate cells not indicated in the MAC CE.

Observation 5: Keep the UE tracking the timing of the TCI states of candidate cells which will not be selected as the target cell may cause the UE unnecessarily monitor TCI states that are no longer relevant for cell switch purposes.

Observation 6: It is possible that TA alignment timer will expire before the T304 expiration.

Observation 7: If TAT expires before the T304 the UE has to wait till T304 will expire as well in order to act.

And proposed the following:

Proposal 1: NW can trigger LTM cell switch over RACH regardless of if the UE-based TA estimation is configured.

Proposal 2: The TAC field in the LTM CSC MAC CE is only present when NW does not indicate LTM cell switch over RACH, otherwise it is absent. TAC field is also placed after the TCI state ID fields in the MAC CE.

Proposal 3: Two R bits are included as the rightmost bits in the byte of UL TCI state ID.

Proposal 4: Explicit indication is introduced for NW to indicate if TCI State ID field(s) are present in the LTM CSC MAC CE.

Proposal 5: Change the MAC reset operation in the LTM cell switch procedure to “upon request by upper layers, perform the MAC reset as specified in clause 5.12:”.

Proposal 6: UE discards the CFRA resources configured for the LTM target cell if RACH-less cell switch is performed.

Proposal 7: The NW indicates to the UE which CFRA resources can be used for LTM recovery procedure.

Proposal 8: Utilize a Reserved bit in the SP CSI reporting on PUCCH Activation/Deactivation MAC CE to indicate whether the MAC CE is for the legacy SP CSI reporting or for the LTM SP CSI reporting. Adopt the text proposal from Appendix A.2 in TS 38.321.

Proposal 9: Support of deactivation of LTM TCI states for a candidate cell (or set of candidate cells).

Proposal 10: The UE can start a timer to maintain the validity of TA at the time of the RTD calculation for the UE based TA estimation.

Proposal 11: RAN2 should discuss on the above options and decide which is the preferred way to be followed.

Proposal 12: The UE should be configured with a timer or a counter of PDCCH occasions to guard the time to access the target cell using LTM RACH-less access. This configuration can be part of the RRC.

Proposal 13: The UE should perform RACH based access to the target cell, once the conditions to guard the cell change process for the reception of the UL grant expires.

# References

[1] TS 38.321 v18.0.0

[2] R1-2312642, LS on MAC CE to activate/deactivate semi-persistent PUCCH report for LTM, RAN1 LS to RAN2, 3GPP TSG RAN WG1 #115, November 13-17, 2023

# Annex A

## A.1 Text proposal related to SP CSI Reporting on PUCCH

Capture the following TP in 38.321.

6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	S/L: This field indicates the CSI report configuration referred by Si. If this field is set to 0, the Semi-Persistent CSI report configuration referred by Si is csi-ReportConfigToAddModList. If this field is set to 1, the Semi-Persistent CSI report configuration referred by Si is ltm-CSI-ReportConfigToAddModList-r18.

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList or ltm-CSI-ReportConfigToAddModList-r18, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the Si field is set to 0 to additionally indicate that all SubConfigurations within csi-ReportSubConfigList shall be deactivated;

-	R: Reserved bit, set to 0.

NOTE:	If a Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the corresponding Si field is not set to 1.

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE