3GPP TSG-RAN WG2 Meeting #125	R2-2400815

Athens, Greece, 26 February - 1 March 2024

Title:	Early RACH for inter-DU LTM

Source:	Huawei, HiSilicon

Agenda item:	7.4.1.3.1

Document for:	Discussion and Decision

# Background

This paper discusses early RACH for inter-DU LTM

# 2	Discussion

Figure 1: Early TA acquisition for inter-DU LTM

The procedure of early TA acquisition is shown in figure 1:

1.	the UE performs RACH to the candidate target cell indicated in PDCCH order, using the indicated preamble;

2.	the DU controlling the candidate target cell determines the TA and provides it to the CU ;

3	the CU provides the TA to the source DU;

4	In the LTM cell switch command, the source DU provides the TA.

Figure 2: A cell is an LTM candidate for multiple source cells

r

One possible scenario is that a cell is the target cell of multiple source cells as shown in figure 2, and "Source cell1", "Source cell2", "Source cell3" and "Target cell" are all controlled by different DUs.

RAN2 previously agreed that the same early RACH configuration is used regardless of the source cell, so:

-	If UE1 is in "Source cell1" and can execute LTM to "Source cell2" or "Source cell 3", UE1 will use the same RACH configuration for the "Target cell".

-	If UE2 is also in "Source cell1" and can also execute LTM to "Source cell2" or "Source cell 3", UE2 will use the same RACH configuration for the "Target cell".

If the network provides a different early RACH configuration for the "Target cell" for every UE, there will not be any collision, but allocating different early RACH resources for the same cell for different UEs would be very costly for network resources. At RAN2#121bis, it was agreed:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

This agreement is why the UE uses information in PDCCH order to determine the RA preamble, while the network allocates the same early RACH configuration.

Observation 1: For different UEs, the "early RACH configuration" towards the same target cell is the same.

Therefore, if UE1 is in "Source cell1" controlled by DU1 and UE2 is in "Source cell2" controlled by DU2, without coordination between "Source cell1" and "Source cell2", DU1 and DU2 may simultaneously send PDCCH order to DU1 and to DU2, indicating the same RO(s) and the same preamble, resulting in RA collision.

Observation 2: If DUs that can send PDCCH order to trigger early RACH to the same LTM candidate cell, controlled by another DU, can indicate the same preamble and RO, without coordination, RA collision may occur.

In addition, if the DUs that can send PDCCH order to trigger early RACH to the same target cell, controlled by another DU, can indicate the same preamble and RO, unless the DU informs the CU before triggering early RACH, the CU cannot know which DU initiated early RACH.

Observation 3: If the DUs that can send PDCCH order to trigger early RACH to the same target cell, controlled by another DU, can indicate the same preamble and RO, the CU cannot know which DU initiated early RACH, unless the DU informs the CU before triggering early RACH.

In order to avoid the above problems, a simple solution would be that, when a DU prepares an LTM candidate cell with an early RACH configuration, the DU allocates a different set of preambles to different DUs to trigger RA using different preambles.

While RAN3 is responsible for network signalling, only RAN2 can agree that the above solution is feasible.

Proposal 1: The DU that provides an early RACH configuration to another DU can also provide a dedicated (i.e. specific to the other DU) range of RA preambles so that when receiving a RA preamble from a UE, the target cell can determine the DU to which to forward TA information (i.e. the one that has this preamble range).

Proposal 2: Inform RAN3 about the RAN2 agreement, ask RAN3 to define the necessary signalling and/or inform RAN2 is any issue or question.

# 3	Conclusion

We made the following observations:

Observation 1: For different UEs, the "early RACH configuration" towards the same target cell is the same.

Observation 2: If DUs that can send PDCCH order to trigger early RACH to the same LTM candidate cell, controlled by another DU, can indicate the same preamble and RO, without coordination, RA collision may occur.

Observation 3: If the DUs that can send PDCCH order to trigger early RACH to the same target cell, controlled by another DU, can indicate the same preamble and RO, the CU cannot know which DU initiated early RACH, unless the DU informs the CU before triggering early RACH.

Accordingly, we have the following proposals:

Proposal 1: The DU that provides an early RACH configuration to another DU can also provide a dedicated (i.e. specific to the other DU) range of RA preambles so that when receiving a RA preamble from a UE, the target cell can determine the DU to which to forward TA information (i.e. the one that has this preamble range).

Proposal 2: Inform RAN3 about the RAN2 agreement, ask RAN3 to define the necessary signalling and/or inform RAN2 is any issue or question.