3GPP TSG-RAN WG2 Meeting #125	R2-2400847

Athens, Greece, 26th February – 1st March, 2024

Agenda item:	7.6.5

Source:	Samsung

Title:	Uplink transmission extension and related RILs

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN#94-e the WID for Rel-18 IoT NTN was agreed [1] and in RAN#98-e it was updated [2]. The updated objectives of the WID is the following:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] - NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements. 4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4]. 4.1.3 Further enhancement to discontinuous coverage - Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2. [RAN2, RAN3].

This contribution addresses the feature that has so far been known as Uplink transmission extension, addressing some key issues as well as RILs C600, Z364and V500.

# Discussion

- Uplink transmission extension enhancement was agreed in RAN1#113:

Agreement

From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition.

RAN1 will decide further details of the above.

- In our understanding, the characteristics of the feature as of RAN2#125 is as follows:

- The feature allows the UE to remain in RRC_CONNECTED after the GNSS position is out-of-date. This is because in baseline Rel-17 IoT NTN, the UE shall always go RRC_IDLE upon GNSS position being out-of-date.

- The synchronization is maintained by the network sending MAC CEs to update the Timing Advance. This is in contrast to the UE autonomously pre-compensating for the Timing advance, as is done when the GNSS position is valid, such as when the UE has recently connected.

- There is nothing that stops the UE from receiving downlink transmissions during this duration.

## Name of the feature

- In RAN2#124 the updated RRC parameter list was sent to RAN2. As part of that list, the parameters relating to the feature “uplink transmission extension” was included [R1-2312697]:

| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for eMTC | ul-Transmission-ExtensionValue | Indicates the duration after original GNSS validity duration expires within which UL transmission is allowed. Value in number of sub-frames, value sf500 corresponds to 500 sub-frames, sf750 corresponds to 750 sub-frames and so on. | {sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240, spare1} |
| --- | --- | --- | --- | --- |
| IoT_NTN_enh-Core | UL transmission after original GNSS validity duration expires for NB-IoT | ul-Transmission-ExtensionValue-NB | Indicates the duration after original GNSS validity duration expires within which UL transmission is allowed. Value in number of sub-frames, value sf500 corresponds to 500 sub-frames, sf750 corresponds to 750 sub-frames and so on. | {sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240, spare1} |

- The current feature name is “uplink transmission extension” which makes it sound as if only uplink transmissions are allowed, and that the transmission duration has somehow be extended.

- Note that RAN2 has more responsibility of maintaining good naming-conventions that correspond to what the network and UE is expected to do during a feature.

Proposal 1: The feature should be re-named to reflect the behavior of the network and the UE.

Proposal 2: Feature should be named “GNSS invalidity duration”.

## New MAC CE or not

- In the last meeting, RAN2 received an LS from RAN1 during the RAN2 meeting in R1-2312696 [X]:

RAN1 has discussed the end of duration X for improved GNSS operations and agreed the following:

Agreement From RAN1 perspective, the start time of duration X is at the point where original GNSS validity duration expires - when timeAlignmentTimer is infinity, the end of X is at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is reset with length equal to Y every time when a MAC CE (to be defined by RAN2) is received - Note 1: It is up to RAN2 to decide whether the MAC CE is the legacy TAC or a new TAC or a new MAC CE. - Note 2: It is up to RAN2 to implement the above behaviour based on new timer, existing timer, or by extending GNSS validity. - Send an LS to RAN2

- The LS left the discussion on whether to introduce a new MAC CE or not to RAN2. Consequently, RAN2 made the following agreement:

5. 	For the case when timeAlignmentTimer is infinity, a (legacy/new) MAC CE is introduced/used to reset ULTransmissionExtentionTimer with length equal to Y)

- The expected use of the MAC CE is to update the Timing advance of the UE and continue to make sure that the UE stays in RRC_CONNECTED.

- From our point of view, the benefits of introducing a new MAC CE is that it is a cleaner solution and since legacy MAC CE used when the UE has a valid GNSS position, then it would make it more clear that a new MAC CE is used. The benefits of re-using the legacy MAC CE is that a new MAC CE does not need to be introduced and tested, lowering the barrier to implement the feature.

- Since the MAC CE will be used in a similar manner as for non-NTN when the GNSS position is no longer valid, we think that the legacy MAC CE can be reused.

Proposal 3: Legacy MAC CE reused to update TA during the period when GNSS is not valid.

## [C600] – Timer modelled in MAC or RRC

- RIL C600 has the following:

| C600 | Better to maintain T390 as a MAC timer than an RRC timer | It is seen that the T390 is operated based on TAT status and UL transmission extension update (i.e application of a TA command MAC CE). Therefore, comparing with maintaining T390 as an RRC timer, T390 is better maintained as a MAC timer which can reduce unnecessary cross-layer interaction. We’ll bring a contribution to address this issue. | (Huawei-Lili) I am open on this issue. It has been brought up in running CR review after RAN2#124, but kept in 331 since no clarification was added in the MAC spec. |
| --- | --- | --- | --- |

- In RAN2#124 it was discussed how to implement the timer that RAN1 refers to as ULTransmissionExtensionTimer, and the following was captured in Chairman notes [X]:

Subsequently, for the value of this timer, RAN2 will down select from the two options below:

•	An existing timer

•	A new timer

Shoud the timer for duration X be specified in MAC or RRC?

- Subsequently, the during the RRC CR review, a new timer was implemented in RRC to model the behaviour.

- Whether to implement the behaviour in MAC or RRC we can see the following arguments:

- Implemented in MAC Pros. Since the MAC CE is received in MAC, the update can be modelled entirely in MAC.

- Implemented in MAC Cons. The interaction between the uplink transmission extension and other procedures requires indications between MAC and RRC. Examples of this are:

  - RRC to MAC: Indicate to MAC to start the timer after GNSS out-of-date

    - This is already implemented in RRC as below:

1>	else if ul-TransmissionExtensionEnabled is configured:

2>	if timeAlignmentTimer is not configured to be infinity:

3>	start timer T390 with the timer value set to remaining time of timeAlignmentTimer;

3>	restart timer T390 upon indication from lower layers that an UL transmission extension update is applied, with the timer value set to remaining time of timeAlignmentTimer;

2>	else:

3>	start timer T390 with the timer value set to ul-TransmissionExtensionValue;

3>	restart timer T390 upon indication from lower layers that an UL transmission extension update is applied;

  - MAC to RRC: Indicate expiry of the timer to RRC to move to RRC_IDLE or start GNSS measurement

    - This is already implemented in RRC as below:

2>	if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layer:

3>	if gnss-AutonomousEnabled is configured:

4>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

3>	else:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

  - RRC to MAC: Need to check whether timer has expired if GNSS measurement has failed

    - This is already implemented in RRC as below:

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 has expired:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

- Implemented in RRC Pros. The interaction with other RRC procedures is more simple.

- Implemented in RRC Cons. There needs to be RRC-MAC interaction every time a MAC CE is received.

- We understand the arguments to implement the timer in MAC given that there needs to be an indication upon receiving MAC CE, but given the fact that the feature interacts with other RRC procedures, we think that it is still more efficient to model the timer in RRC. The number of MAC<->RRC interactions for implementing it in MAC is 3, while implementing it in RRC only requires a single MAC<->RRC interaction. This can be seen in Figure 1. Since these procedures are rather complicated, deciding a good modelling may save RAN2 a lot future correction work.

Figure 1. a) RRC<->MAC interactions when implementing it as in current spec , b) RRC<->MAC interactions when implementing the timer in MAC.

Proposal 4: Reject RIL C600.

Proposal 5: The period when the GNSS is not valid is modelled in RRC (as T390 in 36.331 V18.0.0), with MAC->RRC indication upon receiving MAC CE when the timer is running.

## Other RRC aspects

### [Z364] – Stopping timer upon receiving GNSS position fix

- RIL Z364 has the following:

| Z364 | It’s also possible that T390 is running in this branch for NW triggered GNSS measurement. T390 needs to be stopped. Moreover, we understanding the reference to TS 36.213 is not needed as GNSS measurement is not performed by Physical layer. | 1> if an indication to perform GNSS measurement is received from lower layers: 2> perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23]; 2> stop timer T390, if running. | (Huawei-Lili) On stopping T390, it is already captured in the stop condition of T390 in clause 7.3.1 that "Upon leaving RRC_CONNECTED, or reception of network triggered GNSS measurement". On reference to R1 spec, this is because the start time of the network triggered GNSS measurement gap is defined in 36.213 (R1-2310771). |
| --- | --- | --- | --- |

- The RIL is about whether to stop the T390 timer when an indication to perform GNSS position fix is triggered by the network. The rapporteur indicates that it is already described in Section 7.3.1. However, in our understanding, the section 7.3.1 is an informative description of the timers, compared to how they are used in the procedural text in Section 5. Thus we think that the aspect of RIL Z364 on stopping the T390 timer is reasonable. However, the aspect referring to 36.213 shall not be implemented.

Proposal 6: The timer T390 is stopped upon receiving indication to perform GNSS position fix from lower layers in Section 5.5.9 according to RIL Z364.

### [V500] – Configuring T390

- RIL V500 has the following:

| V500 | All these GNSS parameters are used in the RRC layer, it is not for MAC function. We fail to why we put the parameters into the MAC configuration. | We suggest modelling all these parameters in the RadioResourceConfigDedicated (i.e. child field of RadioResourceConfigDedicated). | (Huawei-Lili) The gap length of autonomous gap is provided via MAC CE. So at least gnss-AutonomousEnabled-r18 has something to do with the MAC layer. About UL extension, there were other RIL proposing to put the UL extension behaivour (i.e. T390) to MAC layer and is still pending. |
| --- | --- | --- | --- |

- In proposal 3 we propose that the timer is modelled in RRC, so with this in mind RIL V500 seems to be a reasonable suggestion. If there was an agreement to model the behaviour in MAC, then subsequently it makes sense to implement the configuration in MAC.

Proposal 7: Agree RIL V500 – i.e uplink transmission extension configuration is implemented in RadioResourceConfigDedicated.

# Conclusion

In this contribution we have the following observations and proposals:

Proposal 1: The feature should be re-named to reflect the behavior of the network and the UE.

Proposal 2: Feature should be named “GNSS invalidity duration”.

Proposal 3: Legacy MAC CE reused to update TA during the period when GNSS is not valid.

Proposal 4: Reject RIL C600.

Proposal 5: The period when the GNSS is not valid is modelled in RRC (as T390 in 36.331 V18.0.0), with MAC->RRC indication upon receiving MAC CE when the timer is running.

Proposal 6: The timer T390 is stopped upon receiving indication to perform GNSS position fix from lower layers in Section 5.5.9 according to RIL Z364.

Proposal 7: Agree RIL V500 – i.e uplink transmission extension configuration is implemented in RadioResourceConfigDedicated.

# Reference

RP-213596, New WID on IoT NTN enhancements, Mediatek Inc., RAN#94-e, December 2021

RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022