3GPP TSG-RAN WG2 Meeting #125	R2-2400868

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.5.1

Source:	Nokia (Rapporteur)

Title:	XR Stage 2 Open Issues

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Information

# 1	Introduction

This contribution echoes the open issues that were discussed for XR in 38.300 on the RAN2 reflector.

# 2	Open Issues

In total, four open issues were identified during the email discussion:

1.	PSIHI is now defined as “PDU Set Integrated Handling Information (PSIHI): indicates whether all PDUs of the PDU Set are needed for the usage of PDU Set by application layer, as defined in TS 23.501 [3]”. Given this definition, the expression “When the PSIHI is set for a QoS flow” is not well defined and “When the PSIHI is set..” should be changed to “When the PSIHI indicates that all PDUs of the PDU Set are needed..”

2.	Current text on PSI-based discarding assumes that the importance can be a per-SDU property. However, in the PDCP spec, the shorter timer is started if, among other conditions, “the PDCP SDU belongs to a low importance PDU Set”. This is a misalignment between the stage-2 and stage-3 specs. It is thus recommended that the Stage-2 text on PSI-based discarding in 16.15.4.2.2 is aligned with PDCP, and thus always based on PDU sets.

3.	Current text on retransmission-less CG operation is inaccurate. It reads “Configured grants may be configured without the need for the UE to monitor possible UL retransmissions…”. However, the UE monitors grants, not UL retransmissions. Also, the UE may have to wake up for something else and these retransmissions are just the ones of the CG. A better wording would therefore be : “Configured grants may also be configured such that the UE does not need to wake-up to monitor possible UL retransmissions grants of the configured grants…”

4.	The abilities to identify PDU sets and PSI are combined in both 38.300 and 38.331, contradicting what 38.306 describes.

Since issues #1, #2 and #3 are straightforward, they are addressed in a rapporteur CR [R2-2400477].

Issue #4 being a bit more complex, requiring changes to multiple specifications, and also flagged in the RIL (as H399), it is addressed in a separate contribution [R2-2400389].