3GPP TSG-RAN WG2 Meeting #125	R2-2400869

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda Item	: 7.7.3 (NR_NTN_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on configuration of ntn-cg-RACH-less-RetransmissionTimer

Document for	: Discussion and Decision

Introduction

In this contribution, we show our view on that ntn-cg-RACH-less-RetrnasmissionTimer should be shorter than the drx-HARQ-RTT-TimerUL-NTN.

Discussion

According to the current specification, in TN, when the DRX is configured, the UE continuously monitors the PDCCH during the RACH-less handover.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network.

In NTN, if the DRX is configured, the UE does not continuously monitor the PDCCH during the RACH-less handover due to long RTT, and the UE monitors the PDCCH based on the DRX operation.

In the following legacy DRX operation for RACH-less handover in NTN, the UE starts the HARQ-RTT-TimerUL-NTN after transmitting uplink initial transmission. Then, if the drx-HARQ-RTT-TimerUL-NTN expires, the UE starts the drx-RetransmissionTimerUL and monitors the PDCCH in order to receive a new uplink grant on the same HARQ process used for uplink initial transmission or a downlink assignment for new transmission. Then, the UE determines the RACH-less handover completion.

However, there is a case where the drx-RetransmissionTimerUL is not started due to the restart of the drx-the HARQ-RTT-TimerUL-NTN if the cg-RACH-less-RetransmissionTimer is larger than the HARQ-RTT-TimerUL-NTN.

For example, during RACH-less handover, the UE transmits the uplink initial transmission and starts the cg-RACH-less-RetransmissionTimer and drx-HARQ-RTT-TimerUL-NTN. If the cg-RACH-less-RetransmissionTimer expires, the UE performs the retransmission and restarts the drx-HARQ-RTT-TimerUL-NTN. In this case, the UE cannot start the drx-RetransmissionTimerUL to monitor the PDCCH because the HARQ-RTT-TimerUL-NTN cannot expire. In addition, since the PDCCH is not monitored, the UE cannot receive a new uplink grant on the same HARQ process used for uplink initial transmission or a downlink assignment for new transmission while running the drx-RetransmissionTimerUL. It would cause the delay of the RACH-less handover completion.

Observation 1. If the cg-RACH-less-RetransmissionTimer is larger than the HARQ-RTT-TimerUL-NTN, it would cause the delay of the RACH-less handover completion.

In order to avoid the above case, the simple way is to configure that the cg-RACH-less-RetransmissionTimer is always shorter than the HARQ-RTT-TimerUL-NTN. However, according to the current specification, there is no restriction on this. Thus, we propose that the network configures that the cg-RACH-less-RetransmissionTimer is always shorter than the HARQ-RTT-TimerUL-NTN.

Proposal 1. In RRC specification, capture the restriction on that the cg-RACH-less-RetransmissionTimer is always shorter than the HARQ-RTT-TimerUL-NTN.

For the above proposal, we provide the text proposal in section 4.

Conclusion

Based on the above discussion, we made following proposals and observations.

Observation 1. If the cg-RACH-less-RetransmissionTimer is larger than the HARQ-RTT-TimerUL-NTN, it would cause the delay of the RACH-less handover completion.

Proposal 1. In RRC specification, capture the restriction on that the cg-RACH-less-RetransmissionTimer is always shorter than the HARQ-RTT-TimerUL-NTN.

Text proposal for 38.331

CG-NTN-RACH-Less-Configuration field descriptions

ntn-cg-RACH-less-RetransmissionTimer Indicates the initial value of the configured grant retransmission timer used for the initial uplink transmission of RACH-less handover (see TS 38.321 [3]) in multiples of periodicity. The value of this timer is always configured shorter than drx-HARQ-RTT-TimerUL plus UE-gNB RTT.

ntn-DMRS-Ports Indicates the set of DMRS ports for SSB to PUSCH mapping (see TS 38.213 [13]). The first (left-most / most significant) bit corresponds to DMRS port 0, the second most significant bit corresponds to DMRS port 1, and so on. A bit set to 1 indicates that this DMRS port is used for mapping.

ntn-NrofDMRS-Sequences Indicates the number of DMRS sequences for SSB to PUSCH mapping (see TS 38.213 [13]).

ntn-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for the pre-allocated uplink grant as specified in TS 38.321 [3].

ntn-SSB-PerCG-PUSCH The number of SSBs per pre-allocated uplink grant PUSCH (see TS 38.213 [13]). Value one corresponds to 1 SSBs per pre-allocated uplink grant PUSCH, value two corresponds to 2 SSBs per pre-allocated uplink grant PUSCH and so on.

ntn-SSB-Subset Indicates SSB subset for SSB to CG PUSCH mapping within one CG configuration. If this field is absent, UE assumes the SSB set includes all actually transmitted SSBs.