# 3GPP TSG RAN WG2 Meeting #125      	                           R2-2400921
Athen, Greece, 26th Feb. – 1st March, 2024

Agenda item:	7.12.2.2

Source:	Apple

Title:	RRC open issues on mobile IAB

WID/SID:	NR_mobile_IAB-Core – Release 18

Document for:	Discussion and Decision

# 1 Introduction

In this contribution, we first share our view on below RRC open issues identified by CR Rapporteur:

| Issue 1 | FFS whether a cell can be barred for a mobile IAB-MT |
| --- | --- |

Then, we discuss RIL A101:

We believe there are some misunderstandings on our proposed RIL. Thus, we will also discuss it in this contribution.

# 2 Discussion

## 2.1 Issue 1

##### 5.2.2.4.2	Actions upon reception of the SIB1

--omitted--

2>	if frequencyShift7p5khz is present and the UE supports corresponding 7.5kHz frequency shift on this band; or frequencyShift7p5khz is not present:

3>	if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

3>	else if UE is IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

3> else if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN:

4>	consider the cell as barred in accordance with TS 38.304 [20];

Editor’s Note: FFS whether a cell can be barred for a mobile IAB-MT.

--omitted--

The related RRC spec can be found below:

The raised issue: considering mobile IAB-MT is mobile, it may still be useful to allow it to access cells not broadcasting mobileIAB-Support to have some limited functionality (e.g. OAM access).

We disagree with above suggestion to allow a mobile IAB-MT to access a cell not broadcasting mobileIAB-Support. It will make no point to introduce mobileIAB-Support. And the NW may really intend to forbid mobile IAB-MT to access due to system overloading. Then, it will not work if we allow it. Thus, we propose to keep the RRC spec as it is.

Proposal 1: Keep the current RRC specification on mobile IAB-MT barring, i.e., if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN, consider the cell as barred in accordance with TS 38.304.

## 2.2 Issue 2 (RIL A101)

First, please note that RAN2#123b [1] has agreed that NW can always provide a beam indication for mIAB in dynamic grant case.

Observation: for mIAB, the network can always provide a beam indication

Meanwhile, NTN session has also agreed the same conclusion in the RAN2#124 [2]:

Agreements:

- For dynamic grant case, beam information is mandatorily included in the RACH-less HO command.

- In NTN RACH-less HO, for dynamic grant case, the beam information included in RACH-less HO command is an SSB index (not tci-stateid).

That is why the beam indication for RACH-less in NTN is captured as mandatory present in current RRC running CR, which is highlighted below.

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need R

tci-StateID-r18                     TCI-StateId                                                 OPTIONAL,   -- Cond MobileIAB

dg-beam-r18                         SSB-Index                                                   OPTIONAL,   -- Cond DG-RACH-LessHO

...

}

| Conditional Presence | Explanation |
| --- | --- |
| DG-RACH-LessHO | The field is mandatory present when dynamic grant is used for initial uplink transmission in RACH-less handover in NTN. Otherwise, it is absent, Need R. |
| MobileIAB | The field is optionally present in case mobileIAB-Cell is broadcasted in SIB1. Otherwise, it is absent, Need N. |

Please note above difference between NTN and mIAB.

Since it is common understanding that NW should always provide beam indication in case of dynamic grant case in both NTN and mIAB, we propose to align with the formulation of RACH-less in NTN.

Proposal 2: Align the presence condition of tci-StateID-r18 for mIAB with dg-beam-r18 for NTN, i.e., modify it as follows:

| MobileIAB | The field is optionally present in case mobileIAB-Cell is broadcasted in SIB1 mandatory present when dynamic grant is used for initial uplink transmission in RACH-less handover in Mobile IAB. Otherwise, it is absent, Need NR. |
| --- | --- |

# 3 Conclusion

In this contribution, we discuss open RRC issues in mobile IAB. Our proposals can be found below.

Proposal 1: Keep the current RRC specification on mobile IAB-MT barring, i.e., if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN, consider the cell as barred in accordance with TS 38.304.

Proposal 2: Align the presence condition of tci-StateID-r18 for mIAB with dg-beam-r18 for NTN, i.e., modify it as follows:

| MobileIAB | The field is optionally present in case mobileIAB-Cell is broadcasted in SIB1 mandatory present when dynamic grant is used for initial uplink transmission in RACH-less handover in Mobile IAB. Otherwise, it is absent, Need NR. |
| --- | --- |

# 4 References

[1] RAN2#123b, Chair Notes

[2] RAN2#124, Chair Notes