3GPP TSG-RAN WG2 Meeting #125                                 R2-2400969

Athens, Greece, 26th February – 1st March, 2024

Agenda item:		7.2.6

Source:				Samsung

Title:					Remaining issues on MAC

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on some remaining issues on MAC.

2	Discussion

2.1 (CA#01) TA validation for SRS BA in RRC_INACTIVE

For legacy SRS Tx in RRC_INACTIVE (without BA), the UE transmits positioning SRS in the serving cell carrier and maintain the TA value for the corresponding TAG (i.e., PTAG). However, when we assume that the UE should transmit positioning SRS in two or three carriers for BA, the UE may need to maintain multiple TA values if the aggregated carriers belong to different TAGs. And this will make the TA validation procedure in 38.321 quite complex (e.g., multiple TATs for different TAGs in RRC_INACTIVE).

From our understanding, since the SRS BA requests UE to transmit larger bandwidth SRS signal in the same symbol across multiple carriers, adjusting the TA within the same symbol is almost impossible. Considering that, it seems reasonable to have the restriction that the aggregated carriers should have the same TAG. Meanwhile, according to RAN1#113 chair note, RAN1 discussed whether the aggregated carriers should belong to the same TAG or not, but there was no clear agreement on this. Thus, we would like to propose the following.

Proposal 1: RAN2 to send LS to RAN1 on whether the aggregated carriers can belong to different TAG for positioning SRS BA in RRC_INACTIVE.

2.2 (CA#02) MAC CE for activation/deactivation of SP positioning SRS aggregated across multiple carriers

Regarding the MAC CE to activate/deactivate linked SRS-PosResourceSet, the following agreements are made in RAN1.

For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

According to the RAN1 agreement above, there are two requirements on the MAC CE for activation/deactivation of the aggregated SRS-PosResourceSets as below.

- The MAC CE should be able to indicate activation/deactivation of one or two or three of the aggregated/linked SRS-PosResourceSets. For example, when three SRS-PosResourceSets (e.g., Set X/Y/Z) are linked via RRC signalling, the (de)activation of various combination (e.g., {SetX}, {SetY}, {SetZ}, {SetX&Y}, {SetY&Z}, {SetX&Y&Z}} should be supported by the MAC CE.

- The MAC CE should be able to indicate the single spatial relation for each combination of aggregated SRS-PosResources. When there are multiple combinations of aggregated SRS-PosResources within the aggregated SRS-PosResourceSets, the spatial relation information for each combination of aggregated SRS-PosResources should be indicated by the MAC CE.

Please, find the figure 1 below for the example.

Figure 1. Example of the combination of linked SRS-PosResources and the spatial relation for each combination.

From our understanding, the aforementioned requirements can not be supported by the legacy MAC CE even in the case of using the reserved 1 bit. Thus, RAN2 needs to design a new MAC CE.

Proposal 2: Introduce a new MAC CE to activate/deactivate the linked SRS-PosResourceSets.

For the detailed design of the new MAC CE, it would be related to the configuration of linked SRS-PosResourceSet in RRC. Since we still have the related open issue (i.e., how many combinations of aggregated SRS-PosResourceSets can be configured per UE), we can postpone the discussion on specific design of the new MAC CE until the open issue is resolved. Note that this issue is related to the RIL Z156 in NR RRC ASN.1 review.

Proposal 3: Postpone the discussion on specific design of a new MAC CE to activate/deactivate the linked SRS-PosResourceSets until the related open issue (i.e., how many combinations of aggregated SRS-PosResourceSets can be configured per UE) is resolved.

2.3 SL-PRS prioritization

For SL-PRS prioritization, RAN2 made the following agreements in RAN2#124:

Agreements:

Uplink transmission can be considered as prioritized when uplink cannot be transmitted together with sidelink and none of the V2X sidelink communications or NR sidelink communications or sidelink PRS transmissions are prioritized.

The prioritization between SR triggered by UL-SCH and SL-PRS shall follow the same principle as that between UL-SCH and SL-SCH, i.e. based on configured UL/SL prioritization thresholds.

The prioritization between SR triggered by SL-SCH and SL-PRS shall be based on direct comparison between the SL priority for SL-PRS and the SL logical channel that triggered the SR.

Reuse the legacy threshold for SL communications for SL-PRS prioritization.

For the yellow-colored part, the conditions for SL-PRS prioritization is captured in 5.22.1.3.1a (for shared pool case) and in 5.22.1.3.5 (for dedicated pool case) separately. However, according to the current procedure text, priority of SL-PRS is not considered for the condition related to sl-PrioritizationThres in shared pool case. Thus, we would like to propose the following.

Proposal 4: Adopt the text proposal TP1 to capture the condition of SL-PRS prioritization correctly for shared pool case in 5.22.1.3.1a.

Text Proposal: TP1

5.22.1.3.1a …. The transmission of the MAC PDU or SL-PRS, if available, is prioritized over uplink transmission(s) of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmission(s) at the time of the transmission, and 1> if none of the uplink transmission(s) is prioritized by upper layer according to TS 23.287 [19], and 1> if none of the NR uplink MAC PDU(s) includes any MAC CE prioritized as described in clause 5.4.3.1.3, and 1> if ul-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) of all the NR uplink transmission(s) is not lower than ul-PrioritizationThres, and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or MAC CE(s) in the MAC PDU or SL-PRS is lower than sl-PrioritizationThres.

Meanwhile, in 5.4.4, the procedure text refer the section for dedicated pool case with the wrong section number ‘5.22.1.3.1b’ that does not exist. The reference section number should be corrected to ‘5.22.1.3.5’ as in the TP2 below.

Text Proposal: TP2

5.4.4 …. 3> if an SL-PRS resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-PRS resource, and either transmission on the SL-PRS resource is not prioritized as described in clause 5.22.1.3.1a or 5.22.1.3.5 or the priority value of the logical channel that triggered SR is lower than ul-PrioritizationThres, if configured; or

2.4 Priority of SL-PRS Resource Request MAC CE

For scheme 1 SL-PRS resource allocation, a new MAC CE (i.e., SL-PRS Resource Request MAC CE) is introduced in R18. However, the priority of the new MAC CE is not discussed and determined yet. In our view, considering the aspect that the new MAC CE is used to request the resource for SL transmission as like SL-BSR, it can have the same priority with the SL-BSR MAC CE.

Proposal 5: SL-PRS Resource Request MAC CE has the same priority with the existing SL-BSR MAC CE. Adopt the text proposal TP3.

Text Proposal: TP3

5.4.3.1.3 …. Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; - MAC CE for Timing Advance Report; - MAC CE for Delay Status Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding, or MAC CE for SL-PRS Resource Request; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding. NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

2.5 Request for SL-PRS BW (related to RAN1 LS, R1-2312630)

According to the RAN1 LS [2], RAN1 requests RAN2/3 to consider how the gNB receives a request for SL-PRS BW from LMF or UE in case of Scheme 1. In our view, RAN2 can focus on whether/how the request for SL-PRS BW from the UE to the gNB is supported.

First, we need to check whether it is already supported with the current specification or not. For scheme 1 resource allocation, the UE can request the resource allocation for SL-PRS either via SL-PRS Resource Request MAC CE (for dynamic grant) or via UEAassistanceInformation (for configured grant). In the case of SL-PRS Resource Request MAC CE, it includes only destination index and SL-PRS priority of SL-PRS. Also, the SL-PRS-TxInfo-r18 in UAI contains only the requested periodicity, priority, and delay budget. Thus, the request for SL-PRS BW from UE to gNB is not specified/supported in the current MAC/RRC spec.

Observation 1: Request for SL-PRS BW from UE to gNB is not specified/supported in the current MAC/RRC spec.

In our view, the straightforward way to support the request for SL-PRS BW is just to include the requested BW in SL-PRS Resource Request MAC CE and SL-PRS-TxInfo-r18 in UEAssistanceInformation. Thus, we would like to propose the following.

Proposal 6: Introduce a new field to indicate the requested SL-PRS BW in SL-PRS Resource Request MAC CE (in MAC) and SL-PRS-TxInfo-r18 in UEAssistnaceInformation (in RRC). Adopt the following text proposal TP4 for MAC correction.

Text Proposal: TP4

#### 6.1.3.74 SL-PRS Resource Request MAC CE The SL-PRS Resource Request MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has the following fields: - Destination index: The Destination Index field identifies the destination. The length of this field is 5 bits. The value is set to one index corresponding to SL destination identity associated to same destination reported in sl-PosTxResourceReqList if present. The value is indexed sequentially from 0 in the same ascending order of SL destination identity in sl-PosTxResourceReqList as specified in TS 38.331 [5]. When multiple lists are reported, the value is indexed sequentially across all the lists in the same order as presented in SidelinkUEInformaitonNR message; - Priority: Priority of pending SL-PRS transmission. The length of this field is 3 bits. - BW index: The BW Index field indicates the requested bandwidth of pending SL-PRS transmission. The length of this field is 3 bits. The value is set to one index corresponding to the requested bandwidth value and it is indexed sequentially from 0 in the same ascending order of {5MHz, 10MHz, 20MHz, 40MHz, 50MHz, 80MHz, 100MHz, 200MHz, 400MHz}. FFS. Need RAN1 confirmation on the specific values for the requested bandwidth. The current values are temporally set based on the feature 41-1-1 in the RAN1 UE features list (R1-2312572). Figure 6.1.3.74-1: SL-PRS Resource Request MAC control element

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

(CA#01) TA validation for SRS BA in RRC_INACTIVE:

Proposal 1: RAN2 to send LS to RAN1 on whether the aggregated carriers can belong to different TAG for positioning SRS BA in RRC_INACTIVE.

(CA#02) MAC CE for activation/deactivation of SP positioning SRS aggregated across multiple carriers:

Proposal 2: Introduce a new MAC CE to activate/deactivate the linked SRS-PosResourceSets.

Proposal 3: Postpone the discussion on specific design of a new MAC CE to activate/deactivate the linked SRS-PosResourceSets until the related open issue (i.e., whether more than one combinations of aggregated SRS-PosResourceSets can be configured or not per UE) is resolved.

SL-PRS prioritization:

Proposal 4: Adopt the text proposal TP1 to capture the condition of SL-PRS prioritization correctly for shared pool case in 5.22.1.3.1a.

Text Proposal: TP1

5.22.1.3.1a …. The transmission of the MAC PDU or SL-PRS, if available, is prioritized over uplink transmission(s) of the MAC entity or the other MAC entity if the following conditions are met: 1> if the MAC entity is not able to perform this sidelink transmission simultaneously with all uplink transmission(s) at the time of the transmission, and 1> if none of the uplink transmission(s) is prioritized by upper layer according to TS 23.287 [19], and 1> if none of the NR uplink MAC PDU(s) includes any MAC CE prioritized as described in clause 5.4.3.1.3, and 1> if ul-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) of all the NR uplink transmission(s) is not lower than ul-PrioritizationThres, and 1> if sl-PrioritizationThres is configured and if the value of the highest priority of logical channel(s) or MAC CE(s) in the MAC PDU or SL-PRS is lower than sl-PrioritizationThres.

Priority of SL-PRS Resource Request MAC CE:

Proposal 5: SL-PRS Resource Request MAC CE has the same priority with the existing SL-BSR MAC CE. Adopt the text proposal TP3.

Text Proposal: TP3

5.4.3.1.3 …. Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; - MAC CE for Timing Advance Report; - MAC CE for Delay Status Report; - MAC CE for SL-BSR prioritized according to clause 5.22.1.6; - MAC CE for (Extended) BSR, with exception of BSR included for padding; - MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH; - MAC CE for Positioning Measurement Gap Activation/Deactivation Request; - MAC CE for the number of Desired Guard Symbols; - MAC CE for Case-6 Timing Request; - MAC CE for (Extended) Pre-emptive BSR; - MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding, or MAC CE for SL-PRS Resource Request; - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding; - MAC CE for SL-BSR included for padding. NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

Request for SL-PRS BW:

Observation 1: Request for SL-PRS BW from UE to gNB is not specified/supported in the current MAC/RRC spec.

Proposal 6: Introduce a new field to indicate the requested SL-PRS BW in SL-PRS Resource Request MAC CE (in MAC) and SL-PRS-TxInfo-r18 in UEAssistnaceInformation (in RRC). Adopt the following text proposal TP4 for MAC correction.

Text Proposal: TP4

#### 6.1.3.74 SL-PRS Resource Request MAC CE The SL-PRS Resource Request MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has the following fields: - Destination index: The Destination Index field identifies the destination. The length of this field is 5 bits. The value is set to one index corresponding to SL destination identity associated to same destination reported in sl-PosTxResourceReqList if present. The value is indexed sequentially from 0 in the same ascending order of SL destination identity in sl-PosTxResourceReqList as specified in TS 38.331 [5]. When multiple lists are reported, the value is indexed sequentially across all the lists in the same order as presented in SidelinkUEInformaitonNR message; - Priority: Priority of pending SL-PRS transmission. The length of this field is 3 bits. - BW index: The BW Index field indicates the requested bandwidth of pending SL-PRS transmission. The length of this field is 3 bits. The value is set to one index corresponding to the requested bandwidth value and it is indexed sequentially from 0 in the same ascending order of {5MHz, 10MHz, 20MHz, 40MHz, 50MHz, 80MHz, 100MHz, 200MHz, 400MHz}. FFS. Need RAN1 confirmation on the specific values for the requested bandwidth. The current values are temporally set based on the feature 41-1-1 in the RAN1 UE features list (R1-2312572). Figure 6.1.3.74-1: SL-PRS Resource Request MAC control element

4	Reference

- R1-2312697, ‘Consolidated Rel-18 higher layers parameter list', Ericsson

- R1-2312630, ‘LS on the request for specific SL PRS resource characteristic(s)/SL-PRS resource configuration’, Qualcomm