3GPP TSG-RAN WG2 Meeting #125	R2-2400975

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.5.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on BSR enhancements

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we address the leftover issue from the post meeting email discussion MAC CR [1] and the open issue identified by the rapporteur on padding BSR:

- Issue 0: Refined Long BSR table index 0 value/range.

- Issue 1: Whether Refined Long BSR can be used as a padding BSR?

# 2	Discussions

## 2.1	Issue 0 – Refined Long BS table index 0.

Following the approval of R2-2313823 the value for index 0 in Table 6.1.3.1-3 is defined as the range >4903 and ≤ 5000 as shown below:

From 3GPP TS 38.321 V18.0.0 (2023-12):

Table 6.1.3.1-3. Buffer size levels (in bytes) for the Buffer Size field in Refined Long BSR

| 0 | >4903 and ≤ 5000 | 64 | ≤ 17584 | 128 | ≤ 61841 | 192 | ≤ 217489 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ≤ 5099 | 65 | ≤ 17933 | 129 | ≤ 63069 | 193 | ≤ 221805 |
| 2 | ≤ 5200 | 66 | ≤ 18289 | 130 | ≤ 64320 | 194 | ≤ 226207 |
| 3 | ≤ 5303 | 67 | ≤ 18652 | 131 | ≤ 65596 | 195 | ≤ 230695 |

…

The definition of a range for index 0 stems from the desire to best utilize the code point.

Observation 1: It is indeed desirable to use a range for index 0 of Table 6.1.3.1-3 as opposed to a single.

The value 4903 bytes however has no background information on how it was derived and seems to stem from the desire to keep consistent quantization error across the new Refined Long BS table.

Observation 2: the consistency of quantization errors within a table is not a goal worth pursuing as the legacy table already does not provide that, but we should rather prioritize overall reduction of quantization errors.

An alternative proposal in the comments to R2-2313823 is to use the value 4751 bytes as this is the legacy Long BS value at index 98 and the one below the discussed threshold value of 5000 bytes for fallback.

Consider these scenarios for the two ranges:

- Range >4903 and ≤5000

- Range >4751 and ≤5000

- First thing to note is that range 2) is a lot clearer as to why it exists.

- Consider that if the Refined table is defined for the LCG the UE is expected to first check if its buffer size fits in the Refined BSR table so it knows if it should do search in it or in the fallback table.

- Consider the following examples:

  - Assume buffer size of 4751 bytes or less.

    - For both ranges 1) and 2) UE selects legacy Long and indicates index from legacy table between 0 and 98

  - Assume buffer size between 4752 and 4903 (including).

    - In case of range 1) UE selects fallback and reports legacy long index 99 indicating 5059 bytes or less

    - In case of range 2) UE stays with Refined long table and reports index 0 from it indicating 5000 bytes or less -> this is 59 bytes less error!

  - Assume buffer size between 4904 and 5000 (including).

    - For both ranges 1) and 2) UE stays with Refined Long table and reports index 0 from it indicating 5000 bytes of less

Observation 3: Using range definition for index 0 of Table 6.1.3.1-3 defined as >4751 and ≤5000 is:

- Clearer on the reasoning for the values used.

- Provides lower overall quantization error without additional implementation complications.

Proposal 1: change the definition of the range for Table 6.1.3.1-3 index 0 to >4751 and ≤5000.

## 2.2	Issue 1 –Refined Long BSR for padding BSR

During the e-mail discussion in the lead up to the approval of R2-2313823 it was concluded that RAN2 did not actually agree on the use of Refined Long BSR for padding and proposed changes were removed from the R2-2313823. This section discussed our views on the topic.

Observation 4: The current padding BSR rules covered under section 5.4.5	Buffer Status Reporting of 3GPP TS 38.321 V18.0.0 (2023-12) do NOT allow for the use of Refined Long BSR with the new eLCID regardless of the settings for additionalBSR-TableAllowed of the individual LCGs.

Before discussing the merits of allowing for Refined Long BSR as padding BSR, it is worth considering the differences between legacy Long BSR and Refined Long BSR. There are two main differences:

- Refined Long BSR uses eLCID instead of LCID. This permanently adds one byte to its MAC sub-header.

- Refined Long BSR includes a non-optional additional byte with the BTi indications. This permanently adds one byte to the MAC CE itself.

Observation 5: Refined Long BSR is always 2 bytes larger than the legacy Long BSR.

As an example of the above consider the minimum and maximum lengths of the two MAC CEs. Those are:

- Long BSR

  - Minimum length = 2 (sub-header) + 3 (MAC CE) = 5 bytes because at least 2 LCGs must be configured and have data for transmission before Long BSR is allowed

  - Maximum length = 2 (sub-header) + 9 (MAC CE) = 11 bytes for a maximum of 8 LCGs

- Refined Long BSR

  - Minimum length = 3 (sub-header) + 3 (MAC CE) = 6 bytes because even if only 1 LCG is defined with additionalBSR-TableAllowed then Refined Long BSR is allowed.

  - Maximum length = 3 (sub-header) + 10 (MAC CE) = 13 bytes for a maximum of 8 LCGs regardless how many of them are configured with additionalBSR-TableAllowed (but at least one must be).

In both cases the maximum length of the MAC CE is driven by the number of configured LCGs when all the LCGs are with data buffered

Observation 6: Refined Long BSR is NOT allowed to be truncated as agreed in the previous meeting.

When there are enough padding bits to include Refined Long BSR and at least one LCG uses the Refined BS table and has data to send, we see no reason to artificially disallow it. It could happen when data for some LCH(s) cannot be included in the grant while there are still large among of data buffered, which would be worth reporting with fine granularity.

If the padding size is smaller than the Long BSR when truncated BSR is needed, it is not enough for Refined Long BSR either, for which case could simply apply the legacy procedure. For the case when there is enough padding space without truncation, the UE should first check if Refined Long BSR is to be reported and if there is enough space to include it, if so then include Refined Long BSR. Otherwise, check if there is enough space to report Long BSR like in legacy.

Proposal 2: Refined Long BSR is allowed as padding BSR and checked before Long BSR.

# 3	Conclusion

This document has made the following observations and proposals with TP to MAC provided in Annex:

Observation 1: It is indeed desirable to use a range for index 0 of Table 6.1.3.1-3 as opposed to a single.

Observation 2: the consistency of quantization errors within a table is not a goal worth pursuing as the legacy table already does not provide that, but we should rather prioritize overall reduction of quantization errors.

Observation 3: Using range definition for index 0 of Table 6.1.3.1-3 defined as >4751 and ≤5000 is:

- Clearer on the reasoning for the values used.

- Provides lower overall quantization error without additional implementation complications.

Proposal 1: change the definition of the range for Table 6.1.3.1-3 index 0 to >4751 and ≤5000.

Observation 4: The current padding BSR rules covered under section 5.4.5	Buffer Status Reporting of 3GPP TS 38.321 V18.0.0 (2023-12) do NOT allow for the use of Refined Long BSR with the new eLCID regardless of the settings for additionalBSR-TableAllowed of the individual LCGs.

Observation 5: Refined Long BSR is always 2 bytes larger than the legacy Long BSR.

Observation 6: Refined Long BSR is NOT allowed to be truncated as agreed in the previous meeting.

Proposal 2: Refined Long BSR is allowed as padding BSR and checked before Long BSR.

# Reference

[1] R2-2313823, MAC CR

# TP to MAC

### 5.4.5	Buffer Status Reporting

…

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

1>	if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3:

2>	report Refined Long BSR for all LCGs which have data available for transmission;

1>	else:

2>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

3>	report Long BSR for all LCGs which have data available for transmission.

2>	else:

3>	report Short BSR.

For Regular and Periodic BSR, the MAC entity for which logicalChannelGroupIAB-Ext is configured by upper layers shall:

1>	if more than one LCG has data available for transmission when the MAC PDU containing the BSR is to be built:

2>	if the maximum LCG ID among the configured LCGs is 7 or lower:

3>	report Long BSR for all LCGs which have data available for transmission.

2>	else:

3>	report Extended Long BSR for all LCGs which have data available for transmission.

1>	else:

2>	report Extended Short BSR.

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall:

1>	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

2>	if more than one LCG has data available for transmission when the BSR is to be built:

3>	if the number of padding bits is equal to the size of the Short BSR plus its subheader:

4>	report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission.

3>	else:

4>	report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID.

2>	else:

3>	report Short BSR.

1>	else if for at least one LCG configured with additionalBSR-TableAllowed, the amount of UL data available for transmission is within the buffer sizes specified in Table 6.1.3.1-3 and the number of padding bits is equal to or larger than the size of the Refined Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

1>	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader:

2>	report Long BSR for all LCGs which have data available for transmission.

#### 6.1.3.1	Buffer Status Report MAC CEs

…

Table 6.1.3.1-3. Buffer size levels (in bytes) for the Buffer Size field in Refined Long BSR

| 0 | >4751 and ≤ 5000 | 64 | ≤ 17584 | 128 | ≤ 61841 | 192 | ≤ 217489 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ≤ 5099 | 65 | ≤ 17933 | 129 | ≤ 63069 | 193 | ≤ 221805 |
| 2 | ≤ 5200 | 66 | ≤ 18289 | 130 | ≤ 64320 | 194 | ≤ 226207 |
| 3 | ≤ 5303 | 67 | ≤ 18652 | 131 | ≤ 65596 | 195 | ≤ 230695 |
| 4 | ≤ 5408 | 68 | ≤ 19022 | 132 | ≤ 66898 | 196 | ≤ 235273 |
| 5 | ≤ 5516 | 69 | ≤ 19399 | 133 | ≤ 68226 | 197 | ≤ 239942 |
| 6 | ≤ 5625 | 70 | ≤ 19784 | 134 | ≤ 69580 | 198 | ≤ 244703 |
| 7 | ≤ 5737 | 71 | ≤ 20177 | 135 | ≤ 70960 | 199 | ≤ 249559 |
| 8 | ≤ 5851 | 72 | ≤ 20577 | 136 | ≤ 72368 | 200 | ≤ 254511 |
| 9 | ≤ 5967 | 73 | ≤ 20985 | 137 | ≤ 73804 | 201 | ≤ 259562 |
| 10 | ≤ 6085 | 74 | ≤ 21402 | 138 | ≤ 75269 | 202 | ≤ 264713 |
| 11 | ≤ 6206 | 75 | ≤ 21827 | 139 | ≤ 76763 | 203 | ≤ 269966 |
| 12 | ≤ 6329 | 76 | ≤ 22260 | 140 | ≤ 78286 | 204 | ≤ 275323 |
| 13 | ≤ 6455 | 77 | ≤ 22702 | 141 | ≤ 79839 | 205 | ≤ 280786 |
| 14 | ≤ 6583 | 78 | ≤ 23152 | 142 | ≤ 81424 | 206 | ≤ 286358 |
| 15 | ≤ 6713 | 79 | ≤ 23611 | 143 | ≤ 83040 | 207 | ≤ 292041 |
| 16 | ≤ 6847 | 80 | ≤ 24080 | 144 | ≤ 84687 | 208 | ≤ 297836 |
| 17 | ≤ 6983 | 81 | ≤ 24558 | 145 | ≤ 86368 | 209 | ≤ 303746 |
| 18 | ≤ 7121 | 82 | ≤ 25045 | 146 | ≤ 88082 | 210 | ≤ 309774 |
| 19 | ≤ 7262 | 83 | ≤ 25542 | 147 | ≤ 89830 | 211 | ≤ 315921 |
| 20 | ≤ 7407 | 84 | ≤ 26049 | 148 | ≤ 91612 | 212 | ≤ 322190 |
| 21 | ≤ 7554 | 85 | ≤ 26566 | 149 | ≤ 93430 | 213 | ≤ 328583 |
| 22 | ≤ 7703 | 86 | ≤ 27093 | 150 | ≤ 95284 | 214 | ≤ 335104 |
| 23 | ≤ 7856 | 87 | ≤ 27631 | 151 | ≤ 97175 | 215 | ≤ 341754 |
| 24 | ≤ 8012 | 88 | ≤ 28179 | 152 | ≤ 99103 | 216 | ≤ 348535 |
| 25 | ≤ 8171 | 89 | ≤ 28738 | 153 | ≤ 101070 | 217 | ≤ 355452 |
| 26 | ≤ 8333 | 90 | ≤ 29309 | 154 | ≤ 103076 | 218 | ≤ 362505 |
| 27 | ≤ 8499 | 91 | ≤ 29890 | 155 | ≤ 105121 | 219 | ≤ 369699 |
| 28 | ≤ 8667 | 92 | ≤ 30483 | 156 | ≤ 107207 | 220 | ≤ 377035 |
| 29 | ≤ 8839 | 93 | ≤ 31088 | 157 | ≤ 109335 | 221 | ≤ 384517 |
| 30 | ≤ 9015 | 94 | ≤ 31705 | 158 | ≤ 111504 | 222 | ≤ 392147 |
| 31 | ≤ 9194 | 95 | ≤ 32334 | 159 | ≤ 113717 | 223 | ≤ 399929 |
| 32 | ≤ 9376 | 96 | ≤ 32976 | 160 | ≤ 115973 | 224 | ≤ 407865 |
| 33 | ≤ 9562 | 97 | ≤ 33630 | 161 | ≤ 118275 | 225 | ≤ 415959 |
| 34 | ≤ 9752 | 98 | ≤ 34298 | 162 | ≤ 120622 | 226 | ≤ 424213 |
| 35 | ≤ 9946 | 99 | ≤ 34978 | 163 | ≤ 123016 | 227 | ≤ 432631 |
| 36 | ≤ 10143 | 100 | ≤ 35672 | 164 | ≤ 125457 | 228 | ≤ 441216 |
| 37 | ≤ 10344 | 101 | ≤ 36380 | 165 | ≤ 127946 | 229 | ≤ 449971 |
| 38 | ≤ 10549 | 102 | ≤ 37102 | 166 | ≤ 130485 | 230 | ≤ 458900 |
| 39 | ≤ 10759 | 103 | ≤ 37839 | 167 | ≤ 133074 | 231 | ≤ 468007 |
| 40 | ≤ 10972 | 104 | ≤ 38589 | 168 | ≤ 135715 | 232 | ≤ 477294 |
| 41 | ≤ 11190 | 105 | ≤ 39355 | 169 | ≤ 138408 | 233 | ≤ 486765 |
| 42 | ≤ 11412 | 106 | ≤ 40136 | 170 | ≤ 141155 | 234 | ≤ 496425 |
| 43 | ≤ 11639 | 107 | ≤ 40933 | 171 | ≤ 143956 | 235 | ≤ 506276 |
| 44 | ≤ 11870 | 108 | ≤ 41745 | 172 | ≤ 146813 | 236 | ≤ 516322 |
| 45 | ≤ 12105 | 109 | ≤ 42573 | 173 | ≤ 149726 | 237 | ≤ 526568 |
| 46 | ≤ 12345 | 110 | ≤ 43418 | 174 | ≤ 152697 | 238 | ≤ 537017 |
| 47 | ≤ 12590 | 111 | ≤ 44280 | 175 | ≤ 155727 | 239 | ≤ 547674 |
| 48 | ≤ 12840 | 112 | ≤ 45158 | 176 | ≤ 158817 | 240 | ≤ 558542 |
| 49 | ≤ 13095 | 113 | ≤ 46055 | 177 | ≤ 161969 | 241 | ≤ 569625 |
| 50 | ≤ 13355 | 114 | ≤ 46968 | 178 | ≤ 165183 | 242 | ≤ 580929 |
| 51 | ≤ 13620 | 115 | ≤ 47900 | 179 | ≤ 168461 | 243 | ≤ 592457 |
| 52 | ≤ 13890 | 116 | ≤ 48851 | 180 | ≤ 171804 | 244 | ≤ 604213 |
| 53 | ≤ 14166 | 117 | ≤ 49820 | 181 | ≤ 175213 | 245 | ≤ 616203 |
| 54 | ≤ 14447 | 118 | ≤ 50809 | 182 | ≤ 178690 | 246 | ≤ 628431 |
| 55 | ≤ 14734 | 119 | ≤ 51817 | 183 | ≤ 182236 | 247 | ≤ 640902 |
| 56 | ≤ 15026 | 120 | ≤ 52846 | 184 | ≤ 185852 | 248 | ≤ 653620 |
| 57 | ≤ 15324 | 121 | ≤ 53894 | 185 | ≤ 189540 | 249 | ≤ 666590 |
| 58 | ≤ 15628 | 122 | ≤ 54964 | 186 | ≤ 193302 | 250 | ≤ 679818 |
| 59 | ≤ 15938 | 123 | ≤ 56054 | 187 | ≤ 197137 | 251 | ≤ 693308 |
| 60 | ≤ 16255 | 124 | ≤ 57167 | 188 | ≤ 201049 | 252 | ≤ 707066 |
| 61 | ≤ 16577 | 125 | ≤ 58301 | 189 | ≤ 205039 | 253 | ≤ 721097 |
| 62 | ≤ 16906 | 126 | ≤ 59458 | 190 | ≤ 209108 | 254 | ≤ 735406 |
| 63 | ≤ 17242 | 127 | ≤ 60638 | 191 | ≤ 213257 | 255 | ≤ 750000 |