3GPP TSG-RAN WG2 Meeting #125		R2-2401015

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda Item:	7.17.2

Source: 	LG Electronics

Title:  	Considerations on Open issues for R18 MUSIM

Document for:	Discussion

Introduction

For Rel-18 Multi-SIM WI, RAN2 has some open issues below and, in this contribution, we discuss some directions for open issues #1, #2, #4, and #8.:

| Issue number | Issue description |
| --- | --- |
| Issue 1 | Network’s action upon receiving of the early indication |
| Issue 2 | UAI/Early indication processing during handover procedure |
| Issue 4 | FFS whether UE should start a timer, e.g., Timer T348, after UE submits preference on the measurement gap requirement information. |
| Issue 8 | A NOTE was added for early indication saying that the UE does not apply failure handling in case the UE is unable to apply part of the configuration and what the baseline configuration is, the similar issue may also occur after UE enters RRC_CONNECTED state, so FFS similar NOTE may be needed for RRC Reconfiguration. |

Discussion

Network’s action upon receiving of the early indication

The UE sends message 5 to indicate the temporary capability restriction. Upon receiving Message 5, the network becomes aware of the need to restrict the UE's capability temporarily. For this, the network will provide other configurations of RRC Reconfiguration for reception of the UE Assistance Information (UAI) message.

Upon reception of the other configurations, the UE will transmit the UAI message to request the temporary capability restriction. In addition, if configured, the UE can start the wait timer to perform the restriction quickly.

Based on the above information, we can confirm that no additional new network actions need to be specified in the spec for the case that the network receive early indication from the UE. Since, all network actions following the reception of early indication are handled by existing RRC procedures, we propose the following.

Proposal 1.	For the case of the reception of the early Indication, the network does not require to specify additional actions into the spec.

UAI/Early indication processing during handover procedure

When the UE performs a mobility, the UE can decide whether to retransmit the UAI by comparing the UAI transmission time with the mobility initiation time. In other words, if the UE performs the mobility within 1 second after transmitting the UAI, the UE can assume that this UAI has not been correctly transmitted to the target cell, the UE can retransmit the UAI to the target cell while mobility.

However, the UAI for temporary capability restriction should not be retransmitted like the legacy principle. This is because the temporary capability restriction is a request based on the source cell configuration, not the target cell configuration. The UE will need to confirm, based on the target cell configuration, whether the temporary capability restriction still needs to be requested in the target cell. Otherwise, unnecessary degradation of the UE performance may occur by request of the restriction that do not actually need in the target cell.

To address this issue, the UE should exclude the UAI for the temporary restriction from retransmission rule that is to check to re-transmit if the UAI has been transmitted within 1-second before handover. Then, after successfully accessing the target cell, and based on the maintaining other configurations, the UE will confirm the need to request for temporary capability restriction based on the target cell configuration and transmit the UAI accordingly.

Thus, we propose that the UE excludes the UAI containing temporary restriction for R18 MUSIM from retransmission rule during handover.

Proposal 2.	The UE excludes the UAI from 1-second retransmission rule during handover if the UAI is for the temporary restriction.

Related to this open issue, the next issue is the wait timer handling.

The problem is when the UE starts a wait timer, which has been configured before while transmitting this restriction request. If the wait timer expires without receiving a response from the network, the UE autonomously restricts configuration for the temporary capability restriction. The configured period of the wait timer is approximately up to 100 ms, the wait timer expiration can be frequently occurs before mobility completion.

Moreover, since this temporary capability restriction is based on the source configuration rather than the target configuration and the target network may not aware of this, it may cause not only unnecessary degradation of the UE performance but also operational errors due to configuration mismatches between the network and the UE.

To address this issue, we propose that if the UE performs the mobility within 1 second after transmitting the UAI to the network for temporary restriction, the UE should stop the wait timer. Based on the current RRC procedure, if the UE successfully completes mobility to the target cell and still determines the need for the temporary capability restriction, the UE can initiate to transmit the UAI and (re)start the wait timer. Thus, as the clearest approach, the UE should stop the wait timer when the UE can assume that the target cell is unaware of the wait timer running during the mobility.

Proposal 3.	The UE stop the wait timer if the UE performs the mobility within 1 second after transmitting the UAI to the network for temporary capability restriction.

Timer T348 handling after UE submits preference on the measurement gap requirement information

The measurement gap requirement is one of the essential point required for the UE to maintain dual RRC connections in MUSIM operations. Since we think supporting the wait timer after the UE requests the measurement gap can be beneficial for the UE performance on the other SIM, we propose the following.

Proposal 4.	The UE starts the wait timer after transmitting the UAI for preference on the measurement gap requirement in MUSIM operation.

Applying RRC Reconfiguration issue in RRC CONNECTED state

If the UE is not in the RRC_INACTIVE or RRC_IDLE state, the UE can initiate the reactive approach by sending UAI for the temporary capability restriction. Otherwise, we think new RRC signaling can be considered to prevent such cases from occurring. That is, in scenarios where the UE in RRC_CONNECTED receives an inapplicable RRC Reconfiguration, the UE based restriction (i.e. restriction first and report the results later) can be a solution without relying on the wait timer. However, this approach has been excluded since RAN2 has agreed not to further discuss this in R18 MUSIM.

Then, we think that if the UE faces a situation where the UE cannot apply the RRC Reconfiguration message, the UE should initiate the reactive approach anyway after applying the RRC Reconfiguration. Even if there can be a minor issue in data transmission, there seems no need for additional notes to be captured into the RRC spec for this.

Proposal 5.	RAN2 does not need to capture the NOTE for the case the UE in RRC_CONNECTED is unable to apply part of RRC Reconfiguration message.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1.	For the case of the reception of the early Indication, the network does not require to specify additional actions into the spec.

Proposal 2.	The UE excludes the UAI from 1-second retransmission rule during handover if the UAI is for the temporary restriction.

Proposal 3.	The UE stop the wait timer if the UE performs the mobility within 1 second after transmitting the UAI to the network for temporary capability restriction.

Proposal 4.	The UE starts the wait timer after transmitting the UAI for preference on the measurement gap requirement in MUSIM operation.

Proposal 5.	RAN2 does not need to capture the NOTE for the case the UE in RRC_CONNECTED is unable to apply part of RRC Reconfiguration message.