3GPP TSG-RAN WG2 Meeting #125						                  R2-2401016

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:		7.24.2

Source:				Samsung

Title:					Discussion on SA2 LS on Redcap UE MBS Broadcast Reception

Document for:		Discussion & Decision

# 1 Introduction

This contribution addresses the questions asked in SA2 LS [ R2-2400078/S2-2401506]

# 2 Discussion

Earlier RAN2 had responded to SA2 as:

An indication that an MBS broadcast session is intended to be received by both non-RedCap UE and RedCap UE may assist the gNB to decide when to transmit the session on both default and RedCap CFR and avoid waste of resources when this is not needed.

SA2 has addressed the above with defining "NR RedCap UE information" parameter that comprises an indication for an MBS broadcast session that can take the following values: "MBS session expected to be received only by RedCap UEs", "MBS session expected to be received both by RedCap UEs and non-RedCap UEs", or "MBS session expected to be received only by non-RedCap UEs" (default value if parameter is absent). [CR 0341 to TS 23.247]

Further SA2 has asked following questions in LS to RAN2

Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)?

Regarding Q1, having the same MBS Frequency Selection Area (FSA) ID for the RedCap UEs and non-RedCap UEs in the same MBS session is feasible and this pertains to a MBS service which is expected to be received both by RedCap UEs and non-RedCap UEs. The case is well supported by RAN2 specifications. However, the actual usage for this case, where MBS FSA ID is commonly targeted for RedCap UEs and non-RedCap UEs, depends on deployment strategy of the network operator.

Proposal 1: Respond to Q1 as:

Having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session is feasible and this pertains to a MBS service which is expected to be received both by RedCap UEs and non-RedCap UEs. The case is well supported by RAN2 specifications. However, the actual usage for this case, where MBS FSA ID is commonly targeted for RedCap UEs and non-RedCap UEs, depends on deployment strategy of the network operator.

Regarding Q2, when RedCap UEs and non-RedCap UEs use separate MBS FSA ID(s), from RAN perspective they pertain to separate MBS sessions with different transmission resources, even though these sessions belong to same MBS service. The case is relevant for the MBS session which is expected to be received only by RedCap UEs and MBS session which is expected to be received only by non-RedCap UEs and these sessions cater to same MBS service.

Proposal 2: Respond to Q2 as:

When RedCap UEs and non-RedCap UEs use separate MBS FSA ID(s), from RAN perspective they pertain to separate MBS sessions with different transmission resources, even though these sessions belong to same MBS service. The case is relevant for the MBS session which is expected to be received only by RedCap UEs and MBS session which is expected to be received only by non-RedCap UEs and these sessions cater to same MBS service.

# 3 Conclusion

Proposal 1: Respond to Q1 as:

Having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session is feasible and this pertains to a MBS service which is expected to be received both by RedCap UEs and non-RedCap UEs. The case is well supported by RAN2 specifications. However, the actual usage for this case, where MBS FSA ID is commonly targeted for RedCap UEs and non-RedCap UEs, depends on deployment strategy of the network operator.

Proposal 2: Respond to Q2 as:

When RedCap UEs and non-RedCap UEs use separate MBS FSA ID(s), from RAN perspective they pertain to separate MBS sessions with different transmission resources, even though these sessions belong to same MBS service. The case is relevant for the MBS session which is expected to be received only by RedCap UEs and MBS session which is expected to be received only by non-RedCap UEs and these sessions cater to same MBS service.