3GPP TSG-RAN WG2 Meeting #125	R2-2401062

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	7.4.1.3.1

Source: 	Fujitsu

Title: 	Considerations on LTM related open issues

Document for:	Discussion and decision

1. Introduction

This contribution will identify some open issues in existing TS38.331 [1] and provide our considerations on these issues.

2. Discussion

Issue 1: LTM execution attempt after initial LTM

attempLTM-Switch is configured in the IE LTM-Config rather than LTM-Candidate according to the following LTM configuration:

LTM-Config information element -- ASN1START -- TAG-LTM-CONFIG-START LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond FirstLTM-Only ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond LTM ... } -- TAG-LTM-CONFIG-STOP -- ASN1STOP

In addition, according to the following text regarding LTM execution, attempLTM-Switch will be released/cleared upon LTM execution.

##### 5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall: 1> release/clear all current dedicated radio configuration associated with the cell group for which the LTM cell switch procedure is triggered except for the following: - the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers; - the UE variables VarLTM-Config, VarLTM-ServingCellNoResetID, and VarLTM-ServingCellUE-MeasuredTA-ID. 2> if the LTM cell switch is triggered on the MCG: - the MCG C-RNTI; - the AS security configurations associated with the master key; - for each SRB/DRB in current UE configuration which is using the master key: - keep the associated PDCP and SDAP entities, their state variables, buffers and timers; - release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity; 2> else, if the LTM cell switch is triggered on the SCG: - the AS security configurations associated with the secondary key; - for each SRB/DRB in current UE configuration which is using the secondary key: - keep the associated PDCP and SDAP entities, their state variables, buffers and timers; - release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity; 1> release/clear all current common radio configuration associated with the cell group for which the LTM cell switch procedure is triggered; 1> use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associate to cell group for which the LTM cell switch procedure is triggered; 1> if the value of field ltm-NoResetID contained within the LTM-Candidate IE in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID: 2> for each logicalChannelId and logicalChannelIdExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered: 3> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in VarLTM-Config; 2> for each drb-Identity value that is part of the current UE configuration: 3> if this DRB is an AM DRB: 4> after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in VarLTM-Config; 2> replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;1> if the LTM-Candidate IE in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID: 2> replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID; 1> if ltm-ConfigComplete is not included within the LTM-Candidate IE in VarLTM-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3: 2> consider ltm-ReferenceConfiguration in VarLTM-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations which have been released and to be released by the actions above in this procedure; NOTE 1: When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3. 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in VarLTM-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in VarLTM-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3; 1> release the radio bearer(s) and the logical channel(s) that are part of the current UE configuration but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete). NOTE 2: When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRC reconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRC reconfiguration message is applied when the LTM cell switch is triggered.

Only if attempLTM-Switch is configured, LTM candidates can be used for RLF/HOF recovery. Based on the existing description, fast recovery using LTM candidate can be supported once, as shown in the figure below. If there has already been one LTM execution which can be triggered either by LTM cell switch command or by MCG RLF/HOF, the UE cannot use LTM candidate for RLF/HOF recovery after the LTM execution.

Observation 1: a UE cannot use LTM candidate for RLF/HOF recovery after the initial LTM because the configured attempLTM-Switch is released during the LTM execution.

However, RAN2 has not discussed this issue yet and we had no such agreement on this. Instead, the benefits to use LTM candidates for fast recovery can be observed. Based on RAN2 agreement, LTM candidate configuration can be maintained by the UE after LTM. It is better that the UE can use LTM candidate not only for cell switch triggered by the network but also for RLF/HOF recovery, if attempLTM-Switch is configured.

Proposal 1: LTM candidate can be used for RLF/HOF recovery after initial LTM if attempLTM-Switch is configured.

To support this, attempLTM-Switch, if configured, should be maintained during LTM execution. So, the following options can be considered.

Proposal 1a: To discuss the following options to use configured attempLTM-Switch after the initial LTM:

- Option 1: maintain attempLTM-Switch, if configured, upon LTM execution;

- Option 2: store attempLTM-Switch, if configured, in VarLTM-Config;

- Option 3: apply attempLTM-Switch, if configured in LTM-candidate

  - In this option, attempLTM-Switch is configured per LTM candidate, i.e. included in LTM-candidate.

Option 1 is simple and straightforward. If Option 1 is agreed, the TP in Annex 1 can be adopted.

Proposal 1b: To adopt the TP in Annex 1 to support fast recovery using LTM candidate after initial LTM.

Issue 2: potential issue to initiate LTM

RAN2 agreed that “an LTM cell switch procedure should not be triggered while an MCG failure recovery procedure is ongoing” [2]. In our understanding, the MCG failure recovery procedure can be MCG failure information procedure to report MCG RLF. The LTM cell switch procedure includes either MCG LTM or SCG LTM. RAN2 needs to confirm the common understanding of this agreement.

Proposal 2: To confirm that an SCG or MCG LTM cell switch procedure should not be triggered while an MCG failure recovery procedure, e.g. the MCG failure information procedure is ongoing.

If it is confirmed, RRC needs to implement this agreement considering the following cases:

- SCG LTM CSC MAC CE is received while the MCG failure recovery procedure is ongoing

- After MCG LTM CSC MAC CE is received but the timer T304 for MCG LTM is not started yet, the MCG failure recovery procedure is initiated

To solve this issue, T316 can be used to control the LTM execution.

Proposal 3: RAN2 to agree that the UE performs SCG/MCG LTM execution if the MCG failure recovery procedure is not ongoing, i.e. T316 is not configured or T316 is not running.

Proposal 3a: To adopt the TP in Annex 2 to stop SCG/MCG LTM execution while the MCG failure recovery procedure is ongoing.

3. Conclusion

In this contribution, we have some discussions on the remaining issues and made some observations and proposals:

Issue 1: LTM execution attempt after initial LTM

Observation 1: a UE cannot use LTM candidate for RLF/HOF recovery after the initial LTM because the configured attempLTM-Switch is released during the LTM execution.

Proposal 1: LTM candidate can be used for RLF/HOF recovery after initial LTM if attempLTM-Switch is configured.

Proposal 1a: To discuss the following options to use configured attempLTM-Switch after the initial LTM:

- Option 1: maintain attempLTM-Switch, if configured, upon LTM execution;

- Option 2: store attempLTM-Switch, if configured, in VarLTM-Config;

- Option 3: apply attempLTM-Switch, if configured in LTM-candidate

  - In this option, attempLTM-Switch is configured per LTM candidate, i.e. included in LTM-candidate.

Proposal 1b: To adopt the TP in Annex 1 to support fast recovery using LTM candidate after initial LTM.

Issue 2: potential issue to initiate LTM

Proposal 2: To confirm that an SCG or MCG LTM cell switch procedure should not be triggered while an MCG failure recovery procedure, e.g. the MCG failure information procedure is ongoing.

Proposal 3: RAN2 to agree that the UE performs SCG/MCG LTM execution if the MCG failure recovery procedure is not ongoing, i.e. T316 is not configured or T316 is not running.

Proposal 3a: To adopt the TP in Annex 2 to stop SCG/MCG LTM execution while the MCG failure recovery procedure is ongoing.

Reference

- TS 38.331 V18.0.0

- Chair Notes, RAN2 #121, Feb. of 2023

Annex 1: TP for issue 1 (Option 1)

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	release/clear all current dedicated radio configuration associated with the cell group for which the LTM cell switch procedure is triggered except for the following:

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers;

-	the UE variables VarLTM-Config, VarLTM-ServingCellNoResetID, and VarLTM-ServingCellUE-MeasuredTA-ID.

2>	if the LTM cell switch is triggered on the MCG:

-	attempLTM-Switch, if configured;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	for each SRB/DRB in current UE configuration which is using the master key:

-	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

2>	else, if the LTM cell switch is triggered on the SCG:

-	the AS security configurations associated with the secondary key;

-	for each SRB/DRB in current UE configuration which is using the secondary key:

-	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

1>	release/clear all current common radio configuration associated with the cell group for which the LTM cell switch procedure is triggered;

Annex 2: TP for issue 2

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered and if T316 is not running or not configured, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	release/clear all current dedicated radio configuration associated with the cell group for which the LTM cell switch procedure is triggered except for the following:

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers;

-	the UE variables VarLTM-Config, VarLTM-ServingCellNoResetID, and VarLTM-ServingCellUE-MeasuredTA-ID.

2>	if the LTM cell switch is triggered on the MCG:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	for each SRB/DRB in current UE configuration which is using the master key:

-	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

2>	else, if the LTM cell switch is triggered on the SCG:

-	the AS security configurations associated with the secondary key;

-	for each SRB/DRB in current UE configuration which is using the secondary key:

-	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

-	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

1>	release/clear all current common radio configuration associated with the cell group for which the LTM cell switch procedure is triggered;