3GPP TSG-RAN WG2 Meeting #125	R2-2401189

Athens, Greece, 26th Feb. – 1st Mar., 2024

Title:		MAC spec open issue list for R18 POS

Source:	Huawei, HiSilicon

Agenda item:	7.2.6

Document for:	Discussion and Decision

- Discussion

In this paper, we provide the list of open issues for MAC spec for R18 positioning. For ease of reading, we use the following table to illustrate the potential issues.

| SL#06 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select multiple SL-PRS transmission when SL-PRS is triggered either by the peer UE or the UE’s own upper layer. |
| --- | --- | --- |
| SL#07 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether the MAC layer can determine to select single SL-PRS transmission when SL-PRS transmission is triggered by its own upper layer or by peer UE. |
| SL#08 | 5.22.1.1 SL Grant reception and SCI transmission | FFS SL-PRS transmission on SL-PRS shared resource pool when the MAC PDU has been positively acked for resource allocation scheme 1 and scheme 2 |
| SL#10 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the MAC entity determines the SL-PRS delay budget. |
| SL#11 | 5.22.1.1 SL Grant reception and SCI transmission | FFS minimum time gap requirement on SL-PRS shared resource pool. |
| SL#12 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how the SL-PRS resource is determined based on the list of RRC configured SL-PRS configurations, priority, PHY sensing and MAC layer random resource selection for resource allocation scheme 2. |
| SL#13 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 2. |
| SL#14 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS occasion on SL-PRS shared resource pool can be cleared when the MAC PDU has been positively acked for resource allocation scheme 1. |
| SL#15 | 5.22.1.1 SL Grant reception and SCI transmission | FFS whether SL-PRS priority is determined by priority in the peer UE’s SCI or the UE’s own higher layer when the trigger comes from the peer UE’s SCI. |
| SL#16 | 5.22.1.1 SL Grant reception and SCI transmission | FFS how SL-PRS priority is determined when SL-PRS is triggered by the UE’s own higher layer. |
| SL#17 | 5.22.1.2a Re-evaluation and Pre-emption 5.22.1.2b Re-selection for using a received resource conflict indication | The same issue as section 5.22.1.1 for the relationship between remaining PDB and SL-PRS delay budget for resource selection on SL-PRS shared resource pool. |
| CA#01 | 5.26.2 TA validation for SRS transmission in RRC_INACTIVE | FFS TA validation for positioning SRS transmission in RRC_INACTIVE with positioning SRS bandwidth aggregation |
| CA#02 | TBD | FFS whether to reuse the current MAC CE or design a new MAC CE for activation/deactivation of SP positioning SRS with multiple carrier indications |

- Conclusions

Companies are invited to contribute to the discussion in RAN2#125 according to the open issue list above.