3GPP TSG-RAN WG2 Meeting #125		 R2-2401229

Athens, GR: 26th February – 1st March 2023

Source:	Qualcomm Incorporated

Title:	Clarification on RAInformationCommon Reporting in SPR

Agenda Item:	7.13.3 - Other

Document for:	Discussion and Decision

# Discussion

In Rel-18, NR-U related enhancements have been agreed for the RA report, RLF report, and SHR [1, 2]. SCG SON reports (e.g., SCGFailureInformation, SPR) enhancements for NR-U were not discussed/agreed in Rel-18. For SPR, RAN2 agreed to include ra-InformationCommon to indicate random-access related information associated to the random access procedure in the target PSCell.

Observation 1: In Rel-18, NR-U related enhancements have been agreed for the the RA report, RLF report, and SHR.

Observation 2: SCG SON reports (e.g., SCGFailureInformation, SPR) enhancements for NR-U were not discussed in Rel-18.

Based on RAN2/RAN3 discussions on SPR and NR-U related enhancements, we believe that RA-informationCommon reported in the SPR should not include the NR-U related information elements. We request RAN2 to clarify whether RA-InformationCommon reported in the SPR should not include the NR-U related information elements.

Proposal 1: RAN2 is requested to clarify whether RA-InformationCommon reported in the SPR includes the NR-U related information elements.

# Conclusion

Observation 1: In Rel-18, NR-U related enhancements have been agreed for the the RA report, RLF report, and SHR.

Observation 2: SCG SON reports (e.g., SCGFailureInformation, SPR) enhancements for NR-U were not discussed in Rel-18.

Proposal 1: RAN2 is requested to clarify whether RA-InformationCommon reported in the SPR includes the NR-U related information elements.

# Reference

[1] TS 38.331

[2] TS 38.306