3GPP TSG-RAN WG2 Meeting #125	R2-2401248

Athens, Greece, February 26 – March 1, 2024

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	LPP Open Issue: PRU Operation

Document for:		Discussion and Decision

# 1.	Introduction

This contribution discusses the following LPP open issue:

| Topic | Open issues | Clause Number | Comments |
| --- | --- | --- | --- |
| RAN1 Led item- Carrier phase positioning | 9)FFS exact IE structure of the request for location+measurements | 6.4.2 Common Positioning | Will discuss based on contribution |

In this contribution, we discuss the issue and propose the "IE structure of the request for location+measurements" for PRUs.

# 2.	Background

## 2.1	RAN1/RAN2 Agreements

PRUs were originally introduced by RAN1 during Rel-17 at RAN1#105e as follows [1]:

Agreement: Send an LS to RAN2/RAN3 (cc SA2), including the following content: - RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning. - Notes: - The term “positioning reference unit (PRU)” is only used as a terminology in this discussion. PRU does not necessarily mean an introduction of a new network node. - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2. The positioning functionalities may include, but not limited to, the following: - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences) - Transmit the UL SRS signals for positioning - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF. R1-2106326 LS on Positioning Reference Units (PRUs) for enhancing positioning performance

RAN2 discussed the above mentioned RAN1 LS first at RAN2#115e and made the following agreements [2]:

Agreements:

Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17).

PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action).

Further discussions in RAN1 happened related to Carrier Phase Positioning in Rel-18. According to RAN1/2 discussions and agreements, a PRU should be able to report location measurements together with PRU coordinates (in case the PRU location changes during the location measurements/reporting) [3], [4]:

RAN1 [3]: A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report.

RAN2 [4]: A PRU should be able to report (a) the location coordinates and associated location quality/uncertainty of the PRU only, (b) the location measurements performed by the PRU only, or (c) the location coordinates and associated location quality/uncertainty of the PRU together with any performed location measurements, where location measurements above include the currently defined location measurements in LPP and potential new location measurements defined in Rel-18; and location coordinates and location quality/uncertainty provide the location and quality of the reported location, respectively, of the PRU.

The PRU reporting issue was further discussed at RAN2#123bis [5] with the following agreement [6]:

Agreements:

TP from R2-2310854 can be migrated into the LPP running CR.

FFS exact IE structure of the request for location+measurements.

A Note for clarification can be added to address concern that the location is based on the measurement:

Note:   For PRU, if PRU is requested to return both location estimate and measurements, the location information is determined independently of the reported measurements.

The FFS item was not resolved/discussed at RAN2#124 (although, contributions were available, e.g., [7]) and LPP still contains the FFS from RAN2#123bis in IE CommonIEsRequestLocationInformation:

FFS exact IE structure of the request for location+measurements in the agreement of RAN2#123bis.

## 2.2	PRU Architecture, Signalling, and Procedures

The architecture, signalling and procedures related to PRU operation are specified in TS 23.273 [8]. In essence:

-	PRUs are a separate entity in the LCS architecture (see e.g., Figure 4.2.1-1 in [8], and may be realized by a UE with a known location (analogous to RAN1 and the PRU definition in TS 38.305)).

- 	A PRU registers at an LMF using Supplementary Services signalling (PRU association/disassociation procedures in clause 6.17 of [8]). These procedures provide PRU information to an LMF (e.g., location of PRU) and a Correlation ID (assigned by the associated AMF), which enables the LMF to instigate LPP transactions with a particular PRU. From an AMF (and LMF) point of view, a PRU is not a target UE. I.e., the LMF does not need/expect a request from an AMF to locate a PRU and provide the location result to an LCS client or AF.

-	A PRU has a dedicated profile in an UDM. A UE which supports the functions of a PRU indicates this in the subscription data together with an indication whether PRU is stationary or not.

-	A LMF which supports PRUs indicates this to NRF via NF profile.

-	For positioning of a target UE, a LMF may exchange LPP Transactions with the target UE and with one or more PRUs.

Observation 1:	In the LCS framework, a PRU is not a target UE whose location is to be determined as a result of a location request from an LCS Client or AF.

Observation 2:	All agreements in RAN1 and RAN2 regarding the reporting of "location+measurements" were made for PRUs only (see section 2.1).

# 3.	Discussion

For reporting "location+measurements", LPP currently extends the locationInformationType:

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...,

[[

locationEstimateAndMeasurementsRequired-r18

]]

}

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible. For 'locationEstimateAndMeasurementsRequired', the PRU shall return both location estimate and measurements if possible, or indicate a location error if not possible. NOTE: If the PRU is requested to return both location estimate and measurements, the location information is determined independently of the reported measurements.

As already discussed in [7], the locationInformationType in CommonIEsRequestLocationInformation controls the requested positioning mode (UE-based, UE-assisted) for position location of a target UE using the procedures defined in TS 23.273 [8] and TS 38.305 [9]. In UE assisted mode the UE obtains location measurements and sends the measurements to an LMF to compute a location; in UE based mode the UE obtains location measurements and computes a location estimate making use of assistance data provided by an LMF [8].

Observation 3:	The locationInformationType in CommonIEsRequestLocationInformation controls the requested positioning mode (UE-based, UE-assisted) for position location of a target UE.

If a UE operates as a PRU, the PRU provides location measurements performed at a known location (e.g., to support carrier phase positioning, etc.). In this case, the PRU is not a "target UE" [8], since the PRU location must already be known via other means. For PRU operation, an LMF requires the PRU measurements (e.g., RSTD, RSCPD, etc.) at an already known location.

Observation 4:	For PRU operation, a LMF requires location measurements (e.g., RSTD, RSCPD, etc.) from an a-priori known location.

I.e., the known location is exploited for PRU operation to determine desired information (e.g., determine error terms, cancel error terms via (single or double) differencing, etc.). Hence, the requirement that the PRU location coordinates are determined independently of the reported measurements (see also section 2 above).

Observation 5:	The PRU location information and measurements are decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements.

In some cases (moving PRU), the location measurements may be made during movement of the PRU, and therefore, the location measurements should be accompanied by the PRU's antenna reference point (ARP) location for the time when the location measurements were obtained. It was agreed at RAN2#123bis that such "location + measurements" reporting should only happen upon LMF request (which is clearly sub-optimal (at best) since a LMF would not know whether a PRU is moving during measurements (note, LMF may know that a UE is able/allowed to move, but this does not imply that the PRU is moving continuously)) and an extension of the LocationInformationType was proposed for this as shown above.

However, the LocationInformationType would not be appropriate for this PRU function since this would imply that the "location measurements" and "location estimate" are not independent of each other.

With the extension of the LocationInformationType with an entry for "location + measurements", the LPP RequestLocationInformation message would include the CommonIEsRequestLocationInformation with LocationInformationType set to "location + measurements" together with a e.g., NR-DL-TDOA-RequestLocationInformation-r16 (in case of DL carrier phase positioning):

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]]

}

If the LocationInformationType would be set to locationEstimateAndMeasurementsRequired (as currently assumed in LPP), it would imply that the "location estimate" is obtained with the same method for which the "location measurements" were obtained (since this is what the LPP Location Request defines; see above field description). However, this would not allow PRU operation, since the PRU location must be obtained independently from the PRU measurements. I.e., it is not possible to specify that the location estimate is obtained with a "method A" and the location measurements with a "method B" in a single LPP Request Location Information message (since this is not needed for LCS (i.e., positioning a target UE)). The current specification of locationEstimateAndMeasurementsRequired would then also imply that only positioning methods supported in UE-based mode can be used for PRU operation (for example, it would not be possible to report Multi-RTT/RSCP measurements together with PRU location, since Multi-RTT is not supported in UE-based mode).

Observation 6:	Extending the locationInformationType with an entry for "location+measurements" would be ambiguous/incorrect, since it implies that the "measurements" were also used for obtaining the "PRU location" (i.e., UE-based and UE-assisted mode combined).

Observation 7:	Extending the locationInformationType with an entry for "location+measurements" would also imply that only positioning methods supported in UE-based mode can be used for PRU operation (e.g., it would not be possible to report Multi-RTT/RSCP measurements together with PRU location, since Multi-RTT is not supported in UE-based mode).

For PRU operation in carrier phase positioning for example, the usual scenario would be that the PRU reports measurements (RSCP or RSCPD) and obtains its location independently via surveying or HA-GNSS.

In order to not mix positioning of a Target UE with PRU operation, it is proposed to introduce a dedicated "PRU Location Information Request" element. This can be extensible if additional PRU information is required in the future.

Proposal 1:	For PRU operation, introduce a new IE for requesting/reporting PRU specific information (PRU/ARP coordinates in this Release).

This would be future proof and unambiguous. I.e., the existing measurement request/report can be used for requesting/providing PRU measurements, and in case the PRU coordinates are required in addition, a separate request/report element is used, which would not mix "positioning of a Target UE" and "PRU operation". A corresponding TP is provided in the Annex of this contribution.

Proposal 2:	Agree the TP in the Annex of this contribution.

# 4.	Summary

In this contribution, we discussed some remaining issues on PRU operation. The following Observations and Proposals were made.

Observation 1:	In the LCS framework, a PRU is not a target UE whose location is to be determined as a result of a location request from an LCS Client or AF.

Observation 2:	All agreements in RAN1 and RAN2 regarding the reporting of "location+measurements" were made for PRUs only (see section 2.1).

Observation 3:	The locationInformationType in CommonIEsRequestLocationInformation controls the requested positioning mode (UE-based, UE-assisted) for position location of a target UE.

Observation 4:	For PRU operation, a LMF requires location measurements (e.g., RSTD, RSCPD, etc.) from an a-priori known location.

Observation 5:	The PRU location information and measurements are decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements.

Observation 6:	Extending the locationInformationType with an entry for "location+measurements" would be ambiguous/incorrect, since it implies that the "measurements" were also used for obtaining the "PRU location" (i.e., UE-based and UE-assisted mode combined).

Observation 7:	Extending the locationInformationType with an entry for "location+measurements" would also imply that only positioning methods supported in UE-based mode can be used for PRU operation (e.g., it would not be possible to report Multi-RTT/RSCP measurements together with PRU location, since Multi-RTT is not supported in UE-based mode).

Proposal 1:	For PRU operation, introduce a new IE for requesting/reporting PRU specific information (PRU/ARP coordinates in this Release).

Proposal 2:	Agree the TP in the Annex of this contribution.

# References

[1]	RP-211313, "Status Report to TSG", RAN#92e.

[2]	RP-212365, "Status Report to TSG", RAN#93e.

[3]	R2-2302409 (R1-2302146), "LS Reply on PRU Procedures", RAN1 LS in REL-18 NR_pos_enh2-Core, 	5G_eLCS_Ph3 To: SA2 Cc: RAN2, RAN3.

[4]	R2-2304459, "Response LS on PRU Procedures", RAN2 LS out Rel-18 NR_pos_enh2 To:SA2.

[5]	R2-2311376, "Report of [AT123bis][402][POS] PRUs (CATT)".

[6]	R2-2311275, "Report from session on positioning and sidelink relay" (RAN2#123bis), Session Chair (MediaTek).

[7]	R2-2312805, "Remaining Issues on PRU Operation", Qualcomm Incorporated.

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

# Annex: LPP TP for PRU Operation

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.305 [2], TS 23.271 [3], 38.305 [40] and TS 23.273 [42] apply. Other definitions are provided below.

[…]

Positioning Reference Unit (PRU): Refers to a Positioning Reference Unit realized by a UE as defined in clause 5.4.5 of TS 38.305 [40].

[…]

### 4.1.1	LPP Configuration

LPP is used point-to-point between a location server (E-SMLC, LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources. Figure 4.1.1-1 shows the configuration as applied to the control- and user-plane location solutions for E-UTRAN and NG-RAN (as defined in TS 36.305 [2], TS 38.305 [40], TS 23.273 [42] and TS 23.271 [3]).

NOTE:	LPP may also be used point-to-point between a location server and a PRU, as specified in TS 23.273 [42].

NB-IoT is a non-backward compatible variant of E-UTRAN supporting a reduced set of functionalities. In this specification, procedures and messages specified for the UE equally apply to the UE in NB-IoT.

Figure 4.1.1-1: LPP Configuration for Control- and User-Plane Positioning in E-UTRAN or NG-RAN

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]],

[[

remoteUE-Indication-r18						BOOLEAN						OPTIONAL	-- Cond NR

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| NR | This field is optionally present, need OR, for NR access if remoteUE-IndicationReq has been received from the location server in this location session. Otherwise it is not present. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

remoteUE-Indication This field indicates whether the target device in NR access is configured as a L2 U2N Remote UE. The target device in NR access may transmit a ProvideCapabilities message with an appropriate value of this field when it starts or stops operation as a U2N Remote UE.

[…]

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]],

[[

pru-LocationInformationReq-r18

PRU-LocationInformationReq-r18

OPTIONAL	-- Cond PRU

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...

}

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

TriggeredReportingCriteria ::=		SEQUENCE {

cellChange							BOOLEAN,

reportingDuration					ReportingDuration,

...

}

ReportingDuration ::=				INTEGER (0..255)

AdditionalInformation ::= ENUMERATED {

onlyReturnInformationRequested,

mayReturnAdditionalInformation,

...

}

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

HorizontalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

VerticalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

HorizontalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

VerticalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

ResponseTimeNB-r14 ::= SEQUENCE {

timeNB-r14							INTEGER (1..512),

responseTimeEarlyFixNB-r14			INTEGER (1..512)		OPTIONAL,		-- Need ON

...,

[[	unitNB-r15				ENUMERATED { ten-seconds, ... }	OPTIONAL		-- Need ON

]]

}

Environment ::= ENUMERATED {

badArea,

notBadArea,

mixedArea,

...

}

MessageSizeLimitNB-r14 ::= SEQUENCE {

measurementLimit-r14				INTEGER (1..512)		OPTIONAL,		-- Need ON

...

}

ScheduledLocationTime-r17 ::= SEQUENCE {

utcTime-r17			UTCTime											OPTIONAL,	-- Need ON

gnssTime-r17		SEQUENCE {

gnss-TOD-msec-r17		INTEGER (0..3599999),

gnss-TimeID-r17			GNSS-ID

}												OPTIONAL,	-- Need ON

networkTime-r17		CHOICE {

e-utraTime-r17			SEQUENCE {

lte-PhysCellId-r17			INTEGER (0..503),

lte-ArfcnEUTRA-r17			ARFCN-ValueEUTRA,

lte-CellGlobalId-r17		CellGlobalIdEUTRA-AndUTRA

OPTIONAL,	-- Need ON

lte-SystemFrameNumber-r17	INTEGER (0..1023)

},

nrTime-r17				SEQUENCE {

nr-PhysCellID-r17		NR-PhysCellID-r16,

nr-ARFCN-r17			ARFCN-ValueNR-r15,

nr-CellGlobalID-r17		NCGI-r15	OPTIONAL,		-- Need ON

nr-SFN-r17				INTEGER (0..1023),

nr-Slot-r17				CHOICE {

scs15-r17				INTEGER (0..9),

scs30-r17				INTEGER (0..19),

scs60-r17				INTEGER (0..39),

scs120-r17				INTEGER (0..79)

}										OPTIONAL	-- Need ON

},

...

}												OPTIONAL,	-- Need ON

relativeTime-r17	INTEGER (1..1024)								OPTIONAL	-- Need ON

}

TargetIntegrityRisk-r17 ::=		INTEGER (10..90)

PRU-LocationInformationReq-r18 ::= SEQUENCE {

arpCoordinates-r18			ENUMERATED { requested }				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ECID | The field is optionally present, need ON, if E-CID or NR E-CID is requested. Otherwise it is not present. |
| PRU | The field is optionally present, need ON, if the UE operates as a Positioning Reference Unit (PRU) as specified in TS 23.273 [42]. Otherwise it is not present. |

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible.

triggeredReporting This IE indicates that triggered reporting is requested and comprises the following subfields: - cellChange: If this field is set to TRUE, the target device provides requested location information each time the primary cell has changed. - reportingDuration: Maximum duration of triggered reporting in seconds. A value of zero is interpreted to mean an unlimited (i.e. "infinite") duration. The target device should continue triggered reporting for the reportingDuration or until an LPP Abort or LPP Error message is received. The triggeredReporting field should not be included by the location server and shall be ignored by the target device if the periodicalReporting IE or responseTime IE or responseTimeNB IE is included in CommonIEsRequestLocationInformation.

periodicalReporting This IE indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64 correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a target device is able to obtain new measurements or obtain a new location estimate. The value 'noPeriodicalReporting' shall not be used by a sender.

additionalInformation This IE indicates whether a target device is allowed to return additional information to that requested. If this IE indicates 'onlyReturnInformationRequested' then the target device shall not return any additional information to that requested by the server. If this IE indicates 'mayReturnAdditionalInformation' then the target device may return additional information to that requested by the server. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include E-CID measurements if A-GNSS measurements were requested but not E-CID measurements).

qos This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows: - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE. - unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. If the enumerated value 'ten-milli-seconds' is included for methods others than NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning the target device shall ignore the unit field. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). - responseTimeNB If the periodicalReporting IE or responseTime IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - timeNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. - responseTimeEarlyFixNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFixNB IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the timeNB IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFixNB IE. A server should set the responseTimeEarlyFixNB IE to a value less than that for the timeNB IE. A target shall ignore the responseTimeEarlyFixNB IE if its value is not less than that for the timeNB IE. - unitNB indicates the unit of the timeNB and responseTimeEarlyFixNB fields. Enumerated value 'ten-second' corresponds to a resolution of 10 seconds. If this field is absent, the unit/resolution is 1 second. - horizontalAccuracyExt indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the horizontalAccuracy field is included in QoS. - verticalAccuracyExt indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the verticalAccuracy field is included in QoS. All QoS requirements shall be obtained by the target device to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable. The single exception is time and timeNB which shall always be fulfilled – even if that means not fulfilling other QoS requirements. A target device supporting NB-IoT access shall support the responseTimeNB IE. A target device supporting HA GNSS shall support the HorizontalAccuracyExt, VerticalAccuracyEx, and unit fields with enumerated value 'ten-seconds'. A target device supporting NB-IoT access and HA GNSS shall support the unitNB field.

environment This field provides the target device with information about expected multipath and non line of sight (NLOS) in the current area. The following values are defined: - badArea: possibly heavy multipath and NLOS conditions (e.g. bad urban or urban). - notBadArea: no or light multipath and usually LOS conditions (e.g. suburban or rural). - mixedArea: environment that is mixed or not defined. If this field is absent, a default value of 'mixedArea' applies.

locationCoordinateTypes This field provides a list of the types of location estimate that the target device may return when a location estimate is obtained by the target.

velocityTypes This fields provides a list of the types of velocity estimate that the target device may return when a velocity estimate is obtained by the target.

messageSizeLimitNB This field provides an octet limit on the amount of location information a target device can return. - measurementLimit indicates the maximum amount of location information the target device should return in response to the RequestLocationInformation message received from the location server. The limit applies to the overall size of the LPP message at LPP level (LPP Provide Location Information), and is specified in steps of 100 octets. The message size limit is then given by the value provided in measurementLimit times 100 octets.

segmentationInfo This field indicates whether this RequestLocationInformation message is one of many segments, as specified in clause 4.3.5

scheduledLocationTime This field indicates that the target device is requested to obtain location measurements or location estimate valid at the scheduledLocationTime T and comprises the following subfields: - utcTime provides T in UTC in the form of YYMMDDhhmmssZ. - gnssTime provides T in GNSS system time of the GNSS indicated by gnss-TimeID. - gnss-TOD-msec specifies the GNSS TOD in 1-milli-second resolution rounded down to the nearest millisecond unit. - networkTime provides T in E-UTRA or NR network time. - lte-PhysCellId, lte-ArfcnEUTRA, lte-CellGlobalId identifies the reference cell (E-UTRA) that is used for the network time. - lte-systemFrameNumber specifies the system frame number in E-UTRA. - nr-PhysCellID, nr-ARFCN , nr-CellGlobalID identifies the reference cell (NR) that is used for the network time. - nr-SFN specifies the system frame number in NR. - nr-Slot specifies the slot number in NR for the indicated subcarrier spacing (SCS). The total NR network time is given by nr-SFN + nr-Slot. - relativeTime provides T in seconds from current time, where current time is defined as the time the CommonIEsRequestLocationInformation was received. NOTE 1: A location estimate returned to an LCS Client, AF or UE for a scheduled location time can be treated by the LCS Client, AF or UE as an estimate of the location of the UE at the scheduled location time (see TS 23.273 [42]). NOTE 2: If this field is present, at least one of utcTime, gnssTime, networkTime, or relativeTime shall be present.

targetIntegrityRisk This field indicates the TIR for which the PL is requested. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

pru-LocationInformationReq This field indicates that the UE operating as a PRU as specified in TS 23.273 [42] is requested to provide additional PRU location information and comprises the following subfields: - arpCoordinates indicates that the PRU is requested to provide the Antenna Reference Point (ARP) coordinates of the PRU for the time when the requested location measurements were obtained.

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]],

[[

pru-LocationInformation-r18

PRU-LocationInformation-r18

OPTIONAL		-- Cond PRU

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																			HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16,

local2dPointWithUncertaintyEllipse-v1800	Local2dPointWithUncertaintyEllipse-r18,

local3dPointWithUncertaintyEllipsoid-v1800	Local3dPointWithUncertaintyEllipsoid-r18

}

Velocity ::= CHOICE {

horizontalVelocity							HorizontalVelocity,

horizontalWithVerticalVelocity				HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty			HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty

HorizontalWithVerticalVelocityAndUncertainty,

...

}

LocationError ::= SEQUENCE {

locationfailurecause			LocationFailureCause,

...

}

LocationFailureCause ::= ENUMERATED {

undefined,

requestedMethodNotSupported,

positionMethodFailure,

periodicLocationMeasurementsNotAvailable,

...

}

EarlyFixReport-r12 ::= ENUMERATED {

noMoreMessages,

moreMessagesOnTheWay

}

LocationSource-r13 ::= BIT STRING {	a-gnss				(0),

wlan				(1),

bt					(2),

tbs					(3),

sensor				(4),

ha-gnss-v1510		(5),

motion-sensor-v1550 (6),

dl-tdoa-r16			(7),

dl-aod-r16			(8) } (SIZE(1..16))

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,

...

}

PRU-LocationInformation-r18 ::= SEQUENCE {

arpCoordinates-r18					LocationCoordinates				OPTIONAL,

timeStamp-r18						NR-TimeStamp-r16				OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PRU | The field is optionally present, need ON, if the UE operates as a Positioning Reference Unit (PRU) as specified in TS 23.273 [42]. Otherwise it is not present. |

CommonIEsProvideLocationInformation field descriptions

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each geographic shape follow the rules in TS 23.032 [15]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [15].

locationError This field shall be included if and only if a location estimate and measurements are not included in the LPP PDU. The field includes information concerning the reason for the lack of location information. The LocationFailureCause 'periodicLocationMeasurementsNotAvailable' shall be used by the target device if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

earlyFixReport This field shall be included if and only if the ProvideLocationInformation message contains early location measurements or an early location estimate. The target device shall set the values of this field as follows: - noMoreMessages: This is the only or last ProvideLocationInformation message used to deliver the entire set of early location information. - moreMessagesOnTheWay: This is one of multiple ProvideLocationInformation messages used to deliver the entire set of early location information (if early location information will not fit into a single message). If this field is included, the IE SegmentationInfo shall not be included.

locationSource This field provides the source positioning technology for the location estimate. NOTE 1: In this version of the specification, the entry 'tbs' is used only for TBS positioning based on MBS signals. NOTE 2: The entry 'sensor' is used only for positioning technology that uses barometric pressure sensor. The entry 'motion-sensor' is used for positioning technology that uses sensor(s) to detect displacement and movement, e.g. accelerometers, gyros, magnetometers.

locationTimestamp This field provides the UTC time when the location estimate is valid and should take the form of YYMMDDhhmmssZ.

segmentationInfo This field indicates whether this ProvideLocationInformation message is one of many segments, as specified in clause 4.3.5

integrityInfo This field provides the integrity result for the locationEstimate. - horizontalProtectionLevel provides the HPL for the locationEstimate along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres. - verticalProtectionLevel provides the VPL for the locationEstimate. Scale factor 0.01 metre; range 0 – 500 metres. - achievableTargetIntegrityRisk indicates the achievable TIR for which the HPL and VPL are provided. The achievable TIR is given by P=10-0.1n [hour-1] where n is the value of achievableTargetIntegrityRisk and the range is 10-1 to 10-9 per hour. If this field is absent, the achievable TIR is the same as the targetIntegrityRisk in CommonIEsRequestLocationInformation.

pru-LocationInformation This field provides PRU location information and comprises the following subfields: - arpCoordinates provides the Antenna Reference Point (ARP) coordinates of the PRU at the location where the reported location measurements were obtained. If this field was requested but not provided, it means that the ARP coordinates are not available. - timestamp provides the time when the arpCoordinates are valid.

NOTE:	Void.