3GPP TSG-RAN WG2 Meeting #125	R2-2401305

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.20.2

Source:	Nokia, Nokia Shanghai Bell

Title:	MAC issue with TAT expiry and 2TA

WID/SID:	NR_MIMO_evo_DL_UL-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This TDoc discusses an issue with TAT expiry when 2 TAGs are configured.

# 2	Discussion

The TS 38.321 v18.0.0 now specifies:

## 5.2 Maintenance of Uplink Time Alignment (…) 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else if the timeAlignmentTimer is associated with a TAG for an SCell, then for all SCells configured with only this TAG; or 2> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running, then for all such SCells: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. 2> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, then for all such Serving Cells: 3> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 3> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 3> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

As can be seen above, for example, whenever a TAT is associated with a TAG for an SCell which is not configured with two TAGs, the yellow highlighted if clause is true above. In this case, if there is a Serving Cell with two TAGs configuration and the other TAT is running, the procedure part with green highlight would not be performed since it is conditioned with “else if”. Another case is whenever a TAT is associated with a TAG for an SCell which is configured with two TAGs and the TAT of the other TAG associated with this SCell has also expired, the yellow highlighted if clause is true above. Also in this case, if there is a Serving Cell with two TAGs configuration and the other TAT is running (naturally this would be different from the SCell), the procedure part with green highlight would not be performed since it is conditioned with “else if”.

It seems all the conditions can be written without the “else if” structure as they are self conditioned so the following TP is proposed:

Proposal: Adopt the following TP into TS 38.321 in section 5.2.

## 5.2 Maintenance of Uplink Time Alignment (…) 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> if the timeAlignmentTimer is associated with a TAG for an SCell, then for all SCells configured with only this TAG; or 2> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running, then for all such SCells: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. 2> if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, then for all such Serving Cells: 3> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 3> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 3> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.

# 3	Conclusion

Proposal: Adopt the following TP into TS 38.321 in section 5.2.

## 5.2 Maintenance of Uplink Time Alignment (…) 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> if the timeAlignmentTimer is associated with a TAG for an SCell, then for all SCells configured with only this TAG; or 2> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running, then for all such SCells: 3> flush all HARQ buffers; 3> notify RRC to release PUCCH, if configured; 3> notify RRC to release SRS, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG. 2> if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, then for all such Serving Cells: 3> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 3> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 3> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 3> maintain NTA (defined in TS 38.211 [8]) of this TAG.