3GPP TSG-RAN WG2 Meeting #125	R2-2401409

Athens, Greece, 26 February – 1 March 2024

Agenda Item:	7.7.5

Source:	Ericsson

Title:	Remaining issue on switch procedure for satellite switch with re-sync

Document for:	Discussion, Decision

# 1	Introduction

In Release 18, 3GPP is introducing a new procedure for handling service link switches in quasi-Earth-fixed deployments referred to as satellite switch with re-synchronization (i.e., without PCI change) which does not require L3 mobility. The objective is to reduce signaling overhead due to the frequent satellite change in LEO/MEO NTN systems and mitigate satellite switch failure due to RACH resource congestion. Satellite switch with re-synchronization can be performed when the so-called source and target satellites operate in the same SSB frequency and utilize the same gNB (no key change). The baseline procedure was endorsed in [1] and two scenarios have been agreed so far by RAN2:

- Hard satellite switch with re-sync, wherein the network stops serving UEs via the source satellite at time t-Service and starts serving UEs via the target satellite at time t-Start. For Release 18, signaling a t-Start higher than t-Service is an unforeseen case and the UE will assume t-Start = t-Service.

- Soft satellite switch with re-sync, wherein t-Start of target satellite is earlier than t-Service of source satellite, the exact time when the UE starts synchronizing with target satellite (between t-Start and t-Service) is up to UE implementation.

For both hard and soft satellite switch with re-synchronization, a UE supporting TA reporting may trigger TAR and TAR-SR based on gNB configuration (as in legacy) after switched to the target satellite.

Figure 1. RACH-less satellite switch procedure.

This contribution addresses remaining issues on switch procedure for satellite switch with re-synchronization.

# 2	Discussion

## 2.1	Confirmation of the switch for the soft scenario

After the switch execution, in step 5 of Figure 1, when RACH-based access is not required, the first UL transmission corresponds to the Timing Advance report (TAR) mechanism. This procedure was introduced in Rel-17 and provides the gNB with an estimate of the UE's Timing Advance value [2]. TAR is an optional UE capability independent from satellite switch with resynchronization (softSatelliteSwitchResyncNTN-r18), i.e., UEs which support the latter do not necessarily need to support the former. Thus, it remains unclear what would be the UE behaviour to confirm the switch execution in case the UE supports satellite switch with resynchronization but not Timing Advance report.

It is unclear how a UE behaves after satellite switch with resynchronization in case TAR is not supported.

Among the different solutions, there are three which are straightforward:

- Modify capability. RAN2 could modify the capability description for softSatelliteSwitchResyncNTN and mandate the support of TAR for this scenario.

- Continue legacy MAC behavior. As long as TAT is running, UE could continue its MAC operation and perform the first and subsequent UL transmission via PUCCH and PUSCH in the target satellite. There is no specification impact.

- Mandatory Random Access. Lastly, UEs without TAR support could be mandated to initiate the UL transmission via RACH. However, this may increase the probability of a signaling storm and RACH congestion after the switch.

As described in the next section, the first option may not work in all scenarios, i.e., when the timing shift is very small. The third option would counter the main benefit of the satellite switch with re-sync procedure, i.e., a smooth transition between satellites without signaling overhead and congestion. The second option suffers from the uncertainty issue in DL transmission presented in [2].. Still, we have a slight preference for the second option as it does not involve specification impact.

During satellite switch with resynchronization, when the UE does not support TAR, legacy MAC behavior is applied to the first and subsequent UL transmissions in the target satellite.

## 2.2	Resumption of DL transmission

The endorsed procedure displayed in Figure 1 captures solely the behavior of the UE. As presented in [2], there exists an inherent uncertainty to the procedure related to when switch really happens at the UE side. This uncertainty depends on the selected scenario and may influence the network behavior. Therefore, RAN2 should clarify when the UE should expect the network to resume operations after the satellite switch.

The case of soft satellite switch is discussed in a separate contribution [2]. For the hard satellite switch, there are two prominent options:

- Option 1: Resume DL transmission after the first SSB is transmitted in the target satellite.

- Option 2: Resume DL transmission after successfully receiving TAR from the UE in the target satellite when TAR is supported.

Depending on whether P1 is agreed, option 2 may be unsuitable. In addition, it may lead to large interruption in DL transmission as TAR may not be immediately triggered in certain scenarios. For instance, when the variation between the current and last reported information about Timing Advance is smaller than offsetThresholdTA. Thus, our preference for the first option.

For the hard satellite switch re-sync, network resumes DL transmission in the target satellite after the first SSB is transmitted.

# 4	Conclusions

In the previous sections we made the following observations:

Observation 1	It is unclear how a UE behaves after satellite switch with resynchronization in case TAR is not supported.

Based on the discussion in the previous sections we propose the following:

Proposal 1	During satellite switch with resynchronization, when the UE does not support TAR, legacy MAC behavior is applied to the first and subsequent UL transmissions in the target satellite.

Proposal 2	For the hard satellite switch re-sync, network resumes DL transmission in the target satellite after the first SSB is transmitted.

# 5	References

R2-2313877, “RACH-less satellite switch procedure”, Apple, RAN2#124, November 2023.

R2-2401400, Remaining issue on soft satellite switch with re-sync, Ericsson, RAN2#125, March 2024.