3GPP TSG-RAN WG2 Meeting #125	                                                                    R2-2401477

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Source:	CATT

Title:	Discussion on MAC Issues for LTM

Agenda Item:	7.4.1.4

Document for:	Discussion and Decision

- 1. Introduction

In this paper, we discuss MAC related issues for LTM.

The detailed discussion is in Section 2, and followed by Section 3 that gives the conclusion of this paper.

- 2. Discussion

Issue 1: configured grant type for RACH-less LTM

We have agreed to support CG-based LTM for RACH-less cell switch,it is understood that only type 1 CG resource can be configured for RACH-less LTM.

In 5.8.2, it captures the following sentence for CG-SDT,

Only configured grant Type 1 can be configured for CG-SDT. CG-SDT can only be configured on initial BWP.

So it is suggested to add the clarification also for RACH-less LTM.

The corresponding TP for Correction 1 is provided in Annex 1.

Proposal 1: Clarify in section 5.8.2 that only type 1 CG resource can be configured for RACH-less LTM cell switch.TP in Annex 1 is adopted.

Issue 2: Purpose of TCI state in cell switch command MAC CE

In the field description of TCI state ID/UL TCI state ID, it says “This field indicates and activates the TCI state/UL TCI state ID for the LTM target cell”. This wording may cause ambiguity as we have introduced Candidate Cell TCI States Activation/Deactivation MAC CE for early TCI activation for a certain candidate cell. If the early TCI state has been activated for a candidate cell, then the TCI state field in the LTM cell switch command MAC CE is only used to indicate the specific TCI that has been early activated.

The corresponding TP for Correction 2 is provided in Annex 2.

Proposal 2: Clarify in the field description of TCI state ID/UL TCI state ID in section 6.1.3.75 that this field indicates and/or activates the TCI state/UL TCI state ID for the LTM target cell. TP in Annex 2 is adopted.

Issue 3:Co-existence between LTM and mTRP

In the open issue list provided by the MAC CR rapptour, open issue#3 is about the cross Rel-18 features, e.g. FFS on LTM cell switch MAC CE to support Rel-18 MIMO two TAG case. Before discussing the co-existence of LTM with mDCI mTRP two TAs, we first check whether LTM can work well with candiadate cell configured with mTRP.

If the candidate cell is configured with mTRP, UE could maintain two joint or two DL and UL TCI separately. For the SRS/CG transmission, it should be indicated with the associated TCI state (first or second or both) based on RRC configuration or based on DCI indication.

Upon receiving the LTM cell switch command, the TCI state information is provided in the LTM cell switch command MAC CE, which indcates the activated TCI used for the target cell. And it refers to the outer TCI state ltm-DL-OrJointTCI-StateToAddModList/ltm-UL-TCI-StatesToAddModList. Based on current spec, it doesn’t specify the activated TCI included in the MAC CE is applied as the first TCI or the second TCI. So which resource could be used to transmit UL data for the indicated TCI is not clear if mTRP is configured by the target cell. In summary, from RAN2 current spec, it couldn’t support the LTM cell switch to a candidate cell configured with mTRP.

Observation 1: Currently RAN2 spec doesn’t support to indicate which resource can be used to transmit UL data using the activated TCI state included in the LTM cell switch command.

Furthermore, in order to support two TA, it should include the associated TAG ID in the TCI state configuration, which is not support for the candidate TCI either. Meanwhile, it doesn’t support which TAG is associated with the TA value included in the LTM cell switch command.

Observation 2: Currently RAN2 spec doesn’t support to indicate which TAG is associated with the activated TCI and TA value included in the LTM cell switch command.

So from RAN2 point view, if to support the LTM candidate cell to configured with mTRP, or to support two TA, more spec impact is needed. Considering the froze time for the spec, we suggest that it doesn’t support to configure mTRP for LTM candidate cell.

Proposal 3: Do not support the co-existence between LTM and mTRP.

- 3. Conclusion

In this paper, we discuss some MAC related issues on LTM. The observation and proposal of this paper are as follows,

Observation 1: Currently RAN2 spec doesn’t support to indicate which resource can be used to transmit UL data using the activated TCI state included in the LTM cell switch command.

Observation 2: Currently RAN2 spec doesn’t support to indicate which TAG is associated with the activated TCI and TA value included in the LTM cell switch command.

Proposal 1: Clarify in section 5.8.2 that only type 1 CG resource can be configured for RACH-less LTM cell switch.TP in Annex 1 is adopted.

Proposal 2: Clarify in the field description of TCI state ID/UL TCI state ID in section 6.1.3.75 that this field indicates and/or activates the TCI state/UL TCI state ID for the LTM target cell. TP in Annex 2 is adopted.

Proposal 3: Do not support the co-existence between LTM and mTRP.

- 4. Reference

[1] 3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification” V18.0.0

- Annex 1

5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

Only configured grant Type 1 can be configured for CG-SDT or for RACH-less LTM cell switch. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	rach-less-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for RACH-less handover;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration;

-	timeReferenceH-SFN: H-SFN used for determination of the offset of a resource in time domain. The UE uses the closest H-SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process;

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial CG-SDT transmission with CCCH message when the UE shall not autonomously retransmit the HARQ process;

-	cg-LTM-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial transmission at LTM cell switch when the UE shall not autonomously initiate a retransmission on the HARQ process;

-	cg-RACH-less-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process of the initial transmission of RACH-less handover when the UE shall not autonomously retransmit the HARQ process.

RRC configures the following additional parameter for a multi-PUSCH configured grant:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

For a configured uplink grant, the MAC entity shall:

1>	if the configured uplink grant is associated with a multi-PUSCH configured grant:

2>	if configured uplink grant has not been indicated to the lower layers as to be unused for PUSCH transmission; and

2>	if the configured uplink grant meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]:

3>	consider the configured uplink grant available for use;

1>	else if the configured uplink grant has not been indicated to lower layers as to be unused for PUSCH transmission:

2>	consider the configured uplink grant available for use.

The MAC entity determines if a configured uplink grant is going to be used for PUSCH transmission or not by considering the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to the lower layers regarding this decision, for use in the procedure for reporting UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]).

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

-	if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured:

- [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

-	else:

- [(H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	((timeReferenceH-SFN × numberOfSFNperH-SFN + timeReferenceSFN)
	× numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × 1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For a multi-PUSCH configured grant Type 1, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured uplink grant:

5>	select this SSB.

4>	else if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is not above cg-SDT-RSRP-ThresholdSSB:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant.

3>	if SSB is selected above:

4>	indicate the SSB index to the lower layer;

4>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

2>	if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

3>	if there is data available for transmission for at least one RB configured for SDT:

4>	initiate Random Access procedure in clause 5.1.

NOTE 1:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

For an uplink grant configured for configured grant Type 1 for LTM cell switch, when there is an on-going RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

For an uplink grant configured for configured grant Type 1 for RACH-less handover, when RACH-less handover is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after the initial transmission of RACH-less handover has been performed according to clause 5.4.1 and 5.33, PDCCH addressed to the MAC entity's C-RNTI has not been received:

2>	if the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for the initial transmission of RACH-less handover (i.e., retransmission of initial transmission of RACH-less handover):

3>	select this SSB;

3>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if at least one SSB corresponding to the configured uplink grant with SS-RSRP above rach-less-RSRP-ThresholdSSB is available:

2>	select an SSB with SS-RSRP above rach-less-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant;

2>	indicate the selected SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid;

2>	initiate Random Access procedure in clause 5.1.

NOTE 1A:	When the UE determines if there is an SSB with SS-RSRP above rach-less-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

- [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within the same periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-LTM-RetransmissionTimer, cg-RACH-less-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

- Annex 2

6.1.3.75	LTM Cell Switch Command MAC CE

The LTM Cell Switch Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.75-1):

-	R: Reserved bit, set to 0;

-	Target Configuration ID: This field indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Timing Advance Command: This field indicates whether the TA is valid for the LTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID field). If the value of this field is set to FFF, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell; Otherwise, this field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6], and that the UE can skip the Random Access procedure for this LTM cell switch. The length of the field is 12 bits;

-	TCI state ID: This field indicates and activates (or only indicates the TCI state for the LTM target cell if the TCI state had been activated by the Candidate Cell TCI States Activation/Deactivation MAC CE) the TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The TCI state is identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList as specified in TS 38.331 [5]. If the value of unifiedTCI-StateType in the configuration indicated by Target Configuration ID field is joint, this field is for joint TCI state, otherwise, this field is for downlink TCI state. The length of the field is 7 bits;

-	UL TCI state ID: This field indicates and activates (or only indicates the TCI state for the LTM target cell if the TCI state had been activated by the Candidate Cell TCI States Activation/Deactivation MAC CE) the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The most significant bits of UL TCI state ID are considered as reserved bits and the remainder 6 bits indicate the TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5]. This field is included if the value of unifiedTCI-StateType in the configuration indicated by Target Configuration ID field is separate. The length of the field is 8 bits;

-	C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present, including Random Access Preamble index field, S/U field, SS/PBCH index field and PRACH Mask index field. If the value of this field is set to 0, Random Access Preamble index field, SS/PBCH index field and PRACH Mask index field are absent, and S/U field is considered as Reserved field.

-	S/U: This field indicates which UL carrier to transmit the PRACH of the contention-free Random Access Resources. If the value of this field is set to 1, SUL is used; otherwise, NUL is used. The length of the field is 1 bit;

-	Random Access Preamble index: This field indicates the Random Access Preamble index of the contention-free Random Access Resources. The length of the field is 6 bits;

-	SS/PBCH index: This field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission of the contention-free Random Access Resources. The length of the field is 6 bits;

-	PRACH Mask index: This field indicates the RACH occasion(s) associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission of the contention-free Random Access Resources, referring to the rach-ConfigDedicated (if not provided otherwise to the rach-ConfigCommon) in the UL BWP configuration of firstActiveUplinkBWP-Id as specified in TS 38.331 [5]. The length of the field is 4 bits.

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE