3GPP TSG-RAN WG2 Meeting #125	R2-2401628

# Athens, Greece, 26 February – 01 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0389 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

X

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of SIB12 configuration for a L2 Remote UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2024-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification mandates a L2 U2N Remote UE that received SIB12 via the relay connection to always consider SL configuration received in SIB12 irrespectively of whether it performs SL communication/discovery for relay connection or for non-relay connection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that an out-of-coverage L2 U2N Remote UE that received SIB12 via the relay connection is only mandated to use the SL configuration received in SIB12 for relay SL communication/discovery. Impact analysis Impacted functionality: Remote UEs sidelink communication Inter-operability: - If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. - If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There can be a negative impact for the non-relay SL communication/discovery of an out-of coverage L2 U2N remote UE that received SIB12 via the relay connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 8.1	NR sidelink communication, and V2X sidelink communication, NR sidelink discovery, and ranging/ sidelink positioning

The UE may transmit or receive NR sidelink communication/discovery if it fulfils the condition(s) defined in TS 38.331 [3], clause 5.8.2. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform NR sidelink communication/discovery according to SIB12, and when out-of-coverage for sidelink, the UE may perform NR sidelink communication/discovery according to SL-PreconfigurationNR or according to SIB12 of the cell on the frequency which provides inter-carrier NR sidelink configuration, or according to SIB12 received from the connected L2 U2N Relay UE as specified in TS 38.331 [3]. The UE shall not perform NR sidelink communication/discovery according to SL-PreconfigurationNR if the UE detects a cell providing NR sidelink configuration or inter-carrier NR sidelink configuration for the frequency UE is interested to perform NR sidelink communication/discovery on. A L2 U2N Remote UE shall not perform NR sidelink relay communication/discovery according to SL-PreconfigurationNR if the UE has received SIB12 configuration from the connected L2 U2N Relay UE for the frequency UE is interested to perform NR sidelink relay communication/discovery on.

The UE may transmit or receive V2X sidelink communication if it fulfills the condition(s) defined in TS 36.331[6], clause 5.10.1d. When UE is in-coverage for sidelink operation as defined in clause 8.2, the UE may perform V2X sidelink communication according to SIB13/ SIB14 of the cell on an NR frequency.

The UE may transmit or receive SL-PRS for ranging/sidelink positioning if it fulfils the conditions defined in TS 38.331 [3].

The U2N Remote UE, the U2N Relay UE, the U2U Remote UE, or the U2U Relay UE may transmit NR sidelink relay discovery (i.e., as specified in TS 23.304 [22]) if it fulfills the condition(s) defined in TS 38.331 [3].

For NR sidelink broadcast and groupcast, the UE may obtain SL DRX configuration from SIB12 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state; or for out-of-coverage, UE as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which the UE is configured to perform NR sidelink communication/discovery and which is not included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12).

For inter-UE coordination (IUC) information configuration, the UE may obtain it from SIB12 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state; or for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12) or SL-PreconfigurationNR (for non L2 U2N Remote UE out-of-coverage, as defined in clause 8.2, on the frequency which UE is configured to perform NR sidelink communication and which is not included in sl-FreqInfoList/sl-FreqInfoListSizeExt in SIB12).

For ranging/sidelink positioning, the UE may obtain the configuration from SIB23 (for in-coverage UE, as defined in clause 8.2, in RRC_IDLE and RRC_INACTIVE state) or SL-PreconfigurationNR (for out-of-coverage UE, as defined in clause 8.2).

End of Changes