3GPP TSG-RAN WG2 Meeting # 125	R2-2401941

# Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0033 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PDCCH CCE Usage for gNB Layer 2 measurement [L2M_PDCCH_Usage] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, China Unicom, Huawei, ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In the current specification, the performance metric PRB Usage for MIMO has been defined (see subclause 4.2.1.7), which can measure the usage of resources of PDSCH/PUSCH. In real-life deployment, it is observed that the number of users is large while PDSCH/PUSCH PRB usage is relatively low. In such case, the PDCCH is the bottleneck channel. Introducing the usage of PDCCH is essential for both OAM performance observability and inter-cell load balancing. - Similar to PDSCH/PUSCH usage metric, multiple MIMO layers of PDCCH can be scheduled to the UE. And several different users can share the same PDCCH CCE at the same time with the deployment of MIMO. If MIMO layer is not taken into account, the calculation of the usage of resources of PDCCH is always higher than the reality for Massive MIMO scenario. Therefore, the introduction of PDCCH CCE Usage need to take the space division multiplexing into consideration to reflect the usage of resources of PDCCH precisely. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement of PDCCH CCE Usage is introduced in TS 38.314 to reflect the usage of resources of PDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is impossible to precisely reflect the usage of PDCCH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

4	Layer 2 measurements

<Partially omitted>

4.2	NR measurements performed by the gNB

4.2.1	Measurements valid for all gNB deployment scenarios

<Partially omitted>

4.2.1.x	PDCCH CCE Usage

4.2.1.x.1	PDCCH CCE Usage per cell

This measurement provides the total usage (in percentage) of PDCCH control-channel elements (CCEs) per cell. The objective of the measurement is to measure usage of time, frequency and space resources.

Protocol Layer: MAC, PHY

Table 4.2.1.x.1-1: Definition for PDCCH CCE Usage per cell

| Definition | PDCCH CCE Usage per cell is calculated in the time-frequency-space domain. Detailed Definition: MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}*Alpha*100, where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for PDCCH CCE Usage per cell

| M(T) | Total PDCCH CCE usage per cell which is percentage of CCEs used for MIMO and non-MIMO, averaged during time period T with integer value range: 0-100. |
| --- | --- |
| M1ij(T) | For MIMO, a count of PDCCH CCEs used for control information transmission for UE i on single MIMO layer per cell at sampling occasion j; For non-MIMO, a count of PDCCH CCEs used for control information transmission for UE i per cell at sampling occasion j. |
| Lij(T) | For MIMO, the number of MIMO layers scheduled for UE i at sampling occasion j; For non-MIMO, this parameter should be set to 1. |
| i | A UE i that is scheduled during time period 𝑇. |
| j | Sampling occasion during time period 𝑇. A sampling occasion is 1 symbol. |
| Pj(T) | For MIMO, total number of PDCCH CCEs available for sampling occasion j on single MIMO layer per cell. For non-MIMO, total number of PDCCH CCEs available for sampling occasion j per cell. |
| T | Time period during which the measurement is performed to calculate MT, e.g., 15min, 1 hour, etc. |
| Alpha | A constant value configured by OAM during time period 𝑇 with float value range: 1.00-100.00. For MIMO, this parameter should be set to 1.00-100.00; For non-MIMO, this parameter should be set to 1.00. With this parameter, M(T) should not be larger than 100. |

<< End of changes >>