3GPP TSG-RAN WG2 Meeting #125bis											R2-2402848

Changsha, China, April 15th – 19th, 2024

Source: 	Xiaomi

Title:  	SFN reading required for RACH-less

Agenda Item:	7.12.2.2

Document for:	Discussion and Decision

# Introduction

According to the current MAC and RRC procedure, the UE can use the RACH-less procedure for mobile IAB by reusing the TA. In this contribution, we provide same analysis on the clarification required for the SFN reading for the RACH-less procedure for mobile IAB.

# Discussion

## SFN/MIB acquisition

According to 3GPP TS 38.331, the periodicity (i.e. reusing the legacy CG periodicity) of the CG configuration for the RACH-less for mobile IAB may require the SFN of the target cell.

Observation 1: The CG configuration for the RACH-less of mobile IAB may require the SFN of the target cell.

38.331: ##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. … NOTE 2: The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

According to the current RRC specification as quoted above, the UE is required to read the MIB (i.e. SFN) during mobility when the periodicity of PRACH occasion requires the UE to obtain the SFN from the target cell. However, the above “NOTE 2” only covers the case of the RACH procedure, not the RACH-less procedure for mobile IAB.

Observation 2: The current RRC specification only clarifies that the MIB reading is required for RACH, not for the RACH-less procedure for mobile IAB.

Thus, we consider that the RRC specification should be clarified that MIB reading is required also for the RACH-less procedure for mobile IAB.

Proposal: The RRC specification is clarified that the UE is required to read MIB if the timing information is required for RACH-less of mobile IAB.

An example of text proposal for the above Proposal 2 can be found as follows:

38.331: ##### 5.3.5.5.2 Reconfiguration with sync The UE shall perform the following actions to execute a reconfiguration with sync. … NOTE 2: The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access. NOTE x: The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for RACH-less handover in a terrestrial network.

# Conclusion

According to the above analysis, we have the following observations and proposals:

Observation 1: The CG configuration for the RACH-less of mobile IAB may require the SFN of the target cell.

Observation 2: The current RRC specification only clarifies that the MIB reading is required for RACH, not for the RACH-less procedure for mobile IAB.

Proposal: The RRC specification is clarified that the UE is required to read MIB if the timing information is required for RACH-less of mobile IAB.

# Reference

[1] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

[2] 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”