3GPP TSG-RAN WG2 Meeting #125bis	R2-2403973

# Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0501 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LPP support for sub 1s location information reporting periodicity [Sub_1s_periodicity] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, T-Mobile, Vivo, Deutsche Telekom, Huawei, HiSilicon, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing support for sub 1s loction information reporting periodicity, which means that the reporting cannot be adapted to positioning reference signal periodicity and report periodicity of related information over other interfaces such as NRPPa cannot be aligned in time. This has been discussed during Rel 17 since March 2023, and companies wanted to await an LS from CT4, which arrived to RAN2#124, got postponed and discussed at RAN2#125, but then it was seen as too late to be considered for Rel 17, and TEI18 was seen as a reasonable alternative.The CR is Cat B, but the discussion is not new. For more details about the history, see [1] R2-2304050 Missing LPP support for sub 1s location information reporting periodicity, Ericsson [2] R2-2304300, Report from [AT121bis-e][427][POS] Rel-17 LPP CRs, Qualcomm [3] RAN2#123 Chairman’s notes [4] RAN2#123-bis Chairman’s notes [5] R2-2312938 Open issues for RAT-dependent integrity, Ericsson [6] TS 38.455 V.16.1.0 NR Positioning Protocol A (NRPPa) [5][7] R2-2311703 , Sub One Second Report Period for Deferred Location over SBI (C4-234472), CT4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following has been added/changed - a new capability to indicate support for sub 1s periodicity, defined per positioning method and positioning mode for applicable positioning methods - a new optional request for sub 1s periodicity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sub 1s periodicity is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.2, 6.5.1.7, 6.5.2.9, 6.5.3.4, 6.5.4.4, 6.5.5.4, 6.5.6.4, 6.5.7.4, 6.5.9.4, 6.5.10.6, 6.5.11.6, 6.5.12.6, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.4.1	Common Lower-Level IEs

[…]

#### –	PeriodicAssistanceDataControlParameters

The IE PeriodicAssistanceDataControlParameters is used in a periodic assistance data delivery procedure as described in clauses 5.2.1a and 5.2.2a.

-- ASN1START

PeriodicAssistanceDataControlParameters-r15 ::= SEQUENCE {

periodicSessionID-r15			PeriodicSessionID-r15,

...,

[[

updateCapabilities-r15		UpdateCapabilities-r15		OPTIONAL	 -- Need ON

]]

}

PeriodicSessionID-r15 ::= SEQUENCE {

periodicSessionInitiator-r15	ENUMERATED { locationServer, targetDevice, ... },

periodicSessionNumber-r15		INTEGER (0..255),

...

}

UpdateCapabilities-r15 ::= BIT STRING {primaryCellID-r15	(0)} (SIZE(1..8))

-- ASN1STOP

PeriodicAssistanceDataControlParameters field descriptions

periodicSessionID This field identifies a particular periodic assistance data delivery session and the initiator of the session.

updateCapabilities This field identifies the capabilities of the sending entity to support an update of periodic assistance data. A bit value set to one indicates a capability is supported and a bit value set to zero indicates a capability is not supported.

#### –	PeriodicReportingIntervalMsSupport

The IE PeriodicReportingIntervalMsSupport is used by the target device to indicate if millisecond reporting intervals are supported by providing the minimum millisecond reporting interval for periodic location information reporting.

-- ASN1START

PeriodicReportingIntervalMsSupport-r18 ::= SEQUENCE {

minPeriodicReportingIntervalMs-r18	ENUMERATED {ms1, ms10, ms100,...},

...

}

-- ASN1STOP

PeriodicReportingIntervalMsSupport field descriptions

minPeriodicReportingIntervalMs This field indicates the minimum sub-second periodic reporting interval supported by the target device, where ms1, ms10, ms100 corresponds to 1, 10, and 100 milliseconds respectively.

#### –	PeriodicReportingIntervalMsSupportPerMode

The IE PeriodicReportingIntervalMsSupportPerMode is used by the target device to indicate if millisecond reporting intervals are supported by providing the minimum millisecond reporting interval for periodic location information reporting for each positioning mode indicated by PositioningModes.

-- ASN1START

PeriodicReportingIntervalMsSupportPerMode-r18 ::= SEQUENCE {

minPeriodicReportingIntervalMs1-Supported-r18	PositioningModes	OPTIONAL,

minPeriodicReportingIntervalMs10-Supported-r18	PositioningModes	OPTIONAL,

minPeriodicReportingIntervalMs100-Supported-r18	PositioningModes	OPTIONAL,

...

}

-- ASN1STOP

PeriodicReportingIntervalMsSupportPerMode field descriptions

minPeriodicReportingIntervalMs1-Supported This field indicates the positioning modes of the target device supporting a minimum of 1 millisecond periodic reporting interval.

minPeriodicReportingIntervalMs10-Supported This field indicates the positioning modes of the target device supporting a minimum of 10 millisecond periodic reporting interval.

minPeriodicReportingIntervalMs100-Supported This field indicates the positioning modes of the target device supporting a minimum of 100 millisecond periodic reporting interval.

[…]

### 6.4.2	Common Positioning

[…]

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationTime-r17

ScheduledLocationTime-r17	OPTIONAL,	-- Need ON

targetIntegrityRisk-r17

TargetIntegrityRisk-r17		OPTIONAL	-- Need ON

]],

[[

periodicalReportingExt-r18	PeriodicalReportingCriteriaExt-r18 OPTIONAL -- Need ON

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...,

locationEstimateAndMeasurementsRequired-r18

}

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

PeriodicalReportingCriteriaExt-r18 ::=		SEQUENCE {

reportingAmount-r18							ENUMERATED {

ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingIntervalMs-r18						INTEGER (1..999),

...

}

TriggeredReportingCriteria ::=		SEQUENCE {

cellChange							BOOLEAN,

reportingDuration					ReportingDuration,

...

}

ReportingDuration ::=				INTEGER (0..255)

AdditionalInformation ::= ENUMERATED {

onlyReturnInformationRequested,

mayReturnAdditionalInformation,

...

}

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

HorizontalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

VerticalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

HorizontalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

VerticalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-v1700 }

OPTIONAL		-- Need ON

]]

}

ResponseTimeNB-r14 ::= SEQUENCE {

timeNB-r14							INTEGER (1..512),

responseTimeEarlyFixNB-r14			INTEGER (1..512)		OPTIONAL,		-- Need ON

...,

[[	unitNB-r15				ENUMERATED { ten-seconds, ... }	OPTIONAL		-- Need ON

]]

}

Environment ::= ENUMERATED {

badArea,

notBadArea,

mixedArea,

...

}

MessageSizeLimitNB-r14 ::= SEQUENCE {

measurementLimit-r14				INTEGER (1..512)		OPTIONAL,		-- Need ON

...

}

ScheduledLocationTime-r17 ::= SEQUENCE {

utcTime-r17			UTCTime											OPTIONAL,	-- Need ON

gnssTime-r17		SEQUENCE {

gnss-TOD-msec-r17		INTEGER (0..3599999),

gnss-TimeID-r17			GNSS-ID

}												OPTIONAL,	-- Need ON

networkTime-r17		CHOICE {

e-utraTime-r17			SEQUENCE {

lte-PhysCellId-r17			INTEGER (0..503),

lte-ArfcnEUTRA-r17			ARFCN-ValueEUTRA,

lte-CellGlobalId-r17		CellGlobalIdEUTRA-AndUTRA

OPTIONAL,	-- Need ON

lte-SystemFrameNumber-r17	INTEGER (0..1023)

},

nrTime-r17				SEQUENCE {

nr-PhysCellID-r17		NR-PhysCellID-r16,

nr-ARFCN-r17			ARFCN-ValueNR-r15,

nr-CellGlobalID-r17		NCGI-r15	OPTIONAL,		-- Need ON

nr-SFN-r17				INTEGER (0..1023),

nr-Slot-r17				CHOICE {

scs15-r17				INTEGER (0..9),

scs30-r17				INTEGER (0..19),

scs60-r17				INTEGER (0..39),

scs120-r17				INTEGER (0..79)

}										OPTIONAL	-- Need ON

},

...

}												OPTIONAL,	-- Need ON

relativeTime-r17	INTEGER (1..1024)								OPTIONAL	-- Need ON

}

TargetIntegrityRisk-r17 ::=		INTEGER (10..90)

-- ASN1STOP

Editor Notes: FFS exact IE structure of the request for location+measurements in the agreement of RAN2#123bis.

| Conditional presence | Explanation |
| --- | --- |
| ECID | The field is optionally present, need ON, if E-CID or NR E-CID is requested. Otherwise it is not present. |

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible. For 'locationEstimateAndMeasurementsRequired', the PRU shall return both location estimate and measurements if possible, or indicate a location error if not possible. NOTE: If the PRU is requested to return both location estimate and measurements, the location information is determined independently of the reported measurements.

triggeredReporting This IE indicates that triggered reporting is requested and comprises the following subfields: - cellChange: If this field is set to TRUE, the target device provides requested location information each time the primary cell has changed. - reportingDuration: Maximum duration of triggered reporting in seconds. A value of zero is interpreted to mean an unlimited (i.e. "infinite") duration. The target device should continue triggered reporting for the reportingDuration or until an LPP Abort or LPP Error message is received. The triggeredReporting field should not be included by the location server and shall be ignored by the target device if the periodicalReporting IE or responseTime IE or responseTimeNB IE is included in CommonIEsRequestLocationInformation.

periodicalReporting This IE indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64 correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a target device is able to obtain new measurements or obtain a new location estimate. The value 'noPeriodicalReporting' shall not be used by a sender.

additionalInformation This IE indicates whether a target device is allowed to return additional information to that requested. If this IE indicates 'onlyReturnInformationRequested' then the target device shall not return any additional information to that requested by the server. If this IE indicates 'mayReturnAdditionalInformation' then the target device may return additional information to that requested by the server. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include E-CID measurements if A-GNSS measurements were requested but not E-CID measurements).

qos This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows: - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE. - unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. If the enumerated value 'ten-milli-seconds' is included for methods others than NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning the target device shall ignore the unit field. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). - responseTimeNB If the periodicalReporting IE or responseTime IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - timeNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. - responseTimeEarlyFixNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFixNB IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the timeNB IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFixNB IE. A server should set the responseTimeEarlyFixNB IE to a value less than that for the timeNB IE. A target shall ignore the responseTimeEarlyFixNB IE if its value is not less than that for the timeNB IE. - unitNB indicates the unit of the timeNB and responseTimeEarlyFixNB fields. Enumerated value 'ten-second' corresponds to a resolution of 10 seconds. If this field is absent, the unit/resolution is 1 second. - horizontalAccuracyExt indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the horizontalAccuracy field is included in QoS. - verticalAccuracyExt indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the verticalAccuracy field is included in QoS. All QoS requirements shall be obtained by the target device to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable. The single exception is time and timeNB which shall always be fulfilled – even if that means not fulfilling other QoS requirements. A target device supporting NB-IoT access shall support the responseTimeNB IE. A target device supporting HA GNSS shall support the HorizontalAccuracyExt, VerticalAccuracyEx, and unit fields with enumerated value 'ten-seconds'. A target device supporting NB-IoT access and HA GNSS shall support the unitNB field.

environment This field provides the target device with information about expected multipath and non line of sight (NLOS) in the current area. The following values are defined: - badArea: possibly heavy multipath and NLOS conditions (e.g. bad urban or urban). - notBadArea: no or light multipath and usually LOS conditions (e.g. suburban or rural). - mixedArea: environment that is mixed or not defined. If this field is absent, a default value of 'mixedArea' applies.

locationCoordinateTypes This field provides a list of the types of location estimate that the target device may return when a location estimate is obtained by the target.

velocityTypes This fields provides a list of the types of velocity estimate that the target device may return when a velocity estimate is obtained by the target.

messageSizeLimitNB This field provides an octet limit on the amount of location information a target device can return. - measurementLimit indicates the maximum amount of location information the target device should return in response to the RequestLocationInformation message received from the location server. The limit applies to the overall size of the LPP message at LPP level (LPP Provide Location Information), and is specified in steps of 100 octets. The message size limit is then given by the value provided in measurementLimit times 100 octets.

segmentationInfo This field indicates whether this RequestLocationInformation message is one of many segments, as specified in clause 4.3.5

scheduledLocationTime This field indicates that the target device is requested to obtain location measurements or location estimate valid at the scheduledLocationTime T and comprises the following subfields: - utcTime provides T in UTC in the form of YYMMDDhhmmssZ. - gnssTime provides T in GNSS system time of the GNSS indicated by gnss-TimeID. - gnss-TOD-msec specifies the GNSS TOD in 1-milli-second resolution rounded down to the nearest millisecond unit. - networkTime provides T in E-UTRA or NR network time. - lte-PhysCellId, lte-ArfcnEUTRA, lte-CellGlobalId identifies the reference cell (E-UTRA) that is used for the network time. - lte-systemFrameNumber specifies the system frame number in E-UTRA. - nr-PhysCellID, nr-ARFCN , nr-CellGlobalID identifies the reference cell (NR) that is used for the network time. - nr-SFN specifies the system frame number in NR. - nr-Slot specifies the slot number in NR for the indicated subcarrier spacing (SCS). The total NR network time is given by nr-SFN + nr-Slot. - relativeTime provides T in seconds from current time, where current time is defined as the time the CommonIEsRequestLocationInformation was received. NOTE 1: A location estimate returned to an LCS Client, AF or UE for a scheduled location time can be treated by the LCS Client, AF or UE as an estimate of the location of the UE at the scheduled location time (see TS 23.273 [42]). NOTE 2: If this field is present, at least one of utcTime, gnssTime, networkTime, or relativeTime shall be present.

targetIntegrityRisk This field indicates the TIR for which the PL is requested. The TIR is calculated by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

periodicalReportingExt This field indicates that extended periodic reporting is requested and comprises the below subfields. If this field is present, the field periodicalReporting is absent. Measurement reports containing no measurements or no location estimate are required when a reportingIntervalMs expires before a target device is able to obtain new measurements or obtain a new location estimate. - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device should continue periodic reporting until an LPP Abort message is received. - reportingIntervalMs indicates the interval between location information reports and the response time requirement for the first location information report in milliseconds.

[…]

#### 6.5.1.7	OTDOA Capability Information

#### –	OTDOA-ProvideCapabilities

The IE OTDOA-ProvideCapabilities is used by the target device to indicate its capability to support OTDOA and to provide its OTDOA positioning capabilities to the location server.

-- ASN1START

OTDOA-ProvideCapabilities ::= SEQUENCE {

otdoa-Mode		BIT STRING {	ue-assisted				(0),

ue-assisted-NB-r14		(1),

ue-assisted-NB-TDD-r15	(2) } (SIZE (1..8)),

...,

supportedBandListEUTRA		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA		OPTIONAL,

supportedBandListEUTRA-v9a0	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9a0

OPTIONAL,

interFreqRSTDmeasurement-r10		ENUMERATED { supported }						OPTIONAL,

additionalNeighbourCellInfoList-r10	ENUMERATED { supported }						OPTIONAL,

prs-id-r14							ENUMERATED { supported }						OPTIONAL,

tp-separation-via-muting-r14		ENUMERATED { supported }						OPTIONAL,

additional-prs-config-r14			ENUMERATED { supported }						OPTIONAL,

prs-based-tbs-r14					ENUMERATED { supported }						OPTIONAL,

additionalPathsReport-r14			ENUMERATED { supported }						OPTIONAL,

densePrsConfig-r14					ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsBandwidth-r14		ENUMERATED { n6, n15, n25, n50, n75, n100, ...}	OPTIONAL,

prsOccGroup-r14						ENUMERATED { supported }						OPTIONAL,

prsFrequencyHopping-r14				ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsConfigs-r14			ENUMERATED { c2, c3 }							OPTIONAL,

periodicalReporting-r14				ENUMERATED { supported }						OPTIONAL,

multiPrbNprs-r14					ENUMERATED { supported }						OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }							OPTIONAL,

numberOfRXantennas-r14				ENUMERATED { rx1, ... }							OPTIONAL,

motionMeasurements-r15				ENUMERATED { supported }						OPTIONAL,

interRAT-RSTDmeasurement-r15		ENUMERATED { supported }						OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupport-r18		OPTIONAL

}

maxBands INTEGER ::= 64

SupportedBandEUTRA ::= SEQUENCE {

bandEUTRA							INTEGER (1..maxFBI)

}

SupportedBandEUTRA-v9a0 ::=		SEQUENCE {

bandEUTRA-v9a0						INTEGER (maxFBI-Plus1..maxFBI2)		OPTIONAL

}

maxFBI								INTEGER	::=	64	-- Maximum value of frequency band indicator

maxFBI-Plus1						INTEGER ::= 65	-- lowest value extended FBI range

maxFBI2								INTEGER ::= 256	-- highest value extended FBI range

-- ASN1STOP

OTDOA-ProvideCapabilities field descriptions

otdoa-Mode This field specifies the OTDOA mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the particular OTDOA mode is supported; a zero value means not supported. A zero-value in all bit positions in the bit string means OTDOA positioning method is not supported by the target device. ue-assisted: Bit 0 indicates that the target device supports UE-assisted OTDOA and LTE PRS. ue-assisted-NB: Bit 1 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS. ue-assisted-NB-TDD: Bit 2 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS for TDD.

SupportedBandEUTRA This field specifies the frequency bands for which the target device supports RSTD measurements. One entry corresponding to each supported E-UTRA band as defined in TS 36.101 [21]. In the case the target device includes bandEUTRA-v9a0, the target device shall set the corresponding entry of bandEUTRA (i.e. without suffix) to maxFBI.

interFreqRSTDmeasurement This field, if present, indicates that the target device supports inter-frequency RSTD measurements within and between the frequency bands indicated in SupportedBandEUTRA.

additionalNeighbourCellInfoList This field, if present, indicates that the target device supports up to 3×24 OTDOA-NeighbourCellInfoElement in OTDOA-NeighbourCellInfoList in OTDOA-ProvideAssistanceData without any restriction for the earfcn in each OTDOA-NeighbourCellInfoElement as specified in clause 6.5.1.2.

prs-id This field, if present, indicates that the target device supports PRS generation based on the PRS-ID as specified in TS 36.211 [16] and support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

tp-separation-via-muting This field, if present, indicates that the target device supports RSTD measurements for cells which have associated transmission points (e.g., Remote Radio Heads) within the cell coverage and where these associated transmission points have the same physical cell identity as the associated cell, and where these transmission points are identified via a different muting pattern. The field also indicates support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

additional-prs-config This field, if present, indicates that the target device supports additional PRS configurations. The additional PRS configuration in PRS-Info IE comprise: - support for prs-ConfigurationIndex > 2399; - support for NPRS values in addition to 1, 2, 4 and 6 (add-numDL-Frames in PRS-Info); - support for muting bit string lengths > 16 bits.

prs-based-tbs This field, if present, indicates that the target device supports RSTD measurements for PRS-only TPs.

additionalPathsReport This field, if present, indicates that the target device supports reporting of timing information for additional detected paths for RSTD reference and each neighbour cell.

densePrsConfig This field, if present, indicates that the target device supports a subset of the additional PRS configurations associated with capability additional-prs-config which comprises: - support for prs-ConfigurationIndex > 2404; - support for NPRS values of 10, 20, 40, 80 and 160 (in addition to 1, 2, 4 and 6). In the case additional-prs-config is present, this field is not present.

maxSupportedPrsBandwidth This field, if present, indicates the maximum PRS bandwidth supported by the target device. Enumerated value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If this field is not present, the target device is assumed to support the PRS bandwidth associated with the target device type, which for LTE devices including Cat-M1/M2 is 100 resource blocks and for NB-IoT devices is 1 resource block.

prsOccGroup This field, if present, indicates that the target device supports PRS occasion groups, which implies that each bit of a configured muting pattern applies per PRS occasion group.

prsFrequencyHopping This field, if present, indicates that the target device supports PRS occasion frequency hopping, as specified in TS 36.211 [16].

maxSupportedPrsConfigs This field, if present, indicates that the target device supports multiple PRS configurations per cell. Enumerated value c2 indicates support for up to 2 configurations; c3 indicates support for up to 3 configurations.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of RSTD measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

multiPrbNprs This field, if present, indicates that the target device supports NPRS configuration in more than one resource block (i.e., maxCarrier in PRS-Info-NB greater than 1).

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform RSTD measurements.

numberOfRXantennas This field is not applicable to NB-IoT devices. This field, if present, indicates the number of UE downlink receive antennas for RSTD measurements (see TS 36.133 [18]). Enumerated value rx1 indicates a single antenna receiver. If this field is absent, the target device is assumed to support two RX antennas for RSTD measurements.

motionMeasurements This field, if present, indicates that the target device supports reporting of motion measurements (delta-SFN and motionTimeSource) in OTDOA-SignalMeasurementInformation. The presence of this field implies presence of sensor-MotionInformationSup in IE Sensor-ProvideCapabilities.

interRAT-RSTDmeasurement This field, if present, indicates that the target device supports inter-RAT RSTD measurements (TS 38.215 [36]); i.e., E-UTRA RSTD measurements when the target device is served by an NR cell.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports millisecond periodic reporting intervals for location information – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.2.9	GNSS Capability Information

#### –	A-GNSS-ProvideCapabilities

The IE A-GNSS-Provide-Capabilities is used by the target device to indicate its capability to support A-GNSS and to provide its A-GNSS location capabilities (e.g., GNSSs and assistance data supported) to the location server.

-- ASN1START

A-GNSS-ProvideCapabilities ::= SEQUENCE {

gnss-SupportList			GNSS-SupportList				OPTIONAL,

assistanceDataSupportList	AssistanceDataSupportList		OPTIONAL,

locationCoordinateTypes		LocationCoordinateTypes			OPTIONAL,

velocityTypes				VelocityTypes					OPTIONAL,

...,

[[ periodicalReportingNotSupported-r14

PositioningModes				OPTIONAL,

idleStateForMeasurements-r14

ENUMERATED { required }			OPTIONAL

]],

[[ periodicAssistanceData-r15

BIT STRING { solicited	 (0),

unsolicited (1)	} (SIZE (1..8))		OPTIONAL

]],

[[ scheduledLocationRequestSupported-r17

ScheduledLocationTimeSupportPerMode-r17				OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

GNSS-SupportList ::= SEQUENCE (SIZE(1..16)) OF GNSS-SupportElement

GNSS-SupportElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-IDs						SBAS-IDs					OPTIONAL,	-- Cond GNSS-ID-SBAS

agnss-Modes						PositioningModes,

gnss-Signals					GNSS-SignalIDs,

fta-MeasSupport					SEQUENCE {

cellTime	AccessTypes,

mode		PositioningModes,

...

}							OPTIONAL,	-- Cond fta

adr-Support						BOOLEAN,

velocityMeasurementSupport		BOOLEAN,

...,

[[

adrEnhancementsSupport-r15	ENUMERATED { true }			OPTIONAL,

ha-gnss-Modes-r15			PositioningModes			OPTIONAL

]],

[[

ha-gnss-MetricsSupport-r17	ENUMERATED { true }			OPTIONAL

]]

}

AssistanceDataSupportList ::= SEQUENCE {

gnss-CommonAssistanceDataSupport	GNSS-CommonAssistanceDataSupport,

gnss-GenericAssistanceDataSupport	GNSS-GenericAssistanceDataSupport,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| fta | The field is mandatory present if the target device supports the reporting of fine time assistance measurements; otherwise it is not present. |

A-GNSS-ProvideCapabilities field descriptions

gnss-SupportList This field specifies the list of GNSS supported by the target device and the target device capabilities associated with each of the supported GNSS. This field shall be present if the gnss-SupportListReq in the A-GNSS -RequestCapabilities IE is set to TRUE and if the target device supports the A-GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports the assisted GNSS positioning method.

gnss-ID This field specifies the GNSS supported by the target device for which the capabilities in GNSS-SupportElement are provided.

sbas-IDs This field specifies the SBAS(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular SBAS is supported; a zero-value means not supported.

agnss-Modes This field specifies the GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

gnss-Signals This field specifies the GNSS signal(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS signal type is supported; a zero-value means not supported.

fta-MeasSupport This field specifies that the target device is capable of performing fine time assistance measurements (i.e., GNSS-cellular time association reporting). The cellTime field specifies for which cellular network(s) this capability is supported. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the specific cellular network time is supported; a zero-value means not supported. The mode field specifies for which GNSS mode(s) FTA measurements are supported by the target device. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the GNSS mode is supported; a zero-value means not supported.

adr-Support This field specifies whether the target device supports ADR measurement reporting. TRUE means supported.

velocityMeasurementSupport This field specifies whether the target device supports measurement reporting related to velocity. TRUE means supported.

assistanceDataSupportList This list defines the assistance data and assistance data choices supported by the target device. This field shall be present if the assistanceDataSupportListReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports GNSS assistance data. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports any GNSS assistance data.

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target device supports for GNSS. TRUE indicates that a location coordinate type is supported and FALSE that it is not. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

velocityTypes This parameter identifies the velocity types that a target device supports for GNSS. TRUE indicates that a velocity type is supported and FALSE that it is not. FALSE for all velocity types indicates that velocity reporting is not supported. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

periodicalReportingNotSupported This field, if present, specifies the positioning modes for which the target device does not support periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is not supported; a zero-value means supported. If this field is absent, the location server may assume that the target device supports periodicalReporting in CommonIEsRequestLocationInformation for each supported positioning mode.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform GNSS measurements.

periodicAssistanceData This field identifies the periodic assistance data delivery procedures supported by the target device. This is represented by a bit string, with a one value at the bit position means the periodic assistance data delivery procedure is supported; a zero value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a.

adrEnhancementsSupport This field, if present, indicates that the target device supports the fields adrMSB, adrSign, adrRMSerror, and delta-codePhase in IE GNSS-MeasurementList. This field may only be present if adr-Support is set to TRUE, and shall be absent if adr-Support is set to FALSE.

ha-gnss-Modes This field specifies the High-Accuracy GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

ha-gnss-MetricsSupport This field specifies that high accuracy GNSS positioning metrics are supported by the target device.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.3.4	E-CID Capability Information

#### –	ECID-ProvideCapabilities

The IE ECID-ProvideCapabilities is used by the target device to indicate its capability to support E-CID and to provide its E-CID location capabilities to the location server.

-- ASN1START

ECID-ProvideCapabilities ::= SEQUENCE {

ecid-MeasSupported	BIT STRING {	rsrpSup		(0),

rsrqSup		(1),

ueRxTxSup	(2),

nrsrpSup-r14	(3),

nrsrqSup-r14	(4)} (SIZE(1..8)),

...,

[[	ueRxTxSupTDD-r13					ENUMERATED { true }				OPTIONAL

]],

[[	periodicalReporting-r14				ENUMERATED { supported }		OPTIONAL,

triggeredReporting-r14				ENUMERATED { supported }		OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }			OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17	OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupport-r18		OPTIONAL

]]

}

-- ASN1STOP

ECID-Provide-Capabilities field descriptions

ecid-MeasSupported This field specifies the E-CID measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Cell ID positioning method is supported by the target device. If the UE Rx-Tx time difference measurement is supported by the target device (i.e., ueRxTxSup field is set to one), it means that the UE supports the UE Rx-Tx time difference measurement reporting via both LPP signaling and RRC signalling. If a target device doesn't support LPP, the E-SMLC may assume the target device can not report the UE Rx-Tx time difference measurement results via RRC signalling.

ueRxTxSupTDD This field, if present, indicates that any UE Rx-Tx time difference measurement reporting for TDD from the target device includes the NTAoffset according to TS 36.211 [16], TS 36.214 [17] and uses the UE Rx-Tx time difference measurement report mapping for TDD as specified in TS 36.133 [18]. This field may only be included if the ueRxTxSup field in ecid-MeasSupported is set to value one.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of E-CID measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

triggeredReporting This field, if present, indicates that the target device supports triggeredReporting for the cellChange event. If this field is absent, the location server may assume that the target device does not support triggeredReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform E-CID measurements.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports millisecond periodic reporting intervals for location information – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.4.4	TBS Capability Information

#### –	TBS-ProvideCapabilities

The IE TBS-ProvideCapabilities is used by the target device to indicate its capability to support TBS and to provide its TBS location capabilities to the location server.

-- ASN1START

TBS-ProvideCapabilities-r13 ::= SEQUENCE {

tbs-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1),

ue-based		(2)} (SIZE (1..8)),

...,

[[	mbs-AssistanceDataSupportList-r14	MBS-AssistanceDataSupportList-r14		OPTIONAL,

periodicalReportingSupported-r14	PositioningModes						OPTIONAL,

mbs-ConfigSupport-r14	BIT STRING {	tb1		(0),

tb2		(1),

tb3		(2),

tb4		(3)} (SIZE (1..8))			OPTIONAL,

mbs-IdleStateForMeasurements-r14		ENUMERATED { required }				OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17	OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

-- ASN1STOP

TBS-ProvideCapabilities field descriptions

tbs-Modes This field specifies the TBS mode(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular TBS mode is supported; a zero-value means not supported.

mbs-AssistanceDataSupportList This list defines the MBS assistance data supported by the target device. This field shall be present if the target device supports MBS assistance data.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

mbs-ConfigSupport This field specifies the MBS configurations supported by the target device. This field shall be present if the target device supports MBS [24].

mbs-IdleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform MBS measurements.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

#### -	MBS-AssistanceDataSupportList

The IE MBS-AssistanceDataSupportList is used by the target device to indicate its capability to support MBS Assistance Data and to provide its capabilities to the location server.

-- ASN1START

MBS-AssistanceDataSupportList-r14 ::= SEQUENCE {

mbs-AcquisitionAssistanceDataSupport-r14	BOOLEAN,

mbs-AlmanacAssistanceDataSupport-r14		BOOLEAN,

...

}

-- ASN1STOP

MBS-AssistanceDataSupportList field descriptions

mbs-AcquisitionAssistanceDataSupport This field specifies whether the target device supports MBS Acquisition Assistance Data. TRUE means supported.

mbs-AlmanacAssistanceDataSupport This field specifies whether the target device supports MBS Almanac Assistance Data. TRUE means supported.

[…]

#### 6.5.5.4	Sensor Capability Information

#### –	Sensor-ProvideCapabilities

The IE Sensor-ProvideCapabilities is used by the target device to provide capabilities for sensor-based methods from to the location server.

-- ASN1START

Sensor-ProvideCapabilities-r13 ::= SEQUENCE {

sensor-Modes-r13			BIT STRING {	standalone	(0),

ue-assisted	(1),

ue-based	(2)} (SIZE (1..8)),

...,

[[	sensor-AssistanceDataSupportList-r14	Sensor-AssistanceDataSupportList-r14	OPTIONAL,

periodicalReportingSupported-r14		PositioningModes						OPTIONAL,

idleStateForMeasurements-r14			ENUMERATED { required }					OPTIONAL

]],

[[	sensor-MotionInformationSup-r15			ENUMERATED { true }						OPTIONAL

]],

[[	adjustmentSupported-r16					ENUMERATED	{ true }					OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17	OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

Sensor-AssistanceDataSupportList-r14 ::= SEQUENCE {

...,

[[	validityPeriodSupported-v1520			ENUMERATED { true }		OPTIONAL,

validityAreaSupported-v1520				ENUMERATED { true }		OPTIONAL

]]

}

-- ASN1STOP

Sensor-ProvideCapabilities field descriptions

sensor-Modes This field specifies the sensor mode(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular sensor mode is supported; a zero-value means not supported.

sensor-AssistanceDataSupportList This field specifies a list of sensor assistance data supported by the target device. This field shall be present if the target device supports assistance data for Barometric pressure sensor.

validityPeriodSupported This field, if present, indicates that the target device supports period i.e. pressure validity period and pressure rate as part of the Sensor-AssistanceDataList.

valitidyAreaSupported This field, if present, indicates that the target device supports area i.e. pressure validity area and North/East pressure gradient as part of the Sensor-AssistanceDataList.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform sensor measurements.

sensor-MotionInformationSup This field, if present, indicates that the target device supports displacement reporting in IE Sensor-MotionInformation.

adjustmentSupported This field, if present, indicates that the target device supports the adjustment IE in Sensor-MeasurementInformation.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.6.4	WLAN Capability Information

#### –	WLAN-ProvideCapabilities

The IE WLAN-ProvideCapabilites is used by the target device to provide its capabilities for WLAN positioning to the location server.

-- ASN1START

WLAN-ProvideCapabilities-r13 ::= SEQUENCE {

wlan-Modes-r13			BIT STRING	{	standalone		(0),

ue-assisted		(1),

ue-based		(2)}	(SIZE (1..8)),

wlan-MeasSupported-r13	BIT STRING	{

rssi-r13		(0),

rtt-r13			(1)}	(SIZE(1..8)),

...	,

[[	wlan-AP-AD-Supported-r14

BIT STRING {	ap-identifier	(0),

ap-location		(1)}	(SIZE (1..8))

OPTIONAL,

periodicalReportingSupported-r14	PositioningModes						OPTIONAL,

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17	OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

-- ASN1STOP

WLAN-ProvideCapabilities field descriptions

wlan-Modes This field specifies the WLAN mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the WLAN mode is supported; a zero value means not supported.

wlan-MeasSupported This field specifies the measurements supported by the target device when accessing a WLAN. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic WLAN positioning method is supported by the target device which is reporting of the WLAN identity. The following bits are assigned for the indicated measurements. rssi: AP signal strength at the target rtt: Round Trip Time between target and AP

wlan-AP-AD-Supported This field specifies the WLAN AP assistance data supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. A zero-value in all bit positions or absence of this field means no assistance data is supported. The following bits are assigned for the indicated assistance data. ap-identifier: WLAN AP identity information ap-location: WLAN AP location information

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform WLAN measurements.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.7.4	Bluetooth Capability Information

#### –	BT-ProvideCapabilities

The IE BT-ProvideCapabilites is used by the target device to provide its capabilities for Bluetooth positioning to the location server.

-- ASN1START

BT-ProvideCapabilities-r13 ::= SEQUENCE {

bt-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1),

ue-based-v1810	(2)}	(SIZE (1..8)),

bt-MeasSupported-r13	BIT STRING {	rssi-r13		(0),

aod-v1800		(1)}	(SIZE (1..8)),

...,

[[

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL,

periodicalReportingSupported-r14

PositioningModes										OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL

]],

[[	bt-AoA-r18								ENUMERATED { request-only, suggestion }		OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18	OPTIONAL

]]

}

-- ASN1STOP

BT-ProvideCapabilities field descriptions

bt-Modes This field specifies the Bluetooth mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the Bluetooth mode is supported; a zero value means not supported.

bt-MeasSupported This field specifies the Bluetooth measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Bluetooth positioning method is supported by the target device which is reporting of the Bluetooth beacon identity. The following bits are assigned for the indicated measurements. - rssi: Bluetooth beacon signal strength at the target device. - aod: Bluetooth beacon AoD at the target device.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform BT measurements.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

bt-AoA This field, if present, indicates that the target device supports Bluetooth AoA, where: - request-only: Target device supports to provide its Bluetooth AoA transmission configuration to LMF upon request. - suggestion: Target device supports obtaining a suggestion from LMF about device Bluetooth AoA transmission configuration and device supports to provide its Bluetooth AoA transmission configuration based on the suggestion back to LMF.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.9.4	NR E-CID Capability Information

#### –	NR-ECID-ProvideCapabilities

The IE NR-ECID-ProvideCapabilities is used by the target device to indicate its capability to support NR E-CID and to provide its NR E-CID positioning capabilities to the location server.

-- ASN1START

NR-ECID-ProvideCapabilities-r16 ::= SEQUENCE {

nr-ECID-MeasSupported-r16		BIT STRING {	ssrsrpSup		(0),

ssrsrqSup		(1),

csirsrpSup		(2),

csirsrqSup		(3)} (SIZE(1..8)),

periodicalReporting-r16		ENUMERATED { supported }							OPTIONAL,

triggeredReporting-r16		ENUMERATED { supported }							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17		ENUMERATED { supported }				OPTIONAL,

scheduledLocationRequestSupported-r17

ScheduledLocationTimeSupport-r17		OPTIONAL

]],

[[ periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupport-r18		OPTIONAL

]]

}

-- ASN1STOP

NR-ECID-ProvideCapabilities field descriptions

nr-ECID-MeasSupported: Indicates the supported NR ECID measurements: - ssrsrpSup indicates the UE supports SSB based cell/beam specific RSRP measurement; - ssrsrqSup indicates the UE supports SSB based cell/beam specific RSRQ measurement; - csirsrpSup indicates the UE supports CSI-RS based cell/beam specific RSRP measurement; - csirsrqSup indicates the UE supports CSI-RS based cell/beam specific RSRQ measurement.

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports millisecond periodic reporting intervals for location information – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.10.6	NR DL-TDOA Capability Information

#### –	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

trpTEG-InfoSup	(3),

integritySup-r18 (4),

pruInfoSup-r18	(5)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,								...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

multiLocationEstimateInSameMeasReport-r17	ENUMERATED { supported }				OPTIONAL

]],

[[

locationCoordinateTypes-r18				LocationCoordinateTypes						OPTIONAL,

symbolTimeStampSupport-r18				ENUMERATED { supported }					OPTIONAL,

periodicAssistanceData-r18				BIT STRING { solicited	 (0),

unsolicited (1)} (SIZE (1..8))	OPTIONAL,

nr-IntegrityAssistanceSupport-r18		BIT STRING {

serviceParametersSup-r18		(0),

serviceAlertSup-r18				(1),

riskParametersSup-r18			(2),

integrityParaTRP-LocSup-r18		(3),

integrityParaBeamInfoSup-r18	(4),

integrityParaRTD-InfoSup-r18	(5)

} (SIZE (1..8)) 					OPTIONAL,	nr-DL-TDOA-OnDemandPRS-ForBWA-Support-r18	ENUMERATED { supported }				OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field. - bit 4 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 4 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 4 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not. - bit 5 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report.

locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for UE-based DL-TDOA. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

symbolTimeStampSupport This field, if present, indicates that the target device supports reporting timestamp in terms of radio frame timing down to OFDM symbol level.

periodicAssistanceData This field identifies the periodic NR assistance data delivery procedures supported by the target device. This is represented by a bit string, with a one value at the bit position means the periodic NR assistance data delivery procedure is supported; a zero value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a.

nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not. - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-DL-TDOA-OnDemandPRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]

#### 6.5.11.6	NR DL-AoD Capability Information

#### –	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

integritySup-r18	(4)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

multiLocationEstimateInSameMeasReport-r17	ENUMERATED { supported }				OPTIONAL

]],

[[

locationCoordinateTypes-r18				LocationCoordinateTypes						OPTIONAL,

nr-DL-AoD-PosIntegritySupport-r18		ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-OnDemandPRS-ForBWA-Support-r18	ENUMERATED { supported }				OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18		OPTIONAL

]]

}

-- ASN1STOP

| NR-DL-AoD-ProvideCapabilities field descriptions |
| --- |
| nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device. |
| periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation. |
| ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation. |
| nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not. - bit 4 indicates whether the target service supports the range of integrity risk (IR) for which the integrity assistance data are valid. |
| nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2,dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2 . Otherwise, the UE does not include this field. |
| nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData. |
| dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset. |  |
| nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData. |
| nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests. |
| nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. |
| scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests. |
| nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports. |
| multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report. |
| mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report. |
| locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for UE-based DL-AoD. TRUE indicates that a location coordinate type is supported and FALSE that it is not. |
| nr-DL-AoD-PosIntegritySupport This field, if present, indicates that the target device supports the RAT-dependent positioning integrity for DL-AoD. |
| nr-DL-AoD-OnDemandPRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation. |
| periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting. |

[…]

#### 6.5.12.6	NR Multi-RTT Capability Information

#### –	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its NR Multi-RTT positioning capabilities to the location server.

-- ASN1START

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					ENUMERATED { supported }					OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,	nr-Multi-RTT-On-Demand-DL-PRS-Support-r17

NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-UE-RxTx-TEG-ID-ReportingSupport-r17	BIT STRING {	case1	(0),

case2	(1),

case3	(2)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType2-r17,

granularity-r17		LOS-NLOS-IndicatorGranularity2-r17,

...

}												OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)		OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

symbolTimeStampSupport-r18				ENUMERATED { supported }					OPTIONAL,

nr-MultiRTT-OnDemandPRS-ForBWA-Support-r18	ENUMERATED { supported }				OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupport-r18		OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideCapabilities field descriptions

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-Multi-RTT-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-UE-RxTx-TEG-ID-ReportingSupport This field, if present, indicates that the target device supports nr-UE-RxTx-TEG-Info reporting in IE NR-Multi-RTT-SignalMeasurementInformation. This is represented by a bit string, with a one-value at the bit position means the particular case is supported; a zero-value means not supported: - bit 0 indicates that the target device supports the 'case1' choice in NR-UE-RxTx-TEG-Info. - bit 1 indicates that the target device supports the 'case2' choice in NR-UE-RxTx-TEG-Info. - bit 2 indicates that the target device supports the 'case3' choice in NR-UE-RxTx-TEG-Info.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-Multi-RTT-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

symbolTimeStampSupport This field, if present, indicates that the target device supports reporting timestamp in terms of radio frame timing down to OFDM symbol level.

nr-multiRTT-OnDemandPRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

periodicReportingIntervalMsSupport This field, if present, indicates that the target device supports millisecond periodic reporting intervals for location information – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting.

[…]