3GPP TSG-RAN WG2 Meeting #126	                     		     R2-2404433

Fukuoka, Japan, 20th – 24th May, 2024

Source:	CATT

Title:	TPs for the Reply LS on questions on RAN1 parameter list

Agenda Item:	7.2.1

Document for:	Discussion and Decision

1 Introduction

The reply LS on questions on RAN1 parameter list was sent to RAN2 in R2-2404112 so the LPP should be updated accordingly.

Question 1: Does the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx also apply to NR DL-AoD positioning?

RAN1’s Response: Yes, the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx is also applicable to NR DL-AoD positioning.

Question 2: Is there only one dl-PRS-ID or are there multiple dl-PRS-IDs associated with the aggregated main and additional measurement, respectively?

RAN1’s Response: In RAN1’s understanding, if nr-DL-PRS-ResourceSetID uniquely identifies the DL-PRS Resource Sets across all the multiple dl-PRS-IDs of the same TRP, then only one dl-PRS-ID is needed to be associated with the aggregated main measurement and additional measurements. Otherwise, multiple dl-PRS-IDs are associated with the aggregated main and additional measurement in the measurement report.

Question 3: If there are multiple dl-PRS-IDs associated with main and additional measurements, respectively, should the list of the dl-PRS-IDs in additional measurements be included in the list of dl-PRS-IDs in the main measurement?

RAN1’s Response: There is no need to provide dl-PRS-ID(s) in additional measurement reporting since both main measurement and additional measurement are derived from the same TRP.

2 Discussion

2.1 Hopping applies to NR DL-AoD positioning

According to the answer from RAN1, Rx hopping should be applied to DL-AoD as well.

Question 1: Does the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx also apply to NR DL-AoD positioning?

RAN1’s Response: Yes, the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx is also applicable to NR DL-AoD positioning.

The TP based on the agreed LPP CR in R2-2403818 to enable Rx hopping is provided as below:

-----------------------Start of TP1--------------------------------------------------------------------------------------------------

6.5.11.4	NR DL-AoD Location Information Elements

–	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL-AoD measurements to the location server.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

[…]

]],

[[

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }	OPTIONAL

]]

}

[…]

NR-DL-AoD-AdditionalMeasurementElement-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17				NR-DL-PRS-ResourceID-r16		OPTIONAL,

nr-DL-PRS-ResourceSetID-r17				NR-DL-PRS-ResourceSetID-r16		OPTIONAL,

nr-TimeStamp-r17						NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r17			INTEGER (0..30)					OPTIONAL, -- Cond rsrp

nr-DL-PRS-RxBeamIndex-r17				INTEGER (1..8)					OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL, -- Cond rsrpp

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

...,

[[

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx-r18

ENUMERATED { singleHop, multipleHop }				OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| rsrpp | The field is mandatory present if the field nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 is absent; otherwise it is optionally present, need ON. |
| rsrp | The field is mandatory present if the field nr-DL-PRS-RSRP-ResultDiff-r17 is absent; otherwise it is optionally present, need ON. |

NR-DL-AoD-SignalMeasurementInformation field descriptions

[..]

nr-ReportDL-PRS-MeasBasedOnSingleOrMultiHopRx This field indicates that the reported measurement is based on receiving single or multiple hops of DL-PRS.

[…]

6.5.11.5	NR DL-AoD Location Information Request

–	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...,

[[

multiMeasInSameReport-r17					ENUMERATED { requested }	OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-RxHoppingRequest-r18			SEQUENCE {

nr-DL-PRS-RxHoppingTotalBandwidth-r18		CHOICE {

fr1											ENUMERATED {mhz40, mhz50, mhz80, mhz100},

fr2											ENUMERATED {mhz100, mhz200, mhz400}

}																		OPTIONAL -- Need ON

}																			OPTIONAL -- Need ON

]]

}

[…]

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

[…]

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to perform DL-PRS Rx hopping for performing RSRP or RSRPP measurements and report the hopping information used for performing the measurements.

[…]

-----------------------End of TP 1--------------------------------------------------------------------------------------------------

Proposal 1: RAN2 to agree the TP 1 above to support Rx hopping in DL-AoD.

2.2 DL-PRS-ID(s) report associated with the aggregated measurement

RAN1 gave the answers on the associated DL-PRS-ID in aggregated measurement as below:

Question 2: Is there only one dl-PRS-ID or are there multiple dl-PRS-IDs associated with the aggregated main and additional measurement, respectively?

RAN1’s Response: In RAN1’s understanding, if nr-DL-PRS-ResourceSetID uniquely identifies the DL-PRS Resource Sets across all the multiple dl-PRS-IDs of the same TRP, then only one dl-PRS-ID is needed to be associated with the aggregated main measurement and additional measurements. Otherwise, multiple dl-PRS-IDs are associated with the aggregated main and additional measurement in the measurement report.

Question 3: If there are multiple dl-PRS-IDs associated with main and additional measurements, respectively, should the list of the dl-PRS-IDs in additional measurements be included in the list of dl-PRS-IDs in the main measurement?

RAN1’s Response: There is no need to provide dl-PRS-ID(s) in additional measurement reporting since both main measurement and additional measurement are derived from the same TRP.

It is concluded that multiple dl-PRS-IDs can be associated with the aggregated main and additional measurement in the measurement report. But there is no need to provide dl-PRS-ID(s) in additional measurement reporting.

The TP 2 based on the agreed LPP CR in R2-2403818 to align with the answer from RAN1 is provided as below.

-----------------------Start of TP 2--------------------------------------------------------------------------------------------------

6.4.3	Common NR Positioning Information Elements

–	NR-AggregatedDL-PRS-ResourceSetID-Element

The IE NR-AggregatedDL-PRS-ResourceSetID-Element is used by the target device to indicate each of the aggregated DL-PRS Resource Set to the location server.

-- ASN1START

NR-AggregatedDL-PRS-ResourceSetID-Element-r18 ::= SEQUENCE {

aggregatedDL-PRS-ID-r18					INTEGER (0..255)			OPTIONAL,

nr-DL-PRS-ResourceSetID-r18		NR-DL-PRS-ResourceSetID-r16,

...

}

-- ASN1STOP

[…]

–	NR-DL-TDOA-SignalMeasurementInformation

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-AggregatedDL-PRS-ResourceSetID-List This field provides the DL-PRS Resource Set IDs which are used for the aggregated RSTD, RSRP, or RSRPP measurement results. This field is optionally present if the field nr-MeasBasedOnAggregatedResources is present; otherwise, it is not present. If the field is present, the field nr-DL-PRS-ResourceID and nr-DL-PRS-ResourceSetID should not be included, and the dl-PRS-ID in IE NR-DL-TDOA-MeasElement shall be ignored by a receiver. When nr-AggregatedDL-PRS-ResourceSetID-List is included in NR-DL-TDOA-AdditionalMeasurementElement, the aggregatedDL-PRS-ID is not present in the nr-AggregatedDL-PRS-ResourceSetID-List.

[…]

6.5.12.4	NR Multi-RTT Location Information Elements

–	NR-Multi-RTT-SignalMeasurementInformation

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-AggregatedDL-PRS-ResourceSetID-List This field provides the DL-PRS Resource Set IDs which are used for the aggregated UE Rx-Tx time difference, RSRP, or RSRPP measurement results. This field is optionally present if the field nr-MeasBasedOnAggregatedResources is present; otherwise, it is not present. If the field is present, the field nr-DL-PRS-ResourceID and nr-DL-PRS-ResourceSetID should not be included, and the dl-PRS-ID in IE NR-Multi-RTT-MeasElement shall be ignored by a receiver. When nr-AggregatedDL-PRS-ResourceSetID-List is included in NR-Multi-RTT-AdditionalMeasurementElement, the aggregatedDL-PRS-ID is not present in the nr-AggregatedDL-PRS-ResourceSetID-List.

-----------------------End of TP 2--------------------------------------------------------------------------------------------------

Proposal 2: RAN2 to agree the TP 2 above for the aggregated report.

The RIL H006 which is Todo is related with this reply LS. According to the answer from RAN1, this RIL can be closed.

Proposal 3: RAN2 to agree set the RIL H006 “Rejected”.

3 Conclusions

In this contribution, we have discusses the LPP impact and potential solutions based on the RAN1 answers.

Proposal 1: RAN2 to agree the TP 1 above to support Rx hopping in DL-AoD.

Proposal 2: RAN2 to agree the TP 2 above for the aggregated report.

Proposal 3: RAN2 to agree set the RIL H006 “Rejected”.

4 Reference

- . R2-2403818 Corrections to TS 37.355	CATT  CR	Rel-18	37.355	18.1.0	0500  1	F	NR_pos_enh2-Core

- . R2-2404112 Reply LS on questions on RAN1 parameter list (R1-2403636; contact: CATT)   RAN1	LS in	Rel-18	NR_pos_enh2-Core	To: RAN2	   Cc: RAN3, RAN4