# 3GPP TSG RAN WG2 Meeting #126      	                           R2-2404664
Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	7.3.3

Source:	Apple

Title:	Discussion on remaining issue on UE capability of aperiodicCSI-RS-FastScellActivation

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#125b [1], we discussed UE capability on inter-band SSB-less CA, and below agreement was made with one FFS left:

Agreements on UE capabilities

=>	  In the field description of capability aperiodicCSI-RS-FastScellActivation-r17, clarify that it is applied to  inter-band SSB-less SCell activation.  FFS whether it is also applied to intra-band SSB-less SCell activation

In this contribution, we share our view.

# 2 Discussion

After checking with our RAN4 colleagues, our understanding is that:

- Rel-15 only supports intra-band contiguous SSB-less CA.

- RAN4 is discussing whether to supports intra-band non-contiguous SSB-less CA. But there is no consensus.

- UE capability aperiodicCSI-RS-FastScellActivation-r17 is applied to intra-band contiguous SSB-less SCell activation. Whether it is also applied to intra-band non-contiguous SSB-less SCell activation depends on RAN4.

Thus, we propose the following clarification:

Proposal 1: On UE capability aperiodicCSI-RS-FastScellActivation-r17, it is applied to intra-band contiguous SSB-less SCell activation. RAN2 wait RAN4 conclusion on whether it is also applied to intra-band non-contiguous SSB-less SCell activation.

# 3 Conclusion

In this contribution, we discuss remaining issue on UE capability of aperiodicCSI-RS-FastScellActivation. We propose:

Proposal 1: On UE capability aperiodicCSI-RS-FastScellActivation-r17, it is applied to intra-band contiguous SSB-less SCell activation. RAN2 wait RAN4 conclusion on whether it is also applied to intra-band non-contiguous SSB-less SCell activation.

# 4 References

[1] RAN2#125b, Chair Notes