3GPP TSG-RAN WG2 Meeting #126	R2-2404750

# Fukuoka, Japan, 20th - 24th May., 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1886 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to mandatory 80ms scheduling offset for positioning SI acquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During RAN2#125bis, the following CR for NR has been agreed in principle. R2-2403553 Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition Ericsson CR Rel-16 38.306 16.16.0 1087 - F NR_pos-Core - Replace “for UEs which support the posSIB types” with “for UEs which support acquisition of the posSIB types”. - Agreed in principle with this change as R2-2403797, with shadow CRs in R2-2403798 and R2-2403799 For LTE, we think the same also needs to be applied since NR and LTE share the common mechanism for positioning SIB broadcast. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when the UE supports posSIB reception, the UE should also support the 80ms scheduling offest for positioning capability mandatorily without capability signaling. Impact analysis Impacted functionality: Positioning SI Inter-operability: There is no inter-operability issue for the CR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing mandatory capability information which may lead UE to not implemnt it and thus unable to receive the desired positioning SIBs and Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR .. |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

7.8	Positioning features

7.8.1	OTDOA Inter-frequency RSTD measurement indication

It is mandatory to support delivery of InterFreqRSTDMeasurementIndication as specified in TS 36.331 [5], clause 5.5.7 for UEs indicating support for inter-frequency RSTD measurements for OTDOA as specified in TS 36.355 [13] and requiring measurement gaps for performing these measurements.

7.8.2	Acqusition of positioning SI message with 80ms offset

It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 36.331 [5].

End of Change