3GPP TSG-RAN WG2 Meeting #126                                                        R2-2404945

Fukuoka, Japan, 20th - 24th May, 2024

Source:              CATT

Title:	             Discussion on the unaligned SFN for Cell DTX/DRX in MAC

Agenda Item: 	7.3.2

Document for:	Discussion and Decision

- Introduction

In current spec, if the Cell DTX/DRX is configured and activated, UE can calculate the start time of celldtxdrx-onDurationTimer based on the SFN. But it is possible that the SFN across carriers in a cell group is not unaligned. Then the calculated start time of Cell DTX/DRX duration is unclear. In this contribution, we share our view on the issue of unaligned SFN for Cell DTX/DRX.

- Discussion

During the previous RAN2 #123bis, #124 meetings, following agreements have been achieved on cell DTX/DRX configuration:

Agreements (123 bis)

Cell DTX/DRX configuration is provided per Serving Cell with the following restrictions:

- 	 A maximum of two cell DTX/DRX patterns can be configured per MAC entity

- 	 The two configured patterns are aligned,

o	   The start and slot offset are common for the two patterns.

o	   one periodicity is an integer multiple of the other.

Agreement (124)

From UE capability’s perspective, the supported number of cell DTX/DRX patterns per cell group is two, regardless of each pattern is for cell DTX only, cell DRX only, or both (i.e. remove the FFS)

According to the above agreement, the start time of the celldtxdrx-onDurationTimer of different cells is the same no matter whether the Cell DTX/DRX pattern are the same or different for different cells. Based on the current MAC spec, it is possible that the SFN across carrier in a cell group may be not aligned. Then if UE calculate the start time of Cell DTX/DRX duration based on each cell’s SFN, the start time of cell DTX/DRX duration may be different which is not aligned with the agreements. Besides, if we do not specify which cell’s SFN used, the calculated start time of cell DTX/DRX duration is unclear which may lead to different start times of cell DTX/DRX duration between the UE and the network. So it is necessary to specify which cell’s SFN is used to calculate the cell DTX/DRX duration.

A similar issue also exists for DRX and there is a note added about unaligned SFN across carriers in a cell group for DRX as below:

5.7 Discontinuous Reception (DRX)

…

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

So we can also add a similar clarification for cell DTX/DRX to resolve the issue of unaligned SFN, such as: NOTE: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration. And we also provide the corresponding TP in Appendix.

Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration.

- Conclusion

According to the analysis in section 2, it is proposed that:

Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration.

- Reference

- TS38.321, Medium Access Control (MAC) protocol specification (Release 18), 3GPP

- Appendix

  - Text proposal for 38.321

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

5.34.2	Cell Discontinuous Transmission

Cell DTX is configured if cellDTXDRXconfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DTX is configured and cellDTXDRXactivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if CellDTXDRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

NOTE: 	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX duration.

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DRX duration.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>