3GPP TSG-RAN WG2 Meeting #126	R2-2405270

Fukuoka, Japan May 20th – 26th, 2024

Agenda item:	8.2.5 A-IoT Random Access

Source: 	Semtech

Title: 	Considerations for Random Access

Document for:	Discussion/Decision

# Introduction

In RAN#102, the A-IoT Rel-19 SID [2] was approved based on the Ambient IoT RAN led study technical report [1]. The SID was updated in RAN #103 [3]. The motivation of Ambient IoT is to support devices that are ultra-low cost and ultra-low power. This tdoc discusses the considerations for random access.

# Contention resolution and access failure

In the RAN2 #125bis [4] it was agreed that the random-access procedure is triggered by the reader:

Agreement

- RAN2 confirms slotted-ALOHA is the baseline for Ambient IoT random access

- We will study the support for access triggering for a single device, group of devices, or all devices.    RAN2 to discuss the contention-based and contention-free access procedures and detailed solutions.

- Random Access is triggered by the reader

- Reader provides the information that the device needs to respond to the random access trigger.  FFS what those parameters are

- Study the solution and benefits of both 2-step like random access procedure and 4-step like random access procedure.  FFS the details on each procedure and how we call it.

- Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access.  FFS details

- For the very first access message from the device to reader in random access an ID is included.  RAN2 to discuss whether a temporary identifier is included, or the permanent device ID is included (considering other WGs input as well).

With LTE and NR, devices autonomously trigger the random-access procedure, which naturally reduces the probability of a two or more devices colliding during the random access. With A-IoT, the random access is triggered by the reader, so it is not as clear how the devices avoid collisions. In the use cases targeted for A-IoT, such as inventory control, there is a high probably that a lot of devices will need to access the system simultaneously. For example, when a truck or pallet of new devices for inventory registration arrives. This sudden surge in new devices can overload the system. One solution maybe for the A-IoT device to select a random delay after the trigger before starting the random-access transmission. The delay value range could be provided to the A-IoT device from the reader.

Study how ambient IoT devices can reduce collisions after a random-access trigger from the reader.

# Summary

- Study how ambient IoT devices can reduce collisions after a random-access trigger from the reader.

# References

TR 38.848 Study on Ambient IoT (Internet of Things) in RAN, 2023

RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”

RAN2 #125bis, RAN2 Chair notes, April 2024.