3GPP TSG-RAN WG2 Meeting #126	R2- 2405325

Fukuoka, Japan May 20th– 24th, 2024

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes

Document for:	Discussion

# 1. Introduction

In RAN2#125bis [1], following workding assumption and agreement were made:

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed. Working assumption: the LP-WUS configuration in SIB at least includes the following information: - - LP-SS configuration - - LP-WUS configuration - - FFS on Entry/exit condition for LP-WUS monitoring The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

In this contribution, we discuss LP-WUS operation in RRC_IDLE/INACTIVE mode.

# 2. Discussion

## 2.1 Dedicated configuration for LP-WUS

For LP-WUS monitoring, an LP-WUR UE needs to receive LP-WUS configuration (e.g., monitoring occasion, LP-SS configuration). RAN2 agreed to receive LP-WUS configuration via system information, however, a necessity of dedicated configuration is still FFS. From our view, dedicated LP-WUS configuration from a gNB is not valid since an idle/inactive LP-WUR UE is allowed to camp on another cell via cell (re-)selection procedure [3].

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Does not support dedicated LP-WUS configuration.

## 2.2 Subgrouping for LP-WUS

In RAN1#116bis, the following agreements were made on subgrouping and PEI [2].

Agreement Regarding the LP-WUS information for idle/inactive UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] subgroups - Option 2: A codepoint value corresponding to one or more subgroup(s) - Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s) - Combination of above options are not precluded - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - Other options are not precluded

Following the legacy PEI, subgrouping is an efficient power saving operation to reduce false alarms and monitor paging occasions in IDLE/INACTIVE modes. Similar to the legacy subgrouping method, subgroup based wake-up indication is supported by LP-WUS. The contents of the indication is upto RAN1 since the LP-WUS signal design, payload and maximum number of subgroups have been discussed by RAN1.

Proposal 2: For PEI based subgrouping, detailed LP-WUS information (e.g., bitmap, codepoint) and maximum number of subgroups are up to RAN1 decision.

## 2.3 Entry/Exit condition

RAN1 made the following working assumptions [2].

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

In the last RAN2 meeting [1], an workding assumption was made that LP-WUS configuration in SIB includes FFS on entry/exit condition for LP-WUS monitoring. In the last RAN1 meeting [2], RAN1 made working assumption for entry/exit condition. When the RAN1 agrees entry/exit condition based on the working assumption, RAN2 also agree that entry/exit conditions for LP-WUS monitoring can be included in the SIB, i.e., always configured by gNB.

Proposal 3: When RAN1 agrees entry/exit conditions, RAN2 agrees to include entry/exit condition in the SIB.

Proposal 4: Support procedure that entry/exit conditions of LP-WUS monitoring are configured by gNB (e.g., SIB).

To support entry/exit conditions for LP-WUS monitoring, the UE is configured with conditions. For example, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS. (e.g., detail threshold value is upto RAN1 decision). The satisfied condition (e.g., above the threshold) of received LP-WUS can be regarded successful reception and can maintain entry condition. In contrast, if the condition is not satisfied, LP-WUS reception may be failed. Upon detecting the number of reception failures, the UE stops LP-WUS monitoring and start to monitor the legacy PO that the exit condition is fulfilled.

Proposal 5: For LP-WUS exit condition, support a number of LP-WUS reception failures. The number is FFS.

While measuring of entry/exit conditions, a UE being capable of OFDM-based LP-WUR can measure both NR-SS (existing SSB) and LP-SS (i.e., if supported) simultaneously. The measured values can be used for the decision of entry/exit condition. Since the qualities of NR-SS and LP-SS are different metrics (e.g., RSRP and LP-RSRP, respectively), RAN2 should discuss how to use the different metrics for the decisions. For example, using the higher value or combined values.

Proposal 6: For OFDM-based WUS, consider measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Support LP-WUS configuration via system information (e.g., cell-specific). Does not support dedicated LP-WUS configuration.

Proposal 2: For PEI based subgrouping, detailed LP-WUS information (e.g., bitmap, codepoint) and maximum number of subgroups are up to RAN1 decision.

Proposal 3: When RAN1 agrees entry/exit conditions, RAN2 agrees to include entry/exit condition in the SIB.

Proposal 4: Support procedure that entry/exit conditions of LP-WUS monitoring are configured by gNB (e.g., SIB).

Proposal 5: For LP-WUS exit condition, support a number of LP-WUS reception failures. The number is FFS.

Proposal 6: For OFDM-based WUS, consider measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #125-bis.

[2] Chairman’s notes, 3GPP TSG RAN WG1 #116-bis.

[3] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.