3GPP TSG-RAN WG2 #126	R2-2405449

Fukuoka, Japan, May 20 – 24, 2024

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Downlink coverage enhancements

Document for:	Discussion/Decision

# 1	Introduction

In this contribution, we discuss the objective of downlink coverage enhancements [1].

The justification for downlink coverage enhancements are:

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS.

DL coverage enhancements can be considered at both

- Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design.

- System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

And the objective is:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

- Notes for this objective:

  - SSB channel enhancement is not considered

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Rel-18 network energy saving techniques should be considered as baseline in the system level study

# 2	Discussion

This objective shall be studied and specified if beneficial.

DL coverage enhancements shall only be specified if studies show it to be beneficial.

The area to evaluate is Satellite beam power sharing and feeder link rate limits, including link and system level enhancements, and the R18 network energy saving techniques are the baseline.

This will require and definition of KPIs and potential effects on the coverage.

Last meeting RAN2 made these agreements:

Agreements:

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

However, we believe there are no RAN2 aspects of DL coverage enhancements that are not dependent on the evaluation in RAN1.

For example, cell/beam level on/off mechanism for solving the EIRP reduction issue:

- On/off is just endpoints of a varying power per cell, what about power sharing (as stated in the WID) to allow each cell to send SIBs and serve some traffic (using part of the cell resources).

- There needs to be an evaluation of what the benefits are and whether coverage can be provided by some hierarchical scheme when a beam is off or some time sharing to provide accessibility

- Comparison to NES is needed

RAN1 has already started extensive evaluations, see the Appendix.

Therefore, RAN2 needs to wait the results in RAN1.

RAN2 waits for RAN1 input on downlink coverage enhancements.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	DL coverage enhancements shall only be specified if studies show it to be beneficial.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 waits for RAN1 input on downlink coverage enhancements.

# 4	References

RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” 3GPP TSG RAN#103, Mar. 2024.

# Appendix RAN1 agreements

### RAN1#116

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

### RAN1#116bis

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19 |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.2 | 72 bits |
| Payload size for PDSCH of SIB1 | FFS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Payload size for PDSCH of SIB19 | FFS |
| Other parameters | Reported by companies. |

Agreement

Antenna gain reduction due to steering loss is not considered in the link level evaluation.

Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement

Observation

The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

Agreement

For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed:

| Payload | value |
| --- | --- |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| Payload size for PDSCH of SIB19 | 616 bits |

Note: At least the above values are simulated and reported. Other values can be considered.

Note: the values above are not the TBS.

Agreement

For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2:

Beam size

Satellite EIRP density /beam (dBW/MHz)

Payload Total DL power level (dBW)

Aggregated EIRP (Total) (dBW)

Satellite Tx max Gain

EIRP per Satellite beam (dBW)

Agreement

For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.