3GPP TSG-RAN WG2 Meeting #126	R2-2405590

Fukuoka, Japan, 20 – 24 May 2024

Agenda item:	7.3.2

Source:	Samsung

Title:	CellDTRX-RNTI description in TS 38.300

Document for:	Discussion & Decision

1. Introduction

This document discusses about a correction on CellDTRX-RNTI description in TS 38.300.

2. Discussion

In clause 8.1 of TS 38.300, all UE identities used by NR connected to 5G are listed and simple descriptions are captured for each feature or purpose. Rel-18 NES WI has introduced CellDTRX-RNTI as a new type of RNTI used to receive DCI format 2-9. We see CellDTRX-RNTI part is missing in the current specification, so the relevant description should be provided for consistency of the specification.

As a reference, we can reuse similar texts for UE power saving-related RNTIs as follows:

For UE power saving purpose, the following identities are used: - PS-RNTI: used to determine if the UE needs to monitor PDCCH on the next occurrence of the connected mode DRX on-duration; - PEI-RNTI: used to determine if the UE needs to monitor the associated PO.

CellDTRX-RNTI can be used for two purposes: 1) activation/deactivation of cell DTX/DRX and 2) triggering NES CHO. We do not need detailed descriptions which could be duplicate with stage-3 specifications. In our view, a simple description is enough, for instance:

For network energy saving purpose, the following identity is used: - CellDTRX-RNTI: identification used for network energy saving indication.

Proposal. A description of CellDTRX-RNTI is added in clause 8 of TS 38.300. The corresponding TP is added in the Annex.

3. Conclusion

RAN2 is requested to discuss and agree the following proposal:

Proposal. A description of CellDTRX-RNTI is added in clause 8 of TS 38.300. The corresponding TP is added in the Annex.

Annex: Text Proposal for 38.300

8	NG Identities

8.1	UE Identities

In this clause, the identities used by NR connected to 5GC are listed. For scheduling at cell level, the following identities are used:

-	C-RNTI: unique UE identification used as an identifier of the RRC Connection and for scheduling;

-	CG-SDT-CS-RNTI: unique UE identification used for Configured Grant-based SDT in the uplink;

-	CI-RNTI: identification of cancellation in the uplink;

-	CS-RNTI: unique UE identification used for Semi-Persistent Scheduling in the downlink or configured grant in the uplink;

-	INT-RNTI: identification of pre-emption in the downlink;

-	MCS-C-RNTI: unique UE identification used for indicating an alternative MCS table for PDSCH and PUSCH;

-	P-RNTI: identification of Paging and System Information change notification in the downlink;

-	SI-RNTI: identification of Broadcast and System Information in the downlink;

-	SP-CSI-RNTI: unique UE identification used for semi-persistent CSI reporting on PUSCH.

For power and slot format control, the following identities are used:

-	SFI-RNTI: identification of slot format;

-	TPC-PUCCH-RNTI: unique UE identification to control the power of PUCCH;

-	TPC-PUSCH-RNTI: unique UE identification to control the power of PUSCH;

-	TPC-SRS-RNTI: unique UE identification to control the power of SRS.

During the random access procedure, the following identities are also used:

-	RA-RNTI: identification of the Random Access Response in the downlink;

-	MSGB-RNTI: identification of the Random Access Response for 2-step RA type in the downlink;

-	Temporary C-RNTI: UE identification temporarily used for scheduling during the random access procedure;

-	Random value for contention resolution: UE identification temporarily used for contention resolution purposes during the random access procedure.

For NR connected to 5GC, the following UE identity is used at NG-RAN level:

-	I-RNTI: used to identify the UE context in RRC_INACTIVE.

For UE power saving purpose, the following identities are used:

-	PS-RNTI: used to determine if the UE needs to monitor PDCCH on the next occurrence of the connected mode DRX on-duration;

-	PEI-RNTI: used to determine if the UE needs to monitor the associated PO.

For IAB the following identity is used:

-	AI-RNTI: identification of the DCI carrying availability indication for soft symbols of an IAB-DU.

For Network-Controlled Repeater the following identity is used:

-	NCR-RNTI: identification of the DCI carrying side control information.

For MBS, the following identities are used:

-	G-RNTI: Identifies dynamically scheduled PTM transmissions of MTCH(s);

-	G-CS-RNTI: Identifies configured scheduled PTM transmissions of MTCH(s) scheduled with configured grant;

-	MCCH-RNTI: Identifies transmissions of MCCH and MCCH change notification for broadcast reception.

-	Multicast MCCH-RNTI: Identifies transmissions of MCCH and MCCH change notification for multicast reception in RRC_INACTIVE state.

For sidelink, the following identities are used:

-	SL-RNTI: unique UE identification used for NR sidelink communication scheduling;

-	SL-CS-RNTI: unique UE identification used for configured sidelink grant for NR sidelink communication;

-	SL Semi-Persistent Scheduling V-RNTI: unique UE identification used for semi-persistent scheduling for V2X sidelink communication;

-	SL-PRS-RNTI: unique UE identification used for SL-PRS transmission scheduling on dedicated SL-PRS resource pool;

-	SL-PRS-CS-RNTI: unique UE identification used for configured sidelink grant for SL-PRS transmission on dedicated SL-PRS resource pool.

For network energy saving purpose, the following identity is used:

-	CellDTRX-RNTI: identification used for network energy saving indication.