3GPP TSG-RAN WG2 Meeting #126	R2-2405703

Fukuoka, Japan, May 20th-24th, 2024

Agenda item:	9.3

Source: 	Session Chair (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT126][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3], [7.0] R18 common (Diana) [7.0.1][7.0.2] [7.0.3] ASN.1 Review common Break out of ASN.1 Review [7.0.4][7.0.5] @NR151617 UP (Diana) | Breakout to start after common session including ASN.1 review NRR1617 SL (Kyeongin) NR18 SL (Kyeongin) | Breakout to start after common session including ASN.1 review: Rel-18 MUSIM (Erlin) NR18 MIMO evo |  |
| 11:00 – 13:00 |  |  |  |  |
| 14:30 – 16:30 | Rel-18 UAV (1hr) NR18 TEI (Diana) SDT, including MT-SDT and related TEI18 | NR R18 SL | 14:30-15:30 NR18 fCovEnh (Eswar) NRLTE1516 Pos (Nathan) NR17 Positioning and SL Relay (Nathan) |  |
| 17:00 – 19:00 | NR18 URLLC (Diana) NR18 Network Energy Saving (Diana) | NR18 Mobile IAB (Johan) NR18 feMob (Johan) | NR18 Pos |  |
| Tuesday |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob (Johan) | NR19 XR [1] (Dawid) | EUTRA&NR151617 (Mattias) |  |
| 11:00 – 13:00 | NR18 XR (Diana) | NR18 NTN NR /IoT(Sergio) - all AIs in 7.7 - all AIs in 7.6 | NR18 Pos (Nathan) |  |
| 14:30 -16:30 | NR19 AI/ML PHY [2] (Diana) | NR19 feMob [2] (Kyeongin) | NR17/18 Relay (Nathan) |  |
| 17:00– 19:00 | Rel-19 Ambient IoT [2] (Diana) | Rel- 19 NR NTN [1] - 8.8.1 - 8.8.2 - 8.8.4 | NR 18 MBS (Dawid) NR18 eQoE (Dawid) |  |
| Wednesday |  |  |  |  |
| 08:30 – 10:30 | NR19 Network Energy Saving [1] (Kyeongin) | NR19 NTN IoT - 8.9.1 - 8.9.2 - 8.9.3 | EUTRA&NR151617 (Mattias) NR18 Redcap (Mattias) |  |
| 11:00 – 13:00 | NR Other (Diana) TEI18 (Diana) | NR19 XR [1] (Dawid) | NR18 SONMDT (Mattias) NR19 SONMDT [0.5] (Mattias) |  |
| 14:30 – 16:30 | AI/ML Mobility [2] (Diana) | Rel-19 LP-WUS [1](Erlin) | NR18 Pos (Nathan) |  |
| 17:00 – 19:00 | AI/ML PHY [2] (Diana) | NR19 feMob [2] (Kyeongin) | Positioning or SL relay offlines for Rel-18 |  |
| Thursday |  |  |  |  |
| 08:30 – 10:30 | CB TBD Johan/Diana/Eswar | R18 NR/IoT NTN CB (Sergio) - TBD [R19 IoT CB]? | CB Kyeongin Comebacks SL [R19 NES CB] |  |
| 11:00 – 13:00 | Rel-19 Ambient IoT [2] (Diana) | Rel-19 NTN NR [1] (Sergio) - 8.8.5 - 8.8.6 | CB Dawid: - R18 QoE. MBS [R19 XR CB] |  |
| 14:30 – 16:30 | CB Diana UAV NES XR [R19 AI/ML PHY CB] | CB Johan - mIAB - feMob | CB Nathan |  |
| 17:00 – 19:00 | AI/ML Mobilitly [2] (Diana) | CB Erlin R18 CBs [R19 LP-WUS CB] CB Eswar | CB Nathan |  |
| Friday |  |  |  |  |
| 08:30 – 10:30 | CB Johan TBD CB Diana TEI 18 CBs NR Others CBs [R19 AI/ML Mobility]? | CB TDB | CB Mattias TBD |  |
| 11:00 – 13:00 | CB Diana [R19 Ambient IoT]? ASN.1 Review common session Reports from breakout sessions EoM | CB Sergio TBD | TBD? |  |
| 14:30 – 16:00 |  |  |  |  |
| 16:00 – 17:00 |  |  |  |  |

List and details of [AT126] offline discussions

NOTE:  No offline email discussions will be kicked off before Monday May 20th, 09:00 local time

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Rapporteur CRs

R2-2405452	IoT NTN Kmac correction	Ericsson	CR	Rel-17	36.300	17.7.0	1402	-	F	LTE_NBIOT_eMTC_NTN-Core

To be handled in AI 4.1.1 (if anything is agreed for R17, then a R18 Cat A mirror CR needs to be agreed as well, while only R18-specific changes should be kept in a separate R18 Cat F CR)

R2-2405453	IoT NTN Kmac and measurment corrections	Ericsson, Huawei	CR	Rel-18	36.300	18.1.0	1401	1	F	IoT_NTN_enh-Core	R2-2403776

(R18 IPA CR subject to changes if a R17 Cat F CR with a mirror R18 Cat A CR will be agreed)

R2-2405128	Corrections to IOT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5021	-	F	IoT_NTN_enh-Core

R2-2405129	IOT NTN ASN1 RIL List	Huawei, HiSilicon	report	Rel-18	IoT_NTN_enh-Core

R2-2404954	Corrections on UE behaviour on DRX for IoT NTN	MediaTek	CR	Rel-18	36.321	18.1.0	1585	2	F	IoT_NTN_enh-Core	R2-2404007

Moved here from 7.6.4

R2-2405534	Miscellaneous correction for IoT-NTN	Nokia	CR	Rel-18	36.304	18.1.0	0873	2	F	IoT_NTN_enh-Core	R2-2403768

R2-2405302	Corrections on terminology fixed cell	Qualcomm Incorporated	CR	Rel-18	36.306	18.1.0	1889	-	F	IoT_NTN_enh-Core

Withdrawn

R2-2404687	Corrections on terminology fixed cell	Qualcomm Incorporated	CR	Rel-18	36.331	18.1.0	5018	-	F	IoT_NTN_enh-Core	Withdrawn

### 7.6.2	Stage 2 corrections

R2-2404593	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	The Differential Koffset is cleared when UE is performing GNSS measurement.

Proposal 2	Adopt the above TP in stage-2 spec if proposal 1 is agreed.

Proposal 3	UE capability of GNSS measurements during C-DRX inactive time is not reported to the network.

Already agreed in the last meeting??

R2-2405117	Remaining issues on GNSS operation	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

- Whether UE goes to IDLE

Proposal 1: Update the wording in Stage 2 to cover the autonomous GNSS measurement cases as follows “Upon outdated GNSS position the UE shall move to idle mode, unless GNSS acquisition was triggered or being performed or uplink transmission extension is active.”

Proposal 2: Upon GNSS validity expiry, UE doesn’t move to idle mode if UE is performing autonomous GNSS measurement during C-DRX inactive time.

- Whether UE triggers RACH

Proposal 3: RACH is not triggered for the GNSS Validity Duration report if there is no UL resource available for the case of autonomous GNSS measurement in C-DRX inactive time.

R2-2405152	Stage 2 corrections related to NTN measurements	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Agree Stage 2 text proposal in Section 5.1 on measuring NTN cells in a terrestrial network.

Proposal 2: If none of the satellite IDs for a specific frequency in SIB3/SIB5 are present in SIB33, it is up to UE implementation to measure the frequency.

Proposal 3: Agree Stage 2 correction text.

### 7.6.3	RRC Corrections

T390

- [S068] (T390 behaviour for HO/CHO)

R2-2405151	[S068] Handling GNSS uplink transmission extension during handover	Samsung, ZTE, Google	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: Release 17 IoT NTN never specified requirements for UE to perform GNSS measurement during a handover.

Proposal 1: Any solution shall ensure that a handover execution to target cell does not occur if UE does not have a valid GNSS position fix.

Proposal 2: RAN2 considers the following solutions:

-	A) Network does not trigger handover while T390 is running

-	B) A GNSS position fix is performed before the handover execution if T390 is running.

Proposal 3: If B) in P2 is agreed, agree the text proposal in Section 5.

R2-2405499	[S068] UE behaviours on T390 upon handover or CHO	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: In handover and CHO, target cell can know whether UE has started T390 based on the absence of gnss-ValidityDuration in the RRC connection reconfiguration complete message.

Proposal 1: Clarify in the spec that UE only includes gnss-ValidityDuration in the RRC connection reconfiguration complete message when it has a valid GNSS. There is no need to specify the stop of T390 upon handover or CHO.

Proposal 2: Clarify in the spec that T390 is stopped if UE autonomously starts GNSS measurements during C-DRX non-active periods.

Proposal 3: Adopt the TP in the Annex.

R2-2405440	On left open issues for T390 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

- T390 behavior for HO/CHO

Observation 1: If the UE stops T390 when handing over to the target cell the UE will be regarded as uplink un-synchronized.

Observation 2: The target cell is not aware whether T390 is running in a UE performing handover to the cell.

Observation 3: For normal handover, the source cell can inform the UE’s T390 status to the target cell via inter-node message.

Observation 4: For conditional handover, the UE can report the UE’s T390 status to the target cell via Uu interface after CHO execution.

Proposal 1: For HO/CHO, the UE shall report the UE’s T390 status (e.g., the remaining value of the T390) to the target cell in the RRCConnectionReconfigurationComplete message.

Proposal 2: If Proposal1 is agreed, RAN2 to adopt the below text proposal for TS 36.331.

- T390 behavior for for C-DRX non-active periods

Observation 5: The UE and cell must have a common understanding of the T390 status to avoid the cell assumes an autonomous GNSS measurement has started or UE has moved to RRC Idle.

Proposal 3: The UE stops the T390 when it receives an acknowledgement of the cell receiving the new remaining GNSS validity duration, when the UE performed a successful GNSS measurement during a C-DRX non-active period.

Proposal 4: If Proposal3 is agreed, RAN2 to adopt the below text proposal for TS 36.331.

R2-2404157	Further Discussion on T390	vivo	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: There is no requirement that UE may need to acquire the valid GNSS position before HO or CHO, which is different than the RRC re-establishment case.

Observation 2: For HO or CHO, if T390 is stopped upon starting handover, the out-of-date GNSS position will be used to synchronize to the target cell, and UE does not have the opportunity to acquire a new valid GNSS position.

Observation 3: For HO or CHO, if T390 is not stopped upon starting handover, although the out-of-date GNSS position is used to synchronize to the target cell before T390 expiration, UE would have the opportunity to acquire a new valid GNSS position after T390 expiration.

Observation 4: Network implementation can ensure T390 not running and UE has a valid GNSS position during the handover procedure.

Proposal 1: T390 is not stopped upon starting HO or CHO (no spec changed).

R2-2404653	T390 handling during mobility [S067][S068]	Apple	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: If T390 expires when UE is performing GNSS acquisition, UE would go to RRC idle before RRC re-establishment is initiated.

Proposal 1: For RRC re-establishment procedure, T390 is stopped once GNSS acquisition is initiated.

Observation 2: In handover procedure, target cell cannot inherit network UL timing correction from source cell during T390.

Observation 3: When UE performs initial access to target cell, UE location is required for UE to perform pre-compensation on timing and frequency.

Proposal 2: RAN2 to down-select one of the two options on T390 handling during handover and CHO.

- Option 1: Network initiates GNSS measurement before handover and CHO to guarantee UE has a valid GNSS upon handover and CHO initiation.

- Option 2: UE should acquire GNSS location upon initiating handover and CHO if T390 is running.

Proposal 3: If Option 2 above is selected, T390 is stopped once GNSS acquisition is initiated during handover and CHO.

R2-2405526	Corrections on the conditions for stopping T390	Google Inc.	discussion	Rel-18	36.331

Proposal 1	UE stops T390 (if it is running) upon performing the GNSS measurement, no matter whether the GNSS measurement is triggered by the network, performed by the UE using an autonomous gap, or performed by the UE using available idle periods.

Proposal 2	Replace the changes relevant to the conditions for stopping T390 in R2-2403774 with the text proposal in Section 3.

Proposal 3	RAN2 to discuss the exceptions (e.g., a very short GNSS position fix duration) that allow UE to remain in RRC_CONNECTED when UE’s GNSS position becomes outdated, even if the UE is neither configured with autonomous GNSS measurement nor provided with an aperiodic GNSS measurement gap.

- RACH triggering during T390

R2-2404594	Discussion on RACH triggering during T390 running	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Neighbour Satellite information in MO

R2-2404896	Redundant ephemeris info and other issues in MO for IoT NTN	ZTE Corporation, Sanechips, Apple, Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: To remove IE ephemerisInfo-r18 from MeasObjectEUTRA and keep the IE satelliteId-r18 as it is.

Proposal 2: To clarify in the field description of IE epochTime-r18 in MeasObjectEUTRA that this IE is only assiocaited with IE referenceLocation-r18, not applicable to ephemeris information. And the previous description “configured in the associated ReportConfigEUTRA” for reference location is no longer correct and can be removed.

Proposal 3: To clarify that the condition “OPTIONAL	-- Cond Moving” for the IE epochTime-r18 and referenceLocation-r18 in MeasObjectEUTRA means the corresponding field is only mandatory present if EventD2 or condEventD2 is configured. Otherwise the field is not present.

Proposal 4: To clarify the description of Event D2 and CondEvent D2 that the referenceLocation's corresponding satellite ephemeris information is the one indicated by the satellite ID. Also to editorially align the description of Ml2 and Thresh2 with that of Event D2 and CondEvent D2.

R2-2404592	Removing ephemeris from dedicated signaling for EMC neighbor cells	OPPO	CR	Rel-18	36.331	18.1.0	5017	-	F	IoT_NTN_enh-Core

Summary of change: Remove the ephemerisInfo-r18 from measObjectEUTRA.

R2-2404209	On the necessity of satellite assistance information for measurement in IoT NTN	CATT	discussion

Observation 1: Different from NR NTN, IoT NTN does not support SMTC based mechanism for RRM measurement; thus a UE does not need to adjust the measurement timing window with the variation of PDD. This makes it questionable on whether/how the satellite assistance information of neighbour cell is useful/necessary for RRM measurements in RRC_IDLE/INACTIVE or in RRC_CONNECTED.

Proposal 1: RAN2 clarifies how/why the satellite assistance information is necessary/useful for RRM measurement in IoT NTN. If no necessity is identified, remove the satellite ID list in SIB3, SIB5 and MeasObjectEUTRA.

[H005]

R2-2405498	[H005] Start of autonomous GNSS measurements	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Update the start condition of autonomous GNSS gap in clause 5.3.3.21 and 5.5.9: UE may re-acquire GNSS (when configured by the network) in the GNSS measurement timer, if eNB does not trigger UE to make GNSS measurement within duration T, where T is latest reported remaining GNSS validity duration plus UL transmission extension duration X (if any).

Proposal 2: Adopt the TP in the Annex.

Misc

Moved here from 7.6.4

R2-2404408	Remaining issues on GNSS operation in IoT NTN	CATT	discussion

Proposal 1: No need to reset the value of N_TA before resuming UL operation after GNSS is fixed.

Proposal 2: Add the start condition "when indication from lower layers to extend UL transmission is received" for T390 in Clause 7.3.1.

Proposal 3: Add the stop condition "when the UE starts GNSS measurement" for T318 in Clause 7.3.1.

Proposal 4: Adopt the TP in Appendix.

R2-2405528	Further RRC corrections for IoT NTN	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: With current implementation UE cannot signal feature for both GSO and NGSO.

Proposal 1: Agree H005.

Proposal 2: Agree E805 and E806.

### 7.6.4	Other Stage 3 corrections

Corrections related to other specs, e.g. 36.321, 36.304, 36.306.

MAC issues

Moved here from 7.6.3

R2-2404686	Remaining issues on out-of-date GNSS fix	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	UE resets the value of N_TA before resuming UL operation after GNSS is fixed. Text proposal is provided above.

Proposal 2	Add stop criteria for timer T390 as “upon receiving indication that new GNSS is valid”.

Proposal 3	If the UE receives GNSS measurement trigger too early (i.e., remaining GNSS validity is still x minutes), the UE is allowed not to trigger the GNSS measurement but trigger the GNSS Validity Duration Report. Text proposal is provided above.

R2-2404900	Remaining issues of MAC spec for IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 assume the exiting way for maintaining N_TA after GNSS is fixed is enough, no enhancement is needed.

Proposal 2: It’s no need to enhance the process for the UE to report remaining GNSS Validity Duration after successful GNSS measurement during C-DRX.

Moved here from 7.6.3

R2-2405198	NTA handling after GNSS fix	NEC	discussion	Rel-18	IoT_NTN_enh-Core

Proposal: reset NTA to 0 after GNSS fix

R2-2405441	Further discussion on UE behaviour after successful GNSS acquisition	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: The value of N_TA is not clear after a UE has completed the GNSS measurement successfully.

Observation 2: If the UE transmits on PUSCH resources to the network after a GNSS measurement, the following PUSCH transmissions may cause interference to other UEs until the NW corrects any potential TA errors after sending multiple Timing Advance Commands MAC CE.

Observation 3: The network can correct large TA errors via the Random Access Response message.

Proposal 1: After a successful GNSS measurement /autonomous GNSS measurement, the UE shall  perform the Random Access procedure for UL synchronization using N_TA = 0.

Proposal 2: Up to UE implementation to decide when to report the GNSS Validity Duration Report MAC CE after a successful GNSS measurement during C-DRX inactive time.

Proposal 3: Add a Note to indicate when the UE should report the new remaining GNSS validity duration to NW after a successful GNSS measurement in C-DRX inactive time.

Proposal 4: If Proposal 3 is agreed, adopt the Note to TS 36.331.

R2-2405451	R18 IoT NTN GNSS extension	Ericsson	discussion	Rel-17	IoT_NTN_enh-Core

Observation 1	The UE shall trigger GNSS measurement reporting every time upon completing GNSS fix operation.

Observation 2	The GNSS Duration Report MAC CE shall not trigger SR; instead CBRA shall be used.

Observation 3	If the UE has a grant for new transmission, it will send the GNSS validity duration report MAC CE on that grant.

Observation 4	The current MAC spec does not always trigger CBRA to report the GNSS validity duration.

Proposal 1	Correct the MAC procedure to make it clear that a triggered GNSS validity duration report always trigger CBRA in line with the RAN2 agreements.

Proposal 2	Consider the MAC text proposal in section 3.1

Observation 5	The network use NTA to compensate for errors due to estimation of UE and satellite’s position/common TA.

Observation 6	Setting NTA=0 after GNSS reacquisition is not optimal as the NTA value, configured by the network, may compensated for more than the UE position error.

Observation 7	Alt A, C and D do not provide optimal performance.

Observation 8	The UE can calculate the timing error due to inaccurate UE position by comparing the values of N"TA,adjUE"  based on its previous GNSS position and its new GNSS position after GNSS reacquisition, i.e., Terror_UE_position=N"TA,adjUE"-N"TA,adj[OLD]UE" .

Proposal 3	After a new GNSS position fix, set NTA=NTA_old-Terror_UE_position where Terror_UE_position=N"TA,adj UE"-N"TA,adj[OLD]UE"  is the timing error due to inaccurate UE position and is calculated from N"TA,adj [OLD]UE"  based on the previous GNSS position and N"TA,adjUE"  based on the new GNSS position.

Proposal 4	Consider the MAC text proposal in section 3.2.

R2-2405454	IoT NTN MAC corrections	Ericsson	CR	Rel-18	36.321	18.1.0	1586	-	F	IoT_NTN_enh-Core

[E805][E806]

R2-2404843	[E805][E806] Revert implementation of UE capability for HARQ and GNSS	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	RIL [H002] proposed to modify the capability for HARQ and GNSS to distinguish between GSO and NGSO cases.

Observation 2	In RAN2#125, RAN2 swiftly agreed to implement the change with the only rationale to align with Rel-17 design and latest agreements reflected in the “UE Feature list”,, without due consideration.

Observation 3	RAN2’s decision to modify the UE capability for HARQ and GNSS contradicts RAN1 consensus without technical justification nor appropriate communication.

Proposal 1	Revert the implementation of [H002] and send an LS to RAN1 to communicate this correction.

Withdrawn

R2-2405530	Miscellaneous correction for IoT-NTN	Nokia, Nokia Shanghai Bell	CR	Rel-18	36.304	18.1.0	0875	-	F	IoT_NTN_enh-Core	Withdrawn

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2404127	LS on UE capability for NW verified location (R4-2406496; contact: Huawei)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN2

R2-2404248	Correction on NR NTN	CATT (Rapporteur)	CR	Rel-18	37.355	18.1.0	0504	-	F	NR_NTN_enh-Core

R2-2404146	LS reply on OAM requirements for UE location verification (S5-242198; contact: CATT)	SA5	LS in	Rel-18	OAM_NTN	To:RAN3	Cc:SA2, RAN1, RAN2

Noted?

Rapporteur CRs

Moved here from 7.7.4

R2-2404839	Proposed conclusions for NR NTN RILs Rel-18	Ericsson	report	Rel-18	NR_NTN_enh-Core

R2-2404855	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.1.0	4761	2	F	NR_NTN_enh-Core	R2-2403772	Revised

This IPA CR is revised in R2-2404863 to include further changes

R2-2404863	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.1.0	4761	3	F	NR_NTN_enh-Core	R2-2404855

R2-2405374	Corrections for Non-terrestrial Networks	InterDigital	CR	Rel-18	38.321	18.1.0	1858	-	F	NR_NTN_enh-Core

R2-2404856	Terminology alignment in 38.304 for NR-NTN	ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0403	-	F	NR_NTN_enh-Core	Revised

Revised in R2-2405623

R2-2405623	Terminology alignment in 38.304 for NR-NTN	ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0403	1	F	NR_NTN_enh-Core	R2-2404856

R2-2404526	Add FR2 band reference to NTN related UE Capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_NTN_Ph3-Core

R2-2405455	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT	CR	Rel-17	38.331	17.8.0	4836	-	F	NR_NTN_solutions-Core

Revised in R2-2405714

R2-2405714	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.8.0	4836	1	F	NR_NTN_solutions-Core

To be handled in AI 6.1.3.1

R2-2405456	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT	CR	Rel-18	38.331	18.1.0	4837	-	F	NR_NTN_enh-Core

Revised in R2-2405715

R2-2405715	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4837	1	F	NR_NTN_enh-Core

Revised in R2-2405751 to make it a Cat A mirror CR for R2-2405714 (R18-specific changes to be moved to the R18 RRC Rapporteur CR)

R2-2405751	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4837	2	A	NR_NTN_enh-Core

To be handled in AI 6.1.3.1

### 7.7.2	Stage 2 corrections

R2-2405243	Stage-2 corrections on NR NTN	THALES	CR	Rel-18	38.300	18.1.0	0858	1	F	NR_NTN_enh-Core	R2-2403773

R2-2404890	Correction on Location-based CHO	OPPO	CR	Rel-18	38.300	18.1.0	0863	-	F	NR_NTN_enh-Core

### 7.7.3	RRC corrections

Rapporteur’s suggestions

R2-2404844	Comments on remaining NR NTN RILs	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Accept E302 to align NR NTN and IoT NTN specification.

Proposal 2	Reject [H115]. UE re-acquires system information upon satellite switch with resynchronization.

Proposal 3	If P2 is not agreed, it is up to network implementation to secure that value tags are set consistently across cells upon satellite switch with resynchronization.

Proposal 4	Reject H010

Satellite switch with resync

- [H115]

R2-2405501	[H115] SIB19 acquisition after satellite switching	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Clarify in clause 5.2.2.2.1 that only SIB19 is re-acquired after satellite switching with re-sync. Add the UE behaviour of obtaining SIB19 in clause 5.2.2.4.21.

Proposal 2: Adopt the TP for [H115] in the Annex.

R2-2404685	SIB acquisition after Satellite switch with re-sync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Unless the validity duration expires or UE receives paging notification of SI change, there is no need for UE to re-acquire the MIB, SIB1, SIB19 and other SIBs after satellite switch with resync.

R2-2405169	Discussion on E302, H115	Samsung	discussion	Rel-18	NR_NTN_enh-Core

- [H010]

R2-2405672	[H010][H115] and skipping MIB acquisition	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: If time offset is not clear enough, consider adding “with the same index”, or using “SSB half-frame” or “SSB burst”

Observation 1: During satellite switch with resync, cell is unchanged hence SI acquisition should not be required

Observation 2: If broadcasted SMTCs needs to be updated at satellite switch, it means configured SMTCs shall also be updated, killing the benefit of the feature

Proposal 2: RAN2 to discuss if any fields require updating, and why (particularly SMTCs)

Observation 3: Omitting PBCH acquisition can reduce interruption time

Observation 4: It is beneficial for the UE to know if cell/SSB timing is unchanged (at gNB)

Proposal 3: Capture that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)

R2-2405680	Soft satellite switch SSB time offset and SMTC impact	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

- Other

R2-2404158	Further Discussion on Satellite Switch with Resync	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Change the granularity of capabilities softSatelliteSwitchResyncNTN-r18 and hardSatelliteSwitchResyncNTN-r18 as per band level.

Proposal 2: RAN2 confirms that parallelMeasurementWithoutRestriction-r17 and/or simultaneousRxDataSSB-DiffNumerology is not the pre-requisite capability of softSatelliteSwitchResyncNTN-r18.

R2-2405143	On Reducing the NW Uncertainty in Satellite Switching with Resynchronization	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2405273	Discussion on beam management for satellite switch with re-sync	ETRI	discussion	Rel-18	NR_NTN_enh-Core

R2-2405423	Discussion on satellite switch with resync in RRC states other than RRC connected	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

(Cond)Event D2

R2-2405079	Clarification on (cond)event D2 configuration	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: It is proposed to reject second changes ( i.e., optionally present part) proposed in RIL 302.

R2-2404208	Corrections on Event D2 measurement reporting	CATT	discussion

Observation 1: Current Spec does not specific how to derive the applicable cells for Event D2. This leads to the issue that when UE initiates measurement reporting procedure for Event D2, it can report nothing (including RRM measurement and PCI) about the neighbour cell that fulfils the reporting conditions.

Observation 2: According to the current ASN.1 structure of MeasObjectNR, one measID can be associated with multiple neighbour cells each of which is further associated with one reference location. When receiving measurement reporting from UE, the NW cannot know which cell fulfils the reporting condition due to Observation 1.

Observation 3: For Event D2, UE fulfils either the entering condition or leaving condition will initiate measurement reporting procedure. According to the current measurement reporting mechanism, the NW could not know whether the UE meets entering condition or leaving condition due to Observation 1.

Proposal 1: The measurement results reporting for event D2 should include the physCellId of the neighbour cells that fulfils the reporting conditions.

Proposal 2: One cellsTriggeredList should be kept for the measID which is associated with event D2:

-	When the entering condition is fulfilled for one neighbour cell of the event D2, the UE includes this cell to the cellsTriggeredList for this measID and initiates measurement reporting procedure.

-	When the leaving condition is fulfilled for one cell included in the cellsTriggeredList which is associated with event D2, the UE removes this cell from the cellsTriggeredList for this measID and initiates measurement report procedure.

Proposal 3: If Proposal 1 and Proposal 2 are agreed, adopt the TP in Annex.

Proposal 4: if Proposal 1 and Proposal 2 are agreed, RAN2 confirms the similar issues exists also in TS 36.331 and agrees to apply a similar way of change as in P1/2 in TS 36.331 as well.

FR2 capability

R2-2405500	Correction on NR NTN FR2 capabilities	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4841	-	F	NR_NTN_enh-Core

Other

R2-2404673	Open issues on NR NTN enhancements	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2404883	Discussion on the leftover issues for NR-NTN	Google Inc.	discussion	Rel-18

### 7.7.4	Other Stage 3 corrections

Corrections related to other specs, e.g. 38.321, 38.304, 38.306.

R2-2405080	Consideration on remaining FR2 UE capability issues	ZTE Corporation, Sanechips, Eutelsat Group	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Update TS 38306 to allow indicate below UE capabilities for NTN FDD-FR2:

- SDT related: cg-SDT-r17, mt-CG-SDT-r18

- MBS related: maxDynamicSlotRepetitionForSPS-Multicast-r17, maxNumberG-CS-RNTI-r17, maxNumberG-RNTI-r17, multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17, priorityIndicatorInDCI-Multicast-r17, priorityIndicatorInDCI-SPS-Multicast-r17, releaseSPS-MulticastWithCS-RNTI-r17, sps-MulticastMultiConfig-r17, re-LevelRateMatchingForMulticast-r17, twoHARQ-ACK-CodebookForUnicastAndMulticast-r17

Proposal 2: If P1 is agreed, RAN2 consider the corresponding TP in annex for CR implementation.

R2-2405170	Discussion on TAT handling in RACH-less CHO	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: For the start of TAT in RACH-less CHO, UE follows the procedure for RACH-less HO (i.e., no specification change).

R2-2405118	Correction on HARQ buffer flush during satellite switch with re-synchronization	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1854	-	F	NR_NTN_enh-Core

R2-2405142	On Scheduling Restrictions in Satellite Soft Switching with Resynchronization – further view	Nokia	discussion	Rel-18	NR_NTN_enh-Core	R2-2403300

Proposal 1: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support parallelMeasurementWithoutRestriction.

Proposal 2: RAN2 is asked to reconsider the decision made at RAN2#125 regarding PDD during satellite switching.

Proposal 3: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support serviceLinkPropDelayDiffReporting-r17.

Proposal 4: If the decision regarding PDD is revised, confirm with RAN4 that service link propagation delay difference (PDD) helps in reducing the scheduling restriction duration during satellite soft-switching with resynchronization.

Proposal 5: If the LS to RAN4 is sent, RAN2 asks how early such measurements, calculations and reporting should be done.

Proposal 6: Adopt the 38.306 and 38.331 Text Proposals in the Annex A and B.

R2-2405424	Discussion on TA timer for satellite switch	ASUSTeK	discussion	Rel-18	38.321	NR_NTN_enh-Core

Proposal 1: RAN2 to discuss whether the TA timer is started or restarted upon satellite switch with resynchronization.

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240775

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240846)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404207	Updated work plan for NR NTN Ph3	CATT, Thales	Work Plan

R2-2404137	LS on Support of Regenerative-based Satellite Access (S2-2405600; contact: vivo)	SA2	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:RAN3	Cc:RAN2

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

R2-2404159	Discussion on Downlink Coverage Enhancements	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404204	Discussion on downlink coverage enhancements	CATT	discussion

R2-2404354	Discussions on beam and cell level DTX DRX	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404582	Discussion on DL coverage enhancement in NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404654	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404682	Discussion on cell DTX/DRX	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404739	Discussion on system level enhancement for downlink coverage enhancements for NTN	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404797	Downlink coverage enhancement in NTN	Lenovo	discussion	Rel-19

R2-2405081	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405124	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405173	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405229	On the applicability of downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

R2-2405240	Discussion on RAN2 Aspects for Downlink Coverage Enhancements in NR NTN evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405299	Questions identified to RAN1 on Downlink Coverage Enhancement	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405312	Downlink Coverage in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405320	Consideration on downlink coverage enhancement	NEC Corporation.	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405375	Downlink coverage enhancement for NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405376	[Draft] LS on DL coverage enhancements	InterDigital	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1

R2-2405449	Downlink coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405613	Downlink coverage enhancements for NTN	NERCDTV	discussion

R2-2405626	Discussion for DL coverage enhancement	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405636	Discussion on downlink coverage enhancements in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

Withdrawn

R2-2405600	Downlink coverage enhancements for NTN	NERCDTV	discussion	Withdrawn

### 8.8.3	Uplink Capacity/Throughput Enhancement

No contributions are expected for this AI at this meeting.

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2404160	Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404206	Discussion on support of broadcast service via NR NTN	CATT	discussion

R2-2404282	Discussion on support of a broadcast service in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404355	Discussions on signaling of the intended service area of a broadcast service	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404429	Discussion on support of broadband services	Continental Automotive	discussion

R2-2404580	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404621	Discussion on MBS Broadcasting Control over NTN access	TCL	discussion

R2-2404655	Broadcast service support over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404679	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404798	On broadcast service area indication in NTN	Lenovo	discussion	Rel-19

R2-2404841	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404853	Discussions on the configuration of intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2404854	Discussions on limiting broadcast service in the intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2404916	Broadcast service area signaling	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404982	Discussion on the support of broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405020	Support of MBS broadcast service for NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405082	Consideration on broadcast service enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405099	Discussion on the service area of a broadcast service	Xiaomi	discussion

R2-2405125	Discussion on supporting MBS broadcast over NTN	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405147	On the Signalling Aspects of MBS over Rel-19 NR NTN	Nokia	discussion	Rel-19	NR_NTN_Ph3	R2-2403306

R2-2405174	Discussion on Broadcast Service Area in NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405204	Discussions on MBS in Rel-19 NTN	TOYOTA Info Technology Center	discussion	Rel-19

R2-2405211	Discussion on support of broadcast service in NTN	LG Electronics France	discussion	Rel-19	NR_NTN_Ph3	R2-2403121

R2-2405239	Discussion on MBS broadcast additional features for NR NTN Evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405277	Clarification on intended service area	NEC Telecom MODUS Ltd.	discussion

R2-2405377	Support for broadcast service in NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405525	Discussion on support of broadcast service	ITL	discussion	Rel-19

R2-2405627	Discussion on MBS service support for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.5	Support of regenerative payload

Contributions should focus on the needed updates for Stage 2 description and on whether any existing essential features would be affected - and potentially need any modifications - in a regenerative payload architecture.

R2-2404161	Discussion on Mobility with Regenerative Payload	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404205	Further discussion on regenerative payload	CATT	discussion

R2-2404256	Further discussion on regenerative payload for satellite switch with resync	NTU	discussion	Rel-19	Late

R2-2404590	Discussion on satellite switch with resynch for regenerative payload	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404680	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404799	Considerations on regenerative payload in NTN	Lenovo	discussion	Rel-19

R2-2404917	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405021	Support of regenerative payload	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405083	Consideration on support of regenerative payload	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405100	Discussion on the support of regenerative payload	Xiaomi	discussion

R2-2405126	Discussion on support of regenerative payload	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405148	On the feasibility of the existing NTN features over regenerative architecture	Nokia	discussion	Rel-19	NR_NTN_Ph3

R2-2405156	Regenerative payload for NR NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405196	Support  Regenerative Payload	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405241	Regenerative NTN payload support in NR NTN Evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405313	Stage-2 updates for regenerative payload	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405448	Stage 2 updates for regenerative payload	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405629	Discussion for regenerative payload	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Withdrawn

R2-2404622	Discussion on Support of NTN Regenerative Architecture	TCL	discussion	Withdrawn

### 8.8.6	LTE to NR NTN mobility

Support for idle mode mobility between LTE and NR NTN.

R2-2404162	Discussion on LTE TN to NR NTN Mobility	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404198	Support for LTE to NR-NTN idle mode mobility	Telit Communications S.p.A. ; Thales	discussion	R2-2403066

R2-2404211	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT	discussion

R2-2404591	Discussion on LTE to NR NTN idle mode mobility	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404656	Mobility from LTE TN to NR NTN	Apple	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2404681	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404759	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.	discussion	NR_NTN_Ph3-Core	R2-2403226

R2-2404800	On LTE to NR-NTN IDLE mobility	Lenovo	discussion	Rel-19

R2-2404840	E-UTRAN TN to NR-NTN mobility	Ericsson	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2404986	Further discussion on idle mode cell reselection form LTE to NR NTN	Transsion Holdings	discussion	Rel-19

R2-2405022	Considerations on cell reselection enhancements from E-UTRAN TN to NR-NTN	CMCC	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2405084	Consideration on idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2405101	Discussion on the cell reselection from LTE to NR NTN	Xiaomi	discussion

R2-2405108	Discussion on LTE to NR NTN mobility	Interdigital, Inc.	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2405127	Discussion on LTE to NR NTN mobility	Huawei, HiSilicon, Turkcell	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2405146	On How to Address E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia	discussion	Rel-19	NR_NTN_Ph3

R2-2405155	E-UTRAN TN to NR NTN mobility basic scenario and signalling	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2405210	Discussion on support of LTE to NR NTN cell reselection	LG Electronics France	discussion	Rel-19	LTE_TN_NR_NTN_mob	R2-2403123

R2-2405314	Consideration of LTE TN to NR NTN mobility	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405630	Discussion for LTE to NR NTN mobility	Sharp	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240776)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404144	LS to SA2 and RAN2 on selected satellite architecture for Store and Forward (S3-241567; contact: InterDigital)	SA3	LS in	Rel-19	FS_5GSAT_Ph3_SEC	To:SA2, RAN2	Cc:SA, RAN3, SA3-LI

R2-2405378	[Draft] Reply LS on selected satellite architecture for Store and Forward	InterDigital	LS out	Rel-19	IoT_NTN_Ph3-Core	To:SA3	Cc:SA2

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2404163	RAN2 Aspects for Store & Forward	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404199	Considerations on S&F operation from device perspective	Telit Communications S.p.A.	discussion	Revised

R2-2404202	Discussion on support of store and forward operation	CATT	discussion

R2-2404321	Overview of the Store and Forward satellite operation	Huawei, HiSilicon, Turkcell 	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404436	Elements of Store & Forward Operation	PANASONIC	discussion

R2-2404589	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404657	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404683	S&F satellite operation with full eNB as regenerative payload	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404801	Store and Forward support in IoT NTN	Lenovo	discussion	Rel-19

R2-2404882	RAN2 impacts of supporting Store&Forward operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2402380

R2-2404885	Discussion on the store and forward operation	Google Inc.	discussion	Rel-19

R2-2404979	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot	discussion	R2-2404199

R2-2404987	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2405012	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405102	Discussion on the support of store and forward satellite operation	Xiaomi	discussion

R2-2405132	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2402942

R2-2405153	On RAN2 aspects of Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405193	Radio Interface Aspects for Store And Forward mode operation of IoT-NTN	Nokia, Nokia Shanghai Bell	discussion

R2-2405197	Support  of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405242	Considerations on Store & Forward Satellite Operation	SHARP Corporation	discussion	Rel-19

R2-2405315	The impact of access for Store & Forward in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405429	Discussion on information for Store & Forward	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405450	Support for store and forward	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405657	Discussion of the Store and Forward satellite operation	TCL	discussion

Withdrawn

R2-2404409	Overview of the Store and Forward satellite operation	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

### 8.9.3	Uplink Capacity Enhancement

At this meeting contributions should only focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

R2-2404164	Discussion on EDT Enhancement for IoT-NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404203	On signalling overhead reduction for EDT in IoT NTN	CATT	discussion

R2-2404322	Overview of capacity enhancement for uplink	Huawei, HiSilicon, Turkcell 	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404561	Discussion on EDT optimisation in IoT-NTN	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404581	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404658	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404684	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404740	Discussion on uplink capacity enhancements for IOT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404802	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2404842	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404884	Consideration on enhanced early data transmission in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2402381

R2-2404922	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2405023	Considerations on EDT enhancements for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405109	Msg3 transmission without msg1/RAR	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405110	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405133	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2402943

R2-2405154	Scenarios and procedures for IoT NTN uplink capacity enhancements	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405201	Efficient delivery of RRCEarlyDataComplete message	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405202	Discussion on Msg3-EDT enhancements	ESA	discussion	Rel-19

R2-2405321	Consideration on EDT enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405442	Discussion on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405655	Consideration on enhanced early data transmission in IoT NTN	TCL	discussion

Withdrawn

R2-2404410	Overview of capacity enhancement for uplink	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

# Summary

Agreed CRs

NR-NTN

IoT-NTN

Approved LSs out

[Post126] Email discussions

Short

Long