3GPP TSG-RAN2 Meeting#129	R2-2500115

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES.

- Discussion

2.1 Issue-1: OD-SSB Activation/Deactivation

Based on the latest R1 progress

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

For the FFS point on R2, considering the following R2#127bis conclusion

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

It is already confirmed by R2, no further conclusion is needed from R2 side.

R2 has already confirmed the feasibility of RRC-based deactivation.

Furthermore, R1 confirmed the two options above, i.e., MAC-CE may or may not be used for OD-SSB deactivation.

For the MAC-CE format design, R2 agreed that

=> Will discuss it later.

=> NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

There are in general two methods: one is to include cell-index of OD-SSB explicitly while the other is to include cell-index implicitly via an ordered bitmap. In general, the latter one is more signaling efficient, considering the saved overhead for cell-index indication due to the cell-index indication (especially in case of simultaneous multi-SCell (de)activation. For the meaning of the bit

- In case of option-1 (Explicit deactivation via MAC-CE): 1/0 is for activation/deactivation, and presence/absence of (at least) periodicity parameter.

- In case of option-2 (Implicit deactivation): 1/0 is for presence/absence of (at least) periodicity parameter.

- Adopt the bitmap-based method to indicate OD-SSB indication for multiple SCells simultaneously.

Besides, R1 had an FFS point regarding which parameter(s), besides periodicity, has to be carried by MAC-CE

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

So that R2 cannot conclude on the detailed design of the MAC-CE format.

- R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

  - Issue-2: L3 Measurement of OD-SSB

Comparing Case#1 and Case#2,

- For OD-SSB, hopefully the conclusion of Case#1 can be reused as much as possible to OD-SSB of Case#2, i.e., for the issues of OD-SSB itself, R2 can prioritized the discussion of Case#1 to simplify the discussion.

- On the other hand, R2 can focus on the relationship between OD-SSB and AO-SSB in Case#2, to clarify the Case#2 specific issues, as complementary part of conclusion of OD-SSB.

    - Case#1-Specific Issues

For serving cell measurement, 127bis discussed that

3. L3 RRM Framework

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

Need more time to think of.

Based on R4#112bis conclusion:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

Based on R4#113 conclusion for case-1

Agreement:

OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement

For the case of CSSF = 1:

- OD-SSB periodicity regardless of measCycleSCell and DRX cycle.

  - FFS whether a lower bound will be specified.

FFS for the other cases of CSSF.

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

i.e., R4 agreed on a hybrid approach, i.e., UE will perform a hybrid fast + slow mode measurement

1) In the fast mode window, the measurement requirement is defined based on SSB periodicity;

2) After the fast mode window, if the OD-SSB is still activated, the measurement requirement is defined as in legacy, i.e., based on measCycleSCell and DRX cycle.

R4 agreed on a hybrid L3 measurement framework for OD-SSB Case#1 for deactivated SCell, where the fast mode measurement relies on SSB periodicity ignoring measCycleSCell and DRX cycle, and the slow mode measurement relies on measCycleSCell and DRX cycle ignoring SSB periodicity.

If the measurement requirement is based on SSB periodicity, there is no additional benefit by introducing an adaptive SMTC, which exactly copied the SSB pattern, without additional functionality introduced.

For fast mode measurement, adaptive SMTC does not bring additional functionality, on top of SSB pattern based L3 measurement, considering both needs to follow SSB periodicity based requirement.

In other words, “fast mode window” already implies that the UE shall be able to perform L3 measurement on SCell based on SSB pattern directly, ignoring SMTC. After the “fast mode window”, the measurement is not related to the SSB periodicity, but relies on the measCycleSCell and DRX cycle, i.e., the corresponding L3 measurement does not change with SSB periodicity

- Firstly, it is feasible for UE to perform L3 measurement based on SSB pattern (but following measCycleSCell and DRX cycle), since anyway SMTC window is more meaningful for neighboring cell for which no prior information on the SSB configuration and synchronization at UE side.

- Secondly, another approach is relying on legacy SMTC, but since dynamicity of OD-SSB goes against the static of SMTC, it is not feasible to do that, unless restrict network implementation to ensure OD-SSB can at least partially overlapping with statically configured SMTC.

For slow mode measurement, legacy SMTC, which was essentially introduced for neighboring cell measurement, goes against dynamicity of OD-SSB since it is statically configured.

Furthermore, this issue can be further extended from L3 measurement on deactivated SCell, to L3 measurement of SCell during activation / activated already.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity

Agreement:

- In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity.

  - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity.

- For OD-SSB Case#1, for L3 measurement of SCell, R2 confirm it is performed based on OD-SSB pattern but ignoring SMTC, to align with R4 conclusion.

    - Case#2-Specific Issues

For case-2, R1 is still working on the feasibility of different frequency scenario

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

R1 already confirmed the feasibility of same-frequency scenario for AO/OD-SSB in Case-2 (when AO-SSB is NCD-SSB), and FFS when AO-SSB is CD-SSB.

Considering that, R2 can proceed for the same-frequency scenario in the coming meeting, e.g., regarding whether to allow combined measurement on always-on SSB and OD-SSB.

Figure 1 An example of Case-2 (AO-SSB and OD-SSB are partially overlapping)

Based on R1 conclusion, Figure-1 gives an example of partially overlapping AO/OD-SSB, e.g., AO-SSB/OD-SSB is of 40ms/20ms periodicity, with same offset, so it leads to 3 SSB bursts which are overlapping between each other. In this case, it is hard to differentiate between AO-SSB and OD-SSB, at the overlapping places.

Furthermore, R2 defined a L3 measurement framework in the following manner, I.e., the configuration of AO-SSB and OD-SSB have to be carried via a same MO.

The network applies the procedure as follows:

[…]

- to ensure that, in the measConfig associated with a CG:

- for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

Combining the two, i.e., same MO and hard to differentiate the SSB attributive (AO-SSB or OD-SSB), it seems easier to perform a combined L3 measurement, and generate a single MR.

- For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 support the L3 measurement configuration based on a single MO and report via a same MR, based on combined L3 measurement based on AO-SSB and OD-SSB.

    - Common Issues for Case#1/Case#2

For L3 measurement, since it is possible that serving cell and neighbouring cell share the same MO configuration, e.g., in an intra-frequency scenario, it is good to understand the behavior for neighboring and serving cell scenario.

For neighbouring cell, it anyway needs to rely on SMTC window to identify the time location of SSB searching. Besides, when it comes to OD-SSB, it is anyway infeasible for UE to know the neighboring cell OD-SSB (de)activation status. Combing the two, it seems more realistic to rely on Case-2, if network would like to configure the UE to perform neighboring cell measurement. Anyway, there seems no much work that R2 can do for the neighbouring cell measurement under OD-SSB objective, and it aligns with R4 conclusion below

Issue 1-1-2: OD-SSB impact on neighbour cell measurement

Agreement:

- For the RRM measurement on neigboring cell, the legacy meaurement framework is applied.

  - Note: The discussion of SMTC configuration is a separate issue.

- This agreement can be revisited if it is not aligned with further agreement in other WGs.

- For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Another issue is whether to rely on legacy event-based MR triggering or rely on MAC-CE based MR triggering.

We understand that MAC-CE based MR triggering was introduced for FR2 unknown SCell, while OD-SSB can be sent before SCell activation, and thus enable known case, which would lead to an even quicker SCell activation compared with the “semi-known” case of MAC-CE triggered MR. Considering that, the motivation of introducing MAC-CE based MR triggering is not strong.

Issue 1-4-1: The known cell condition – measurement reporting

Agreement:

- In case 1, known/unknown condition should be defined based on only OD-SSB.

- In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB.

  - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue.

  - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB.

  - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

- R2 not pursue MAC-CE triggered MR for OD-SSB.

- Conclusion

Proposal 1	Adopt the bitmap-based method to indicate OD-SSB indication for multiple SCells simultaneously.

Proposal 2	R2 wait for R1 conclusion on the content of MAC-CE before concluding on the MAC-CE format design.

Proposal 3	For OD-SSB Case#1, for L3 measurement of SCell, R2 confirm it is performed based on OD-SSB pattern but ignoring SMTC, to align with R4 conclusion.

Proposal 4	For OD-SSB Case#2, for same-frequency scenario between AO-SSB and OD-SSB, R2 support the L3 measurement configuration based on a single MO and report via a same MR, based on combined L3 measurement based on AO-SSB and OD-SSB.

Proposal 5	For L3 measurement of neighbouring cell based on OD-SSB, R2 not pursue further specification work.

Proposal 6	R2 not pursue MAC-CE triggered MR for OD-SSB.