3GPP TSG-RAN WG2 Meeting #129	R2-2500123

# Athens, Greece, 17th – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of AI/ML for NR Air interface feature in TS38.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of AI/ML for NR Air interface feature in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture RAN2 agreements up to RAN2#128. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specification TS38.300 does not include the AI/ML for NR Air interface feature in Rel-19. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: AI/ML for NR Air interface Inter-operability: No inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## X.Y	Support of AI/ML for NR Air Interface

### X.Y.1	Overview

The support of AI/ML for NR air interface is to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NR air interface.

The objective of AI/ML for NR air interface is to improve network performance and user experience, through analysing the data collected and enhancement AI/ML-enabled Features, e.g., beam management enhancement, positioning accuracy enhancement.

### X.Y.2	AI/ML enhanced Beam Management

#### X.Y.2.1	Introduction

AI/ML enhanced beam management utilizes intra-cell beam prediction of the serving cell to reduce overhead and latency of beam measurement and to improve the accuracy of beam selection. Two types of beam prediction are supported as follows:

-	Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams, where Set B can be different from Set A, or a subset of Set A.

-	Temporal-domain Downlink beam prediction for Set A of beams based on historic measurement results of Set B of beams. Set B can be the same as Set A, or different from Set A, or a subset of Set A.

For the deployment of AI/ML enhanced beam management, both gNB-side model and UE-side model are supported.

#### X.Y.2.2	Data Collection for Model Training

Data collection for model training is performed under network control, i.e., the network determines when to start or stop the data collection by provision of data collection configuration.

For gNB-side model, model training can be performed at the OAM or the gNB. The enhanced immediate MDT, as specified in TS 37.320 [xx2], is used to support data collection for model training at the OAM. The following enhancements are introduced for training data collection:

-	UE can be configured by gNB to include multiple instances of logged L1 measurement results (e.g., L1-RSRPs and/or beam-IDs) in a RRC message.

-	Both periodic and radio condition-based event-triggered data logging are supported. The UE can store the logged data at the AS layer with a minimum AS memory size supported by the UE. When the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection purposes, and UE can indicate the memory status to the network.

Editor's note: FFS if other event(s) for data logging are supported.

Editor's note: FFS when UE reports the indication, e.g., before and/or after buffer become full.

-	To reduce UE power consumption, when low power state is detected, the UE will not automatically stop data logging, but the UE can indicate the low power state to the network. Upon reception of the low power status indication, the network should reconfigure UE to release the data collection configuration.

NOTE 1:	How the UE determines low power state is up to UE implementation.

-	The UE can indicate the availability of logged data to the network. Then, the network can trigger the UE information procedure, as specified in TS38.331[12], to request the logged data and UE can report the logged data via SRBx.

Editor's note: FFS introduce a new SRB or reuse SRB4.

For UE-side model, the network can configure whether UE is allowed to initiate a request for training data collection. Data collection-related configuration (e.g., measurement resources configuration) and associated ID can be included in the training data collection configuration.

Editor's note: FFS whether an indication from UE to network is needed when UE cannot perform data collection based on received configuration.

#### X.Y.2.3	Applicability Reporting

Editor's note: The content in this sub-clause will be further refined/update based on further progress.

UE can indicate its supported functionalities via UE capability information. For UE-side model, UE can decide the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in the UE. To assist the network to manage the UE-side model, the UE can report its applicable functionalities to the network proactively and/or reactively:

-	Proactive reporting: The network can configure the UE to report its applicable functionalities via the UE Assistance Information procedure. When a functionality status changes from non-applicable to applicable, or vice versa, the UE should report this to the network. When an activated functionality becomes non-applicable, the UE does not autonomously deactivate it. Upon reception of UE indication of the functionality becoming non-applicable, the network should deactivate or release this activated functionality.

Editor's note: FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.

-	Reactive reporting: The network can provide inference configuration to UE via CSI report configuration, and the UE reports the applicable functionalities in the RRCReconfigurationComplete message.

During handover, the target gNB can provide UE with NW-side additional conditions and/or the inference configuration related to the target gNB UE via handover command. Then the UE can report its applicable functionalities to the network after the handover completion via either RCReconfigurationComplete or UAI.

Editor's note: FFS how the applicable functionality is decided if NW-side additional condition is not provided

NOTE 1:	UAI can be sent from the source gNB to the target gNB using existing signaling. Xx.2.3	Inference

For gNB-side model, the CSI report framework is used to acquire input data for inference.

NOTE 2:	For gNB-side model, how gNB determines the applicability is up to implementation.

#### X.Y.2.4	Monitoring and Management

For gNB-side model, the gNB is responsible for performance monitoring. UE will not be involved in management decisions (e.g., selection, (de)activation, switching, fallback, etc.) of gNB-side model, except for being configured to provide the required measurement/data. Additionally, the UE will not be informed about any gNB-side management decision.

### X.Y.3	AI/ML-based Positioning accuracy enhancement

AI/ML-based Positioning accuracy enhancement is described in TS 38.305 [42].

END OF CHANGES

# Annex of meeting agreements:

RAN2 agreements related to the WI phase are listed in the following, and the agreements related to the study part, e.g., UE side data collection, are not listed since they are not to be captured in this specification.

## RAN2 128

### General

### Beam management

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

### Positioning

Agreements 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

## RAN2 127bis

### General

### Beam management

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/ UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

### Positioning

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

## RAN2 127

### General

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

### Beam management

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

### Positioning

## RAN2 126

### General

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

### Beam management

Agreements for NW-sided model for Beam Management

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Agreements

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

### Positioning

Agreements for NWside model

1	RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

## RAN2 125bis

### General

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

### Beam management

Agreements:

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

- There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- FFS whether rhere is specification impact associated to gNB-side model monitoring.

Agreements

1	For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches

2	We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

3	RAN2 supports enhancements to MDT for data collection framework for training.  FSS Whether to enhance logged or immediate MDT

### Positioning

Agreements:

- For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs

- For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3.