3GPP TSG-RAN WG2 Meeting #129		R2-2500124

Athens, Greece, Feb. 17th – 21st, 2025

Source:	vivo

Title:          Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN2#128 meeting, RAN2 achieved the following agreements regarding LCM for UE-sided model for beam management use case:

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

6.	For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In this contribution, we will further discuss the LCM for UE-sided model for beam management use case based on RAN2 agreements and RAN1’s reply LS[1].

# Discussion

## 2.1 Whether to use “functionality” concept in RAN2

In the RAN1’s reply LS [1]:

In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. … Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

RAN1 thinks that the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, and they refer to different things. Moreover, if going through RAN1’s chairman note in the RAN1#119 meeting, it seems that RAN1 is avoiding to use the “functionality” concept. One issue is whether to remain using the “functionality” concept in RAN2 from the RAN2’s perspective. We think that it may be too early to make the conclusion. We suggest RAN2 to keep the term on the table now and whether it needs to be explicitly used in the specification can be discussed during the stage 3 phase.

- From the RAN2’s perspective, keep the “functionality” concept on the table now and whether it needs to be explicitly used in the specification can be discussed during the stage 3 phase.

## 2.2 Signalling for inference configuration and applicability reporting

Based on current RAN1 and RAN2 agreements, we think that it can be categorized into two aspects from the signalling view. One is inference configuration and the other is applicability reporting. We will discuss the signaling for the two aspects separately in the following two subsections.

### 2.2.1 Inference configuration

There are the following RAN1’s agreements in [2]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN1 agreed two options: Option A) and Option B). The inference configuration is configured in Step 3 for Option A) and in Step 5 for Option B). For Option A), the UE needs to report applicability for the inference configuration. For Option B), the UE does not need to report applicability for the inference configuration in Step 5. One issue is how the UE knows whether to report applicability upon receiving inference configuration since inference configuration is configured via the RRCReconfiguration message in both two options. To address the issue, we think that there needs to be an indicator associated with configured inference configuration to indicate whether to feedback the applicability report for the inference configuration.

- An indicator associated with configured inference configuration is introduced to indicate whether to feedback the applicability report for the inference configuration.

RAN1 has agreed that inference configuration is one or more of CSI-ReportConfig, i.e., reusing the legacy CSI report framework. Since inference configuration is a type of radio configuration, we think that inference configuration should be configured outside of otherConfig.

- Confirm that inference configuration is configured outside of otherConfig.

Inference configuration configured by the network may be non-applicable, especially for Option A). What is UE behavior upon receiving a non-applicable inference configuration? Two potential approaches are as below:

- Alt 1: the UE autonomously releases the non-applicable inference configuration;

- Alt 2: the UE maintains the configuration until the network releases it;

We think that Alt 2 is desirable since it is more in line with RAN2's usual practice.

- If the inference configuration configured by the network is non-applicable, the UE maintains the configuration until the network releases it.

### 2.2.2 Applicability reporting

At the RAN2#127bis meeting, RAN2 agreed “UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality”. However, it is unclear when to use UAI or when to use the RRCReconfigurationComplete message for applicability reporting. There are two scenarios for applicability reporting. One is reactive applicability reporting upon receiving inference configuration or upon receiving one set or multiple sets of inference related parameters for Option B), and the other is proactive applicability reporting (i.e., reporting for applicability status change). In the following subsections, we will discuss how to report the applicability separately in the two scenarios.

#### 2.2.2.1 Reactive applicability reporting

Since inference configuration is a radio configuration in the RRCReconfiguration message and the applicability reporting is a kind of immediate feedback for inference configuration, it is natural that the applicability reporting is included in the RRCReconfigurationComplete message.

- Upon receiving the inference configuration, the UE feedbacks the applicability reporting for the inference configuration via the RRCReconfigurationComplete message.

How does the UE feedback the applicability reporting via the RRCReconfigurationComplete message? In the previous discussion, there were the following two options:

- Option 1: The RRCReconfigurationComplete message directly includes the related IE for applicability reporting;

- Option 2: The applicability reporting in the RRCReconfigurationComplete message is a container that corresponds to a UAI message;

We have a little preference to Option 2 because Option 2 has less specification impact.

- Upon receiving the inference configuration, the applicability reporting in the RRCReconfigurationComplete message is a container that corresponds to a UAI message.

The otherConfig field is used to contain configuration related to other configurations. To our understanding, the “other configuration” means configuration which is not real typical radio configuration. It is the reason that it is called “other configuration”. For Option B), we think that “one set or multiple sets of inference related parameters” should be other configuration since it is only used to obtain the applicability reporting. Therefore, the one set or multiple sets of inference related parameters for Option B) should be configured in the otherConfig.

- The one set or multiple sets of inference related parameters for Option B) are configured in the otherConfig.

Upon receiving inference related parameters for Option B), the UE needs to feedback the applicability reporting. If Proposal 7 is agreed, similar to other “other configuration”, e.g., overheating assistance config, the UE should feedback the applicability reporting via a UAI message.

- Upon receiving one set or multiple sets of inference related parameters for Option B), the UE feedbacks the applicability reporting via a UAI message.

#### 2.2.2.2 Proactive applicability reporting (i.e., reporting for applicability status change)

In general, the separate UAI message (not container in the RRCReconfigurationComplete message) is configured to trigger via the otherConfig. We think that applicability reporting for applicability status change should also follow the rule.

- The UE is configured to allow triggering the UAI message for applicability status change via the otherConfig.

According to RAN1 agreements, the UE may be configured with multiple CSI-ReportConfig for Option A) or multiple sets of inference related parameters for Option B). One issue is how to configure the triggering of applicability reporting, e.g., whether it is necessary to configure the triggering per CSI-ReportConfig or per set of inference related parameters? We think that if the network wants to know the applicability status change, it should want to know it for all the configured CSI-ReportConfig, not for just the part of them. Therefore, it is sufficient to configure the triggering per UE.

- The allowance of applicability reporting for applicability status change is configured per UE, not per CSI-ReportConfig.

#### 2.2.2.3 Content of applicability reporting

According to RAN1 agreements, the applicability status is associated with a CSI-ReportConfig for Option A) or a set of inference related parameters for Option B). For reactive applicability reporting, the UE reports the applicability status for each CSI-ReportConfig or each set of inference related parameters which are configured in the RRCReconfiguration message. For proactive applicability reporting, the UE reports the new applicability status for each CSI-ReportConfig or each set of inference related parameters that are allowed to report and its applicability status changes. Therefore, we think that for a CSI-ReportConfig or a set of inference related parameters, at least the corresponding index and the current status (applicable or non-applicable) should be included in the applicability reporting.

- For a CSI-ReportConfig or a set of inference related parameters, the corresponding index and the current status (applicable or non-applicable) are included in the applicability reporting. FFS on other information.

## 2.3 Signalling of data collection configuration for UE-side model training

Regarding data collection for UE-side model training, RAN2 achieved the following agreements in the latest meeting:

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

RAN2 agreed that the network can configure whether UE is allowed to initiate request for data collection. We think that the configuration can be put in the system information. Otherwise, in case the network does not allow it, an idle UE which wants to collect data does not know the information and will still go into connected state to obtain the information, which increases the signalling overhead.

- The network can configure whether UE is allowed to initiate a request for data collection via system information.

For the UE request, one issue is whether it is a new RRC message or it reuses an existing message. We think that it is desirable to reuse the UAI message.

- The UE request reuses the UAI message. The detailed content of the request waits for RAN1’s progress.

For the training of the UE-sided model, the UE needs to obtain the training configuration (e.g., associated ID(s), CSI-RS configuration for training) of the serving cell. There may be two possible approaches to obtain the training configuration of the serving cell:

- Approach 1: via the system information of the serving cell

Figure 1: via system information

- Approach 2: via the RRC dedicated signaling, e.g., after requested by the UE

Figure 2: via RRC dedicated signaling

For positioning feature, the network can broadcast the positioning assistance information via the posSIBs, or the network can send the related information via dedicated signaling. Similar to positioning feature, we think that both of the above two approaches should be supported and it is left to network implementation to support which one or both.

- For the signaling to configure the UE for training, support both two approaches:

1)NW broadcasts the UE collection configuration;

2) NW sends the collection configuration to the UE via dedicated signaling.

It is left to network implementation to support which one or both.

In the latest meeting, there was an FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration. We think that there should be data collection capability in the UE radio access capability.

- Introduce data collection capability in the UE radio access capability.

For dedicated data collection configuration, the network can determine the configuration based on UE capability. For broadcast data collection configuration, the UE can initiate the request to request dedicated configuration if the UE can’t perform data collection based on broadcast configuration. Therefore, we think that the indication is not needed.

- An indication from UE to network which indicates UE cannot perform data collection based on the received configuration is not needed:

- For dedicated data collection configuration, the network can determine the configuration based on UE capability.

- For broadcast data collection configuration, the UE can initiate the request to request dedicated configuration if the UE can’t perform data collection based on broadcast configuration.

# Conclusion

In the contribution, we have the following observations and proposals:

Whether to use “functionality” concept in RAN2

- From the RAN2’s perspective, keep the “functionality” concept on the table now and whether it needs to be explicitly used in the specification can be discussed during the stage 3 phase.

Inference configuration

- An indicator associated with configured inference configuration is introduced to indicate whether to feedback the applicability report for the inference configuration.

- Confirm that inference configuration is configured outside of otherConfig.

- If the inference configuration configured by the network is non-applicable, the UE maintains the configuration until the network releases it.

Reactive applicability reporting

- Upon receiving the inference configuration, the UE feedbacks the applicability reporting for the inference configuration via the RRCReconfigurationComplete message.

- Upon receiving the inference configuration, the applicability reporting in the RRCReconfigurationComplete message is a container that corresponds to a UAI message.

- The one set or multiple sets of inference related parameters for Option B) are configured in the otherConfig.

- Upon receiving one set or multiple sets of inference related parameters for Option B), the UE feedbacks the applicability reporting via a UAI message.

Proactive applicability reporting (i.e., reporting for applicability status change)

- The UE is configured to allow triggering the UAI message for applicability status change via the otherConfig.

- The allowance of applicability reporting for applicability status change is configured per UE, not per CSI-ReportConfig.

Content of applicability reporting

- For a CSI-ReportConfig or a set of inference related parameters, the corresponding index and the current status (applicable or non-applicable) are included in the applicability reporting. FFS on other information.

Signalling of data collection configuration for UE-side model training

- The network can configure whether UE is allowed to initiate a request for data collection via system information.

- The UE request reuses the UAI message. The detailed content of the request waits for RAN1’s progress.

- For the signaling to configure the UE for training, support both two approaches:

1)NW broadcasts the UE collection configuration;

2) NW sends the collection configuration to the UE via dedicated signaling.

It is left to network implementation to support which one or both.

- Introduce data collection capability in the UE radio access capability.

- An indication from UE to network which indicates UE cannot perform data collection based on the received configuration is not needed:

- For dedicated data collection configuration, the network can determine the configuration based on UE capability.

- For broadcast data collection configuration, the UE can initiate the request to request dedicated configuration if the UE can’t perform data collection based on broadcast configuration.

# Reference

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1

- Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0