3GPP TSG-RAN WG2 Meeting #129	R2-2500131

Athens, Greece, Feb 17th – 21st, 2025

Source:	vivo

Title:	Discussion on AIoT data transmission

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

According to the WID for Ambient IoT (AIoT) [1] approved in the RAN#106 meeting, the RAN2 scope is as the following:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

Besides, RAN2 has made some conclusions and recommendations for AIoT during study phase, see the descriptions under clause 6.3 and 6.10 [2]. With considering the baseline conclusions captured in TR 38.769, in this contribution, we will further discuss the issues regarding AIoT data transmission and other general aspects, including:

- AS ID

- MAC PDU format

- Segmentation

- D2R message size report

# Discussion

# AS ID

According to TR 38.769, the baseline conclusions on AS ID are captured as below:

### 6.3.5 A-IoT data transmission From higher layer perspective, it is assumed that "AS ID" (if defined according to the design in clause 6.1) is used at least for purpose of D2R scheduling and R2D reception. From higher layer perspective, it is assumed that this "AS ID" should be a short AS layer ID, rather than the full upper layer device ID. It needs to be further discussed if this "AS ID" can be based on partial upper layer device ID. It needs to be further discussed on the length of this "AS ID". From higher layer perspective, following options are possible for this "AS ID" (it is aimed to define one common design for all access procedures in clause 6.3.4, if technically possible): - Option 1: a random ID (if used in first D2R message) can be reused; - Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message. - Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message. NOTE 1: The further down-selection for "AS ID" needs to consider the involved message and whether the reader needs to handle the collision.

Based on the above text, the followings are identified as the remaining issues of AS ID:

- Further Down-selection for AS ID assignment;

- Whether we need the reader to handle the ID collision;

- FFS the length of AS ID;

- FFS via which R2D message if the reader assigns a new AS ID;

- FFS if AS ID can be based on partial upper layer device ID;

- Whether and how to maintain the AS ID.

Moreover, our analysis and views are provided for each issue as follows.

Issue 1: Further Down-selection for AS ID assignment

The following options are candidates for AS ID assignment, while it is aimed to define one common design for all access procedures if technically possible.

- Option 1: a random ID (if used in first D2R message) can be reused;

- Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

- Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

From a technical point of view, it is suggested that the potential pros and cons should be considered before making the final decision. For Option 1, it mandates a device to send RN16 in all cases, which has the obvious drawback of bringing radio resource waste and extra signaling overhead in case of a contention-free access procedure. Basically, an RN16 is introduced for contention resolution purposes. Unlike the CBRA procedure, the access occasion of AIoT Msg 1 is device-specific i.e., there is no possibility of collision with other devices, thus there is no usage of sending RN16 in the contention-free access procedure. Therefore, it is suggested that at least Option 1 be excluded. For Option 2, the drawback for Option 1 can be avoided. However, for CBRA procedure, we believe that RN16 in AIoT Msg1, which is reused as AS ID is a more robust way for AS ID assignment due to two reasons behind it. Firstly, there may be an ambiguity time window for reader to know whether AS ID assignment via AIoT Msg2 is successful or not. Let’s take 3-step CBRA as an example (see below Figure 1). If AIoT Msg3 which is scheduled by this AS ID is successfully received by the reader, it is clear that the AS ID assignment is successful. While if AIoT Msg3 scheduled by this AS ID is not received by the reader, the reader has no idea whether this AS ID assignment is successful or not because the reader cannot differentiate the two cases between the AIoT Msg2 reception failure and AIoT Msg 3 transmission failure at the device side. Consequently, this AS ID may not be valid/available to be used for subsequent AIoT Msg 4 in order to support failure/success feedback indication for AIoT Msg 3 for re-access purpose. In other words, directly reusing the RN16 received in AIoT Msg1 as AS ID is the only choice in such case. Secondly, during SI phase some companies proposed that it is beneficial to allocate a new AS ID via AIoT Msg2 to handle collision case e.g., when device-1 and device-2 performing CBRA both send the same RN16 in AIoT Msg1. However, it may not be feasible to support the allocation of a new AS ID via AIoT Msg2 at least when the Msg2 monitoring window are overlapped/shared by devices. Therefore, we think the benefit to support Option 2 for CBRA procedure has not been justified yet. Based on the above observations, Option 2 is also not preferred.

Figure 1 Example of 3-step CBRA

For Option 3, we understand that it is the combination of Option 1 and Option 2. Typically, when the case that no RN16 is sent in AIoT Msg1 happens (i.e., for a contention-free procedure), the reader can directly assign an AS ID to a device. And when the case that an RN16 is sent in AIoT Msg1 happens (i.e., for a CBRA procedure), the reader can directly reuse RN16 as AS ID. As above, Option 3 has the most flexibility for reader implementation and can overcome both of the disadvantages of the Option 1 and Option 2. Therefore, we propose:

- Option 3 is adopted for AS ID assignment i.e., it is up to the reader whether to reuse the random ID (if used in first D2R message) as the AS ID or to assign a new AS ID.

Issue 2: Whether we need the reader to handle the ID collision

In general, two kinds of potential collision cases can be detected at the reader side:

Case 1: Collision between the received random IDs. For example, the CBRA procedure is triggered by the reader for a group of A-IoT devices (e.g., device-1 and device-2), then device-1 and device-2 respectively may generate and send the random IDs with the same value in its own AIoT Msg1.

Case 2: Collision between the allocated AS ID and the received random ID. For example, the contention free access procedure is firstly triggered by the reader for a single A-IoT device (e.g., device-1). The device-1 has completed the contention free access procedure and been allocated an AS ID for subsequent command type service. Hereafter, the CBRA procedure is triggered by the reader for a group of A-IoT devices (e.g., device-2 and device-3). Then device-3 may generate and send a random ID in its AIoT Msg1, which has the same value as the allocated AS ID for device-1.

Accordingly, there are two ways regarding whether and how to handle the collision cases by the reader.

Way #1: Handle colliding devices as access failure. E.g., the reader will send AIoT Msg2 with NAK to trigger the colliding devices to perform re-access.

Way #2: Handle colliding devices as access success. E.g, the reader will assign a new AS ID to one of the colliding devices to make both devices access successfully.

For Case 1, given that the random ID is 16-bit, we think the probability of multiple A-IoT devices selecting the same random ID for A-IoT Msg1 transmission will be sufficiently low. Hence, supporting Way #1 for Case 1 is sufficient and Way #2 for Case 1 is over optimization. Moreover, Way #2 requires the reader to assign a new AS ID to one of the colliding devices, which is not feasible at least when the Msg2 monitoring window are overlapped/shared among multiple devices. See highlighted Option 1b captured in TR 38.769 as below.

### 6.1.4 Random access For Msg2 transmission in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: - Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device. Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

For Case 2, Way #1 is the baseline solution. While there is feasibility issue for Way #2 similar to that for Case 1, i.e., when the Msg 2 monitoring window for CBRA device are overlapped with the R2D reception for contention free device. Therefore, we propose that:

- At least support the reader to handle the ID collision cases as access failure, i.e., by triggering the colliding devices to perform re-access. Wait for RAN1 progress on AIoT Msg2 transmission/monitoring before decision on whether to support the collision handling as access success e.g., by assigning a new AS ID for some colliding device.

Issue 3: FFS the length of AS ID

In accordance with Option 3 for AS ID assignment i.e., it is up to the reader whether to reuse the random ID (if used in first D2R message) as the AS ID or to assign a new AS ID, it is straightforward that the AS ID and random ID share the value range to uniquely identity a device by the reader. In other words, the AS ID should have the same length as that of the random ID (i.e., 16-bit). Therefore, we propose:

- RAN2 to agree that the length of AS ID is the same as that of random ID i.e., 16-bit.

Issue 4: FFS via which R2D message if the reader assigns a new AS ID

In general, the typical use case to assign AS ID is for contention-free access procedure. While the need and feasibility to assign AS ID for CBRA procedure should wait for more RAN1 progress, as we propose in the above Proposal 2. Candidate R2D messages to assign AS ID is for contention-free access procedure are listed as below:

- Option 1: reader assigns AS ID before Msg1 (e.g. via A-IoT paging);

- Option 2: reader assigns AS ID in the Msg2;

- Option 3: reader assigns AS ID after Msg3 (e.g. via Msg4).

It is suggested that Option 1 is to be excluded at first due to the following disadvantages. Firstly, the device corresponding to the assigned AS ID may be out of the reader coverage, the timing for assignment is too early. Secondly, since AIoT paging message is transmitted in a broadcast way, the other devices which are not intended for the assigned AS ID can also receive the paging payload, which is an signaling inefficient way for AS ID assignment. Last but not the least, even though the WID scope is limited to page one identifier which means one device for contention-free access, put one AS ID in paging can be easy.  However, the TB size of AIoT Paging message has an upper bound, it is not future-proof given that the number of devices to be paged can be huge. And thus, relying on AS ID assignment for intended devices response to the AIoT Paging message is not always feasible. For Option 2 and Option 3, we think Option 2 is more reasonable since Msg2 is the earliest timing that the reader can decide to assign a AS ID for subsequent command service. Therefore, we proposed that:

- If the reader needs to assign a new AS ID (at least for contention-free access procedure), the earliest R2D message for AS ID assignment can be via AIoT Msg2.

Issue 5: FFS if AS ID can be based on partial upper layer device ID

The following text in TR 38.769 are related to how device ID is provided to the reader, see highlighted in yellow.

#### 6.5.1.1 Inventory Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: (1) A-IoT Device Identification (to find a single device, a group of devices, or all devices) NOTE 1: The definition of this identification is out of RAN3 scope. For topology 1, A-IoT RAN needs to store the A-IoT Device Identification received from the inventory request. NOTE 2: Whether to interpret and process AIoT Device Identification received from the inventory request needs further discussion. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered) Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report. NOTE 3: For topology 1, whether device ID received from the device is sent transparently to the A-IoT CN by the A-IoT RAN depends on RAN2 and SA2. NOTE 4: Whether the description in this clause apply to A-IoT enabled gNB in topology 2 needs further discussion and is dependent on SA2 and RAN2.

It is observed that whether the reader can interpret or process Device Identification is still FFS. Moreover, even if the reader can interpret or process Device Identification, device ID is only one type of Device Identification which is used to find a single device. There are other cases when device ID is not available for the reader e.g., to find a group of devices wherein other types of Device Identification like filtering information is provided to the reader instead. As a consequence, we believe that it is not feasible for reader to assign a AS ID based on partial upper layer device ID. From our understanding, this issue can be simplified as an implementation matter on how reader to assign a device specific AS ID (e.g., based on partial upper layer device ID, randomly generated etc), as long as the uniqueness of the assigned AS ID is guaranteed per reader scope. Therefore, we propose:

- It is up to reader implementation on how reader assigns an AS ID to a device as long as the uniqueness of the assigned AS ID is guaranteed per reader scope.

Issue 6: Whether and how to maintain the AS ID

The AS ID is somewhat similar to C-RNTI for legacy RRC_CONNECTED UE. The difference is that legacy UE has RRC connection management procedure to manage C-RNTI, e.g., release it when back to RRC_IDLE, but AIoT device has no RRC state and connection concept. It should be addressed how to release the AS ID after this ID is no longer used. Explicit release indication from reader is the most straightforward solution, e.g., upon reader reception of service ending indication from CN. Furthermore, the AS ID can also be released based on a pre-defined/configured timer expiry. The timer will be restarted upon each R2D/D2R transmission and separate timers can be maintained at both device and reader side. When timer expires, the AS ID is released respectively at device and reader side without explicit release signaling. Therefore, we propose:

- The AS ID can be maintained at device and reader side, and released on a timer expiry or an explicit indication from reader.

# 2.2.	MAC PDU format

The MAC PDU can be used for different purposes: paging, RACH access, RACH response and upper layer data transmission. In the following, the MAC PDU structure/content for data transmission is discussed, while the MAC PDU structure/content for other purposes are to be discussed in the companion papers in this meeting.

When the reader transmits upper layer data to the target AIoT device(s), the corresponding R2D MAC PDU should carry the AS ID of the target AIoT device, so that the target AIoT device can identify that it is the desired target AIoT device. In the meanwhile, there can be a D2R message in response from the AIoT device. In the PRDCH transmitting the R2D MAC PDU, the scheduling information for the response D2R message transmission may be included.

To enable an AIoT device to identify the R2D MAC PDU structure and derive the content, a message type field can be included in R2D MAC PDU, wherein the message type field indicates the MAC PDU format /content, some parameters could optionally be present and there could be some associated fields to indicate the presence of these optional fields. The message type indication in the R2D MAC PDU can also indicate whether there is upper layer data in the R2D MAC PDU.

With respect to the transmission of R2D reception/D2R scheduling information, whether the D2R/R2D scheduling message is transmitted as L1 R2D control signalling or as higher layer signalling (i.e. as a part of MAC PDU) is still for FFS in RAN1 according to the TR [2]. The corresponding part is copied below for reference:

#### 6.1.1.9 Scheduling of R2D For R2D reception, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - ID associated with device(s) intended for the reception of R2D, potentially including all devices (if supported) For each information, it is for further study whether higher-layer signalling and/or L1 R2D control signalling is used - see Clause 6.1.1.10. #### 6.1.1.10 R2D control information For transmission of R2D control information for R2D reception and D2R scheduling, following two options are studied and captured in the TR 38.769: - Option 1: L1-control signaling - Option 2: Higher-layer signaling […]

If it is determined that the scheduling message (i.e. the R2D control information) is transmitted as a part of MAC PDU, the MAC PDU may also carry the R2D/D2R radio resource allocation information.

Based on the above discussion, the following R2D MAC PDU structure is proposed:

- Design an R2D MAC PDU format for data transmission comprising:

- message type, which indicates the R2D MAC PDU format/content;

- associated parameters, at least AS ID;

- R2D control information for R2D reception/D2R scheduling, depending on RAN1;

- associated upper layer data, if any.

When a R2D MAC PDU is received, the AIoT device may also generate a D2R MAC PDU for subsequent response based on the content of the R2D MAC PDU. Similar to the R2D MAC PDU format, the D2R MAC PDU should also carry the AS ID so that the reader can determine that the received R2D message is the response to the R2D MAC PDU and is from the target AIoT device. For one instance, the AIoT device may response with a MAC PDU comprising an upper layer ID (e.g. EPC ID) upon reception of Msg2 from the reader in 3-step RA procedure.  For another instance, the D2R MAC PDU in response to the R2D MAC PDU comprising read command may comprise the associated upper layer data.  Further, if segmentation occurs for upper layer data, the D2R MAC PDU carrying the first segment of the upper layer data may also carry the remaining D2R message size report (see clause 2.4).

- Design an D2R MAC PDU format for data transmission comprising:

- message type, which indicates the D2R MAC PDU format/content;

- associated parameters, at least AS ID;

- remaining D2R message size, if any;

- associated upper layer data, if any.

# 2.3.	Segmentation

In the TR[2], there are the following summary regarding segmentation:

The segmentation study focuses on D2R direction (it can be further considered on R2D direction). For the candidate solution for segmentation: - The sequence number, the segment number and the number of segments are not supported; - An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details; - It is beneficial for the reader to be able to trigger a re-transmission of a segment; - It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

# 2.3.1. Triggering of D2R segmentation

One basic issue is the condition to transmit the upper layer data in segments. The reasonable policy for segment-based transmission of the upper layer message should be:

- If the TBS (i.e. transport block size) is sufficient for carrying the whole D2R message, the AIoT device should just transmit the whole D2R message in one shot;

- Otherwise, i.e. if the TBS has some available room for upper layer message transmission but not enough to carry the whole D2R message, the AIoT device should segment the D2R message according to the available room for upper layer message transmission and transmits the D2R message segment by segment in multiple transmissions.

Based the above discussion, we propose:

- RAN2 assumes that an AIoT device performs D2R segmentation when the message size is larger than the TBS.

# 2.3.2. D2R Segmentation scheme

For segmentation, there is an issue on how to generate the D2R segment by the AIoT device upon reception of a D2R grant. An AIoT device should at least know the start position in the memory for fetching the target upper D2R message and the segment size to determine the amount of data to be fetched from the upper layer D2R message, in order to generate the segment that can well fit the available capacity of the D2R grant, wherein the start position of the whole upper layer message (i.e. the start position of the first segment of upper layer data) is determined by the upper layer signal received in the R2D MAC PDU while the segment size is determined based on the TBS of the D2R grant.

For start position of upper layer message for deriving a segment after the first segment, there are two schemes:

- Scheme 1: Explicit message offset indication based D2R segmentation

The reader sends to the AIoT device an explicit message offset, using which the AIoT device fetch the D2R segment from the upper layer or deliver the received segment to lower layer for D2R TX. The reader should maintain the message offset based on the reception status of each segment of one upper layer D2R message. For one instance, the reader can shift the message offset for scheduling the transmission of the next segment once a D2R segment is correctly received and there is still remaining D2R segment(s).

- Scheme 2: Local message offset based D2R segmentation

The AIoT device itself maintains a local message offset based on the accumulated total size of all transmitted segments. When the reader schedules the AIoT device to transmit the next segment, the AIoT device fetches the data from the byte after the end byte of the old segment, wherein the end byte of the last segment is identified by a local message offset maintained by the AIoT device itself. For the first segment, the local message offset is zero. The AIoT device should record local message offset and shift the local message offset by a segment size when a segment is generated and correctly transmitted.

Scheme 1 means that the message offset for segmentation is explicitly signalled to AIoT device, while the benefit is that the AIoT device does not have to maintain a local message offset. Scheme 2 saves the message offset transmission signalling at the cost of the additional complexity of local message offset maintenance at AIoT device side.

- RAN2 to down-select from the following two D2R segmentation schemes：

- Scheme 1: Explicit message offset indication from the reader to AIoT device based D2R segmentation.

- Scheme 2: AIoT device generates and maintains a local message offset based D2R segmentation.

# 2.3.3. D2R Segment transmission failure handling

The following conclusion regarding protocol stack from the TR[2] (in Clause 6.3.2 Protocol stack and functionality aspects) should be confirmed before discussing the detail solutions for segment retransmission.

As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): ……………………….non-related part is omitted ………………………...… - Per-packet QoS and per-QoS flow at AS level - HARQ - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. ……………………….non-related part is omitted ………………………...…

From the above reference, we can observe:

- HARQ is not supported and there is no AS layer buffer for buffering a MAC PDU.

As there is no upper layer message stored in MAC layer, in case of D2R segment transmission fails, the segment retransmission schemes with respect to Segmentation Scheme 1 and Scheme 2 are analysed respectively below:

If Segmentation Scheme 1 is applied, the message offset of an D2R upper layer message is maintained by the reader. Upon detection of a D2R segment failure, the reader may indicate the same message offset as the initial transmission of the D2R segment for scheduling the corresponding retransmissions. From AIoT device perspective, it can just generate a new MAC PDU based on the received message offset indication, in the same way as MAC PDU generation for initial transmission of a segment. It is not necessary for the AIoT device to know whether a D2R scheduling message from the reader is for the next segment or for a retransmission corresponding to a D2R segment transmission failure.

If Segmentation Scheme 2 is adopted, the message offset is maintained locally by the AIoT device itself. Before a new segment transmission or segment retransmission, the AIoT device should determine whether to shift the local message offset based on the D2R scheduling message.

When the D2R scheduling message is for scheduling retransmission of a segment, the AIoT device can determine the segment just transmitted is not correctly received by the reader and it should not shift the local message offset, and reuse the existing message offset for fetching upper layer data for the segment for retransmission.

If Segmentation Scheme 2 is adopted, the AIoT device should be informed whether to shift the local message offset or not for fetch data for the segment to be transmitted. To achieve this, some indicator should be introduced in the scheduling message from the reader to the AIoT device.

Therefore,

- For D2R Segment transmission failure handling, one of the following solutions can be considered:

- If the segmentation offset is explicitly indicated to AIoT device, the reader schedules the retransmission corresponding to a segment transmission failure by indicating the same message offset as for initial transmission of the segment;

- If the segmentation offset is locally managed by AIoT device, the reader schedules the retransmission corresponding to a segment transmission failure by indicating a retransmission indicator.

Furthermore, there is an issue on whether segment retransmission corresponding to the reception failure of the first segment of the upper layer D2R message is supported. In the TR [2]，there are the following general conclusion regarding upper layer (MAC and above) retransmission in 6.3.2 Protocol stack and functionality aspects：

As to the A-IoT required functionalities, at least the following functionalities are NOT supported (see TS 38.300 [153] for references for any legacy NR functionality): … - RLC ARQ/AM - AS-layer (above physical layer) RLC-like/ARQ-like retransmission - AS-layer (above physical layer) repetition NOTE 1: It is not precluded that the reader and A-IoT device send the "payload" again as new transmission from A-IoT MAC perspective. …

- RAN2 does not explicitly support of MAC retransmission of unsegmented upper layer message.

And in the 6.3.5	A-IoT data transmission in the TR [2], there are the following regarding segment retransmission:

- It is beneficial for the reader to be able to trigger a re-transmission of a segment;

- RAN2 concluded that segment retransmission in MAC is beneficial.

In practice, it could happen that the reader has failed to receive the first segment of a D2R message. In this case, the reader does not know that whether segmentation occurs for the failed transmission or not as the end/non-end indication associated with the first segment is also lost due to the failed transmission. From the reader perspective, the failed transmission could be either for transmitting a segment of upper layer D2R message or a whole D2R message. In case the failed transmission is for transmission a whole D2R message, the AIoT device does not keep the D2R message related context (e.g. the memory information or filename of the upper layer message, see clause 2.3.5) after the transmission. In this condition, if the UE receives a grant for segment retransmission, the UE has ambiguity to determine the upper layer D2R message for the retransmission.

To avoid such ambiguity, we propose:

- Segment retransmission corresponding to the reception failure of the first segment of the upper layer D2R message is not supported.

# 2.3.4. End/non-end segment indication

As RAN2 agreed, there is yet another issue on whether to use single bit or two bits to indicate end segment or non-end segment. If single bit is used, the UE can indicate end segment or non-end segment for one upper layer message with one bit associated with each segment. If two bits are used, the UE can indicate start segment, middle segment, end segment or unsegmented transmission with two bits associated with each segment. An example is shown in below Figure 2. One benefit to indicate start segment of one message is to avoid the segments two neighboring messages mixed with each other if the end segment of the preceding message is lost.

Figure 2 Segment indication with two bits

However, for AIoT service, the reasonable assumption can be that there is only single upper layer message to be transmitted from AIoT device to the reader (Read) for one Write command or Read command. With this assumption, there is no risk of mixing the segments of two neighboring messages for one AIoT service.

- The benefit of two-bit segment indication may be to avoid the segments of two neighboring upper layer messages mixed together if the end segment of the preceding upper layer message is lost.

- For Read, it can be assumed that there is only single upper layer message to be transmitted for one Read cmd, i.e., there is no risk of mixing segments of two neighbouring upper layer messages for one AIoT Read service.

The rest issue is how to indicate the transmission of one whole message without segmentation if single-bit end/non-end segment indication is applied. In this case, the transmitter can just set the single bit associated with the unsegmented upper layer message to end-segment indication. For scheduled transmissions, when the receiver receives a “segment” with end-segment indication but without any preceding segment(s) with non-end segment indication, the receiver can identify that the received “segment” carries one whole upper layer message.

- For MAC segmentation, it is enough to only indicate end-segment or non-end segment, i.e., indication of unsegmented message or middle segment is not needed.

Based on the above discussion, it can be concluded that single-bit segment indication is enough. To reduce the larger overhead caused by 2-bit segment indication, we propose:

- Support single-bit end/non-end segment indicator associated with each segment for D2R transmissions.

- If Proposal 13 is agreeable, if a MAC PDU contains one whole D2R upper layer message, the single-bit end/non-end segment indicator indicating transmission end should be included.

# 2.3.5. D2R message context storage

For an upper layer D2R message transmission in form of segments, for example, in response to a read cmd sent to AIoT device by the reader, the AIoT device should keep the context of the read cmd, for instance, the upper layer D2R message identifier, the file identifier or the memory block information for the upper layer D2R message.  Upon reception of the scheduling message either for segment retransmission or new segment transmission, the AIoT can identify the upper layer message for deriving the segment for transmission or retransmission based on the stored context of the read cmd.

Another option could be that the reader transmits the upper layer command message to the AIoT device associated with each scheduling message for transmission/retransmission of each segment. This would generate a large R2D overhead. Considering this, we prefer the AIoT device to store the upper layer cmd message (e.g., read) related context in relation to the D2R message for transmission in form of segments.

- The AIoT device should store the necessary upper layer context (e.g., D2R message identifier, the related file information/memory identifier) about the upper layer D2R message to be transmitted in form of segments.

# 2.4.	D2R message size report

In the TR of Ambient IoT [2], there were the following conclusions regarding the D2R message size awareness for AIoT and D2R message size report:

- It is beneficial for the reader to know an estimate of expected D2R message size. This D2R message size information would be beneficial but it is not essential. Two options are studied: - Option 1: The estimate of expected following D2R message size from the CN to the reader; - Advantages/disadvantages of option 1: If this information is available from CN, it saves the overhead/latency caused by Option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed. - Option 2: The simple message size indication from the device to the reader. - Advantages/disadvantages of option 2: It is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

A LS [3] was sent from RAN2 to SA2 regarding on the message size awareness. In the recent LS reply from SA2 [4], there is no corresponding answer on whether CN can inform the reader regarding the D2R message size. However, RAN2 may assume that D2R message report may still be needed at least in some cases.

For D2R message report, there are two options:

- Alt.1 Report the size of the whole D2R message;

- Alt.2 Remaining D2R message size report, i.e. report the size of the remaining D2R message part.

With Alt.1, for a D2R message, the reader needs to derive the amount of remaining message bits based on the received D2R message size minus the total number of bits of all received D2R segments. While for Alt.2, the reader can directly determine the amount of the remaining message bits. In such sense, Alt.2 leads lower reader complexity.

- Support to report the remaining D2R message size (i.e., the amount of the remaining D2R message bytes/bits) from AIoT device to reader.

Upon reception of a D2R grant for D2R message transmission, the AIoT device should firstly determine if the available resources are enough to carry whole D2R message. If yes, the remaining message size is zero and no remaining D2R message size is not reported.  When the AIoT device determines that there will still be some remaining message bits after the transmission of the first segment, remaining D2R message size report is triggered and submitted in the MAC PDU comprising/carrying the first segment of D2R message.

- The remaining D2R message size report is transmitted in the MAC PDU carrying the first segment of the D2R message.

# Conclusion

In this contribution, remaining issues on AIoT data transmission and other general aspects are discussed, and we have the following observations:

- HARQ is not supported and there is no AS layer buffer for buffering a MAC PDU.

- RAN2 does not explicitly support of MAC retransmission of unsegmented upper layer message.

- RAN2 concluded that segment retransmission in MAC is beneficial.

- The benefit of two-bit segment indication may be to avoid the segments of two neighboring upper layer messages mixed together if the end segment of the preceding upper layer message is lost.

- For Read, it can be assumed that there is only single upper layer message to be transmitted for one Read cmd, i.e., there is no risk of mixing segments of two neighbouring upper layer messages for one AIoT Read service.

- For MAC segmentation, it is enough to only indicate end-segment or non-end segment, i.e., indication of unsegmented message or middle segment is not needed.

Based the above discussions and observations, we have the following proposals:

AS ID

- Option 3 is adopted for AS ID assignment i.e., it is up to the reader whether to reuse the random ID (if used in first D2R message) as the AS ID or to assign a new AS ID.

- At least support the reader to handle the ID collision cases as access failure, i.e., by triggering the colliding devices to perform re-access. Wait for RAN1 progress on AIoT Msg2 transmission/monitoring before decision on whether to support the collision handling as access success e.g., by assigning a new AS ID for some colliding device.

- RAN2 to agree that the length of AS ID is the same as that of random ID i.e., 16-bit.

- If the reader needs to assign a new AS ID (at least for contention-free access procedure), the earliest R2D message for AS ID assignment can be via AIoT Msg2.

- It is up to reader implementation on how reader assigns an AS ID to a device as long as the uniqueness of the assigned AS ID is guaranteed per reader scope.

- The AS ID can be maintained at device and reader side, and released on a timer expiry or an explicit indication from reader.

MAC PDU format

- Design an R2D MAC PDU format for data transmission comprising:

- message type, which indicates the R2D MAC PDU format/content;

- associated parameters, at least AS ID;

- R2D control information for R2D reception/D2R scheduling, depending on RAN1;

- associated upper layer data, if any.

- Design an D2R MAC PDU format for data transmission comprising:

- message type, which indicates the D2R MAC PDU format/content;

- associated parameters, at least AS ID;

- remaining D2R message size, if any;

- associated upper layer data, if any.

Segmentation

- RAN2 assumes that an AIoT device performs D2R segmentation when the message size is larger than the TBS.

- RAN2 to down-select from the following two D2R segmentation schemes：

- Scheme 1: Explicit message offset indication from the reader to AIoT device based D2R segmentation;

- Scheme 2: AIoT device generates and maintains a local message offset based D2R segmentation.

- For D2R Segment transmission failure handling, one of the following solutions can be considered:

- If the segmentation offset is explicitly indicated to AIoT device, the reader schedules the retransmission corresponding to a segment transmission failure by indicating the same message offset as for initial transmission of the segment;

- If the segmentation offset is locally managed by AIoT device, the reader schedules the retransmission corresponding to a segment transmission failure by indicating a retransmission indicator.

- Segment retransmission corresponding to the reception failure of the first segment of the upper layer D2R message is not supported.

- Support single-bit end/non-end segment indicator associated with each segment for D2R transmissions.

- If Proposal 13 is agreeable, if a MAC PDU contains one whole D2R upper layer message, the single-bit end/non-end segment indicator indicating transmission end should be included.

- The AIoT device should store the necessary upper layer context (e.g., D2R message identifier, the related file information/memory identifier) about the upper layer D2R message to be transmitted in form of segments.

D2R message size report

- Support to report the remaining D2R message size (i.e., the amount of the remaining D2R message bytes/bits) from AIoT device to reader.

- The remaining D2R message size report is transmitted in the MAC PDU carrying the first segment of the D2R message.

- Reference

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

- 3GPP TR 38.769 V19.0.0 (2024-12) Study on solutions for ambient IoT (Internet of Things) in NR.

- R2-2409412 LS on assistance information from the CN to the Reader for A-IoT.

- S2-2501241 Reply LS on Assistance Information from CN to the Reader.