3GPP TSG-RAN WG2 Meeting #129	R2-2500135

Athens, Greece, Feb. 17th – 21st , 2025

Agenda Item:	8.4.2

Source:	MediaTek Inc.

Title:	Discussion of LR and MR operating in same/different frequency band

Document for:	Discussion, Decision

Introduction

The topic of LR and MR operating in different bands/frequencies has been discussed in previous WG and plenary meetings. In the last TSG RAN#106 meeting, the scenarios for the same/different bands/frequencies has been further analysed [1]. Descriptions can be found in the following table.

Table 1 Scenarios for LR and MR operating in the same/different bands/frequencies

| Scenarios |  | Description |
| --- | --- | --- |
| Intra-frequency | Solution 1 | MR is prioritized (via SIBs) to camp on the same band as LR. (System information, Paging, RACH on LR band) |
|  | Solution 2 | MR is prioritized to camp on the same band as LR. (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) |
|  | Solution 3 | MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) |
| Inter-frequency | Solution 4 | MR (SI/Paging/RACH) is camped on a different band than LR. LR can wake up MR on the band it is camped on |

During the meeting, the way forward on Rel-19 LP-WUS separate band issues has been endorsed:

Way Forward on LP-WUS: - For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs

In this contribution, we made further analysis based on the above solutions and made the corresponding proposals.

Discussion

Inter-frequency LR and MR operation

In scenario 4 above, it is described that LR operates in one frequency band while MR is camped on another. When UE is triggered by LP-WUS, the MP receiver needs to acquire system information in the MR band and perform reselection to the MR frequency. This inter-band operation introduces additional measurement and synchronization latency for the UE to wake up and switch to the other band.

Additionally, it is unclear how to define the entry/exit condition by two different frequency measurement in Idle state. Currently, the entry condition is defined by serving cell quality measured from MR (as baseline), and the exit condition is defined by serving cell quality measured from LR. If LR and MR are in different frequency, UE may need to measure on both LR and MR or introduce new measurement mapping rules between the two frequencies. Either approach increases UE complexity/consumption and measurement requirement overload in RAN4. The different LR/MR frequency range may also be unavailable due to mobility, which requires additional study and specification work.

Observation 1: LR and MR camped on different frequency cause extra wake up latency and more specification work on RAN4 RRM requirement.

As guide by the RP, the way forward has been endorsed that the inter-frequency solution for LR and MR operation will not be considered in Rel-19. We would like to confirm this understanding here:

Proposal 1: For Rel-19 LP-WUS, the LR and MR operate on the same frequency/band.

Intra-frequency solutions

In the intra-frequency solutions discussed in plenary (i.e., solution 1,2,3), the exemplary procedures can be summarized in the following figure:

Figure 1 Exemplary procedures for scenarios under intra-frequency solutions

The differences of the three solutions are summarized in the following table:

Table 2 Analysis on scenarios for LR and MR operating in the same frequency band

|  | LP-WUS frequency prioritization | Paging received in | RACH in | Band in CONNECTED mode |
| --- | --- | --- | --- | --- |
| Solution 1 | Yes | LP-WUS band | LP-WUS band | LP-WUS band (or another MR band by NW implementation) |
| Solution 2 | Yes | LP-WUS band | Another MR band (based on reselection criteria) | MR band |
| Solution 3 | No | LP-WUS band | LP-WUS band | MR band (Re-directed via dedicated signalling by NW implementation) |

Discussion on solution 2

In solution 2, the system information and paging message is received in LP-WUS band (BW) while the RACH is performed in MR band (BW) by UE re-selection after paging.

However, the only benefit of solution 2 compared to solution 3 or 1 is to release RACH overload (not the subsequent data overload in RRC CONNECTED mode) in MR band. In 5G, the capacity of RACH to LR band should be sufficient, as not many UEs will implement R19 LP-WUS. Also, this cause more latency to wake up as the system information in the new MR band need to be acquired first. It may also introduce discussion on new cell re-selection requirement in RAN4, which may be an overload by the end of Rel-19.

Observation 2: In R19 LP-WUS, performing re-selection and RACH to another MR band after received paging (i.e., solution 2) has marginal benefit in releasing RACH overload with more latency and spec complexity.

Moreover, this new UE behaviour in solution 2 should not be binded to LP-WUS, since the trigger can be paging message itself, and UE not supporting LP-WUS can also apply this new behaviour (i.e., re-select to another band and perform RACH). Therefore, it requires independent study and would be more appropriate to discuss in a future release.

Observation 3: Re-selecting and perform RACH toward another band after receive paging is a new UE behavior and should be independent to R19 LP-WUS.

Therefore, we propose to not support this new feature in R19 LP-WUS.

Proposal 2: LP-WUS capable UE performs RACH in the same band after receiving paging message.

Discussion on solution 1 and 3

Solution 1 focus on prioritizing LP-WUS frequency band, so the LP-WUS capable UE are more likely to camp on and access to that band. This can be beneficial for UEs that support LP-WUS in that frequency to have lower power consumption. However, if more LP-WUS capable UEs camp on the LP-WUS band due to frequency priority configuration broadcasted in the SIB, it may lead to band overload and cause access congestion when those UEs wake up. It also requires additional overhead in the SIB to add signaling to distinguish LP-WUS capable/non-capable UEs and provide different frequent priorities.

In Solution 3, UEs are not prioritized to the LP-WUS band but can be redirected or switched to another MR band via dedicated signaling by network implementation. The UE can either be released with redirection to the MR band or directly handover to MR band after entering RRC CONNECTED. This can offload some LP-WUS UEs to another MR band, depending on the load in the LP-WUS band and network configuration. This method is more flexible and should be more suitable for load balancing. Therefore, it can be agreed upon first.

Proposal 3: Dedicated signaling is supported to re-direct/handover LP-WUS UE to another MR band after RACH.

Solutions 1 and 3 are not mutually exclusive. A UE prioritized to camp on the LP-WUS band can also be redirected to another MR band based on network implementation to mitigate congestion on the LP-WUS frequency. And it might be beneficial in some scenario that some LP-WUS capable UEs prioritize to camp on the LP-WUS frequency, while other UE camp on other cells. If different frequency priority rules can be used for LP-WUS capable/non-capable UEs, LP-WUS capable UEs will camp on LR band first and no need for the following dedicated signaling.

However, as mentioned above. Broadcast this frequency priority via SIB may cause overload in LR band if more LP-WUS capable UEs camp on this band. But anyway, if this method is agreed, it is still up to the NW to implement it or not, which can depend on different scenarios.

With the support of dedicated signaling, RAN2 can continue to discuss the potential benefits and impact of supporting frequency prioritization for the LP-WUS frequency.

Proposal 4: RAN2 to discuss whether to introduce different frequency priority rules in SIB for LP-WUS capable/non-capable UEs.

Conclusion

In this contribution, the following observations and proposals are made:

Observation 1: LR and MR camped on different frequency cause extra wake up latency and more specification work on RAN4 RRM requirement.

Observation 2: In R19 LP-WUS, performing re-selection and RACH to another MR band after received paging (i.e., solution 2) has marginal benefit in releasing RACH overload with more latency and spec complexity.

Observation 3: Re-selecting and perform RACH toward another band after receive paging is a new UE behavior and should be independent to R19 LP-WUS.

Proposal 1: For Rel-19 LP-WUS, the LR and MR operate on the same frequency/band.

Proposal 2: LP-WUS capable UE performs RACH in the same band after receiving paging message.

Proposal 3: Dedicated signaling is supported to re-direct/handover LP-WUS UE to another MR band after RACH.

Proposal 4: RAN2 to discuss whether to introduce different frequency priority rules in SIB for LP-WUS capable/non-capable UEs.

References

- RP-242656 On LR and MR operating in different bands in release 19. Vodafone, Vivo,

- Huawei, HiSilicon

- RP-243266 Way forward on Rel-19 LP-WUS separate band issues RAN2 chair (InterDigital)

- R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode