3GPP TSG-RAN WG2 Meeting #129	R2- 2500148

Athens, Greece, 17th Feb – 21th Feb, 2025

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

According to the RAN plenary #106 meeting, the approved work item on NTN for NR contains the following objective on DL coverage enhancements [1]:

The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

During RAN2 #128 meeting, RAN2 made some progress with the following statements:

Agreements:

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

- As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

- RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

- For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states

In this contribution, we provide our views on DL coverage enhancements.

2. Discussion

SMTC impacts due to extended SSB periodicity

In the RAN2#128, there was discussion on SMTC impacts due to the extended SSB periodicity for the DL-CE cell. Especially, introducing a new SMTC list for the DL-CE cell was discussed as below.

Agreements:

- RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

As the SSB periodicity assumed by UE during initial access is extended for the DL-CE cell, the SSB periodicity of the DL-CE cell may be different from the SSB periodicity of the legacy non-DL-CE cell [1]. Also, if the cells are served by the different NTN payload, the separate SMTC with different offsets are required to measure the cells. Considering the above-mentioned case, the UE may need to measure neighboring cells consisting of cells with different SSB periodicities and offsets for cell reselection. In that case, cells with SSB periodicities and offsets that differ from those of the serving cell or other cells require their own distinct SMTCs. However, the current SMTC list for the NTN cell measurement, i.e., SMTC4list, does not support having different SMTC periodicity from the SMTC1.

Observation 1	There may exist neighbor cells with different SSB periodicities and offsets from other neighbor cells or a serving cell.

Observation 2	The SMTC4list, configured for the NTN cell, cannot support different SMTC periodicity from the SMTC1.

If the SMTC cannot be configured with a different periodicity from the SMTC1, some of the cell's SMTCs are not associated with its own SSB periodicity but instead are linked to the SSB periodicity or SMTC periodicity of other cells. In that case, the UE may perform measurements, even when the cell is not transmitting SSB. Conversely, the UE may not perform measurements, even when the cell is transmitting SSB. Therefore, the SMTC configuration that does not consider the cell's SSB transmission periodicity could result in unnecessary UE power consumption or delays in the cell reselection procedure due to the longer SS-RSRP measurement.

In addition, the above-mentioned scenario is not limited to DL-CE cells having different SSB periodicities from non-DL-CE cells. The DL-CE cells may also have different SSB periodicities from each other, depending on the cell's traffic or the power capacity of the NTN payload. Therefore, a new SMTC list should be introduced to configure separate periodicity and offset from the SMTC1.

Observation 3	The SMTC configuration that does not consider the cell's SSB transmission periodicity may increase the UE power consumption or the latency of the cell reselection procedure.

Proposal 1	RAN2 should introduce a new SMTC list, which can have different periodicity and offset from the SMTC1.

Related UE power consumption issue

In the RAN2#128 meeting, the issue of UE power consumption, caused by the SMTC impact from the extended SSB periodicity, was raised. Based on the SMTC discussions in previous meetings, we understand that the UE power consumption issue is related to the number of SMTCs configured by the network. As the number of SMTCs increases for measuring neighbor cells, the UE must spend more time and power on SSB measurements using those SMTCs.

And the reason why the extended SSB periodicity increases the number of SMTCs required by the UE is as follows. If the SSB periodicity is extended, and if the different SSB transmission timings are applied among cells through SSB beam-hopping transmission, UE might require different SMTCs to measure neighbor cells.  For example, the number of SMTCs to measure the adjacent neighbor cells can be six at maximum, as the number of neighbor cells adjacent to a serving cell is six. As neighbor cells have different SSB transmission timing offsets, the number of SMTCs required for measurement increases, resulting in a higher power burden on the UE, and which may not be supported with the current UE capacity.

Observation 4	As the SSB periodicity is extended, the number of SMTCs required for measurement might increase, which leads to the UE’s power burden.

In principle, even if the SSB periodicity is extended and the network transmits SSBs via beam hopping, multiple cells will transmit SSBs with the same SSB transmission timing. And network can group the cells adjacent to each other, ensuring that adjacent cells are placed in the same SSB beam hopping area. The cells included in the same group have the same SBB transmission timing. For example, in the good case, the UE only needs one SMTC to measure all neighbor cells adjacent to a serving cell. And, in the worst case, the UE needs 3 SMTCs to measure all neighbor cells adjacent to a serving cell as shown in figure below. Therefore, the network implementation can minimize the increase in SMTCs required for neighbor cell measurements caused by the extended SSB periodicity and SSB beam hopping. Enabling the UE to perform measurements with fewer SMTCs through network implementation can reduce the UE's power burden.

Figure 1. Illustrating the SSB beam hopping area and the worst-case scenario.

Observation 5	Enabling the UE to perform measurements with fewer SMTCs through network implementation can reduce the UE's power burden.

Proposal 2	RAN2 assumes that there is no UE power consumption issue due to the SMTC impact.

Access Control

From the last several meetings, RAN2 made some progress about the access control mechanism for DL-CE cell as below.

Agreements:

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

However, from the last RAN plenary meeting, it was determined that the maximum additional SSB periodicity assumed by the UE during the initial cell search procedure is 160ms, which is the supported SSB periodicity in non-DL-CE cells. Additionally, the system level enhancements of the DL-CE cell do not consider the Rel-18 network energy saving techniques, according to the updated WID [1]. Based on this, if the UE successfully camps on DL-CE cells, there is no backward compatibility issue. Therefore, UEs that do not support DL-CE should not be barred from the DL-CE cells. In conclusion, there is no need to discuss using separate access control for DL-CE cells.

Proposal 3	RAN2 assumes that the UE not supporting DL-CE should not be barred from the DL-CE cell.

Proposal 4	RAN2 assumes that the DL-CE cell does not require a separate access control mechanism from the non-DL-CE cell.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1	There may exist neighbor cells with different SSB periodicities and offsets from other neighbor cells or a serving cell.

Observation 2	The SMTC4list, configured for the NTN cell, cannot support different SMTC periodicity from the SMTC1.

Observation 3	The SMTC configuration that does not consider the cell's SSB transmission periodicity may increase the UE power consumption or the latency of the cell reselection procedure.

Proposal 1	RAN2 should introduce a new SMTC list, which can have different periodicity and offset from the SMTC1.

Observation 4	As the SSB periodicity is extended, the number of SMTCs required for measurement might increase, which leads to the UE’s power burden

Observation 5	Enabling the UE to perform measurements with fewer SMTCs through network implementation can reduce the UE's power burden.

Proposal 2	RAN2 assumes that there is no UE power consumption issue due to the SMTC impact.

Proposal 3	RAN2 assumes that the UE not supporting DL-CE should not be barred from the DL-CE cell.

Proposal 4	RAN2 assumes that the DL-CE cell does not require a separate access control mechanism from the non-DL-CE cell.

4. References

[1] RP-243300	Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3	Moderator (THALES).