3GPP TSG-RAN WG2 #129	R2-2500151

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.2

Source: 	MediaTek Inc.

Title: 	Rel-18 LTM issues

Document for:	Discussion

# 1 	Introduction

In this paper, we discuss following issues for Rel-18 LTM:

- Missing early UL sync parameters for TDD LTM candidate

- Unspecified UE behaviour for NZP-CSI-RS-Resource IE

- Usage of LTM reference configuration

- RS alignment between LTM TCI state and CFRA resource

- Ambiguity of UE capability pdcch-RACH-AffectedBandsList-r18

# 2 	Discussion

## 2.1 Missing early UL sync parameters for TDD LTM candidate

In EarlyUL-SyncConfig IE, legacy IE FrequencyInfoUL is used for UL frequency information. But since the FrequencyInfoUL IE is originally specified for serving cell configuration, it does not contain frequency band indicator or absolute frequency of Point A for TDD, see condition FDD-OrSUL. (For TDD serving cell, the UE uses these parameters from FrequencyInfoDL.)

EarlyUL-SyncConfig-r18 ::=         SEQUENCE {

frequencyInfoUL-r18                FrequencyInfoUL,

rach-ConfigGeneric-r18             RACH-ConfigGeneric,

bwp-GenericParameters-r18          BWP,

ssb-PerRACH-Occasion-r18           ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL, -- Need R

prach-RootSequenceIndex-r18        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                             OPTIONAL, -- Need R

ltm-PRACH-SubcarrierSpacing-r18    SubcarrierSpacing                                                          OPTIONAL, -- Cond L139

n-TimingAdvanceOffset-r18          ENUMERATED { n0, n25600, n39936, spare1 }                                  OPTIONAL, -- Need R

...,

[[

ltm-tdd-UL-DL-ConfigurationCommon-r18 TDD-UL-DL-ConfigCommon                                                  OPTIONAL, -- Cond TDD

ltm-restrictedSetConfig-r18        ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL  -- Need R

]]

}

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

additionalSpectrumEmission-v1760    AdditionalSpectrumEmission-v1760                        OPTIONAL    -- Need S

]],

[[

additionalSpectrumEmissionAerial-r18   AdditionalSpectrumEmission-r18                       OPTIONAL   -- Need S

]]

}

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

Since there is no frequency band indicator or absolute frequency of Point A anywhere within LTM-Candidate IE, the UE has no knowledge of this information for a TDD LTM candidate for early UL sync purposes.

Based on current ASN.1, the does not know frequency band indicator or absolute frequency of Point A for a TDD LTM candidate for early UL sync purposes.

We think that frequency band indicator and absolute frequency of Point A are fundamental parameters for the UE to know for an UL carrier, so we think the network should always signal fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE, regardless of the duplex mode of the LTM candidate. Without the parameters, the UE cannot transmit RA preamble for the early UL sync purposes.

The network always signals fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE (TP in Annex A).

## 2.2 Unspecified UE behaviour for NZP-CSI-RS-Resource IE

In LTM-TCI-Info IE (within LTM-Candidate IE), a list of NZP-CSI-RS resources (of type NZP-CSI-RS-Resource IE) are signalled with ltm-NZP-CSI-RS-ResourceToAddModList field:

LTM-TCI-Info-r18 ::=             SEQUENCE {

ltm-DL-OrJointTCI-StateToAddModList-r18        SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18

OPTIONAL,    -- Need N

ltm-DL-OrJointTCI-StateToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF TCI-StateId

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToAddModList-r18               SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18

OPTIONAL,    -- Need N

ltm-UL-TCI-StateToReleaseList-r18              SEQUENCE (SIZE (1.. maxNrofCandidateUL-TCI-r18)) OF TCI-UL-StateId-r17

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToAddModList-r18        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r18       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r18     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,    -- Need N

pathlossReferenceRS-ToAddModList-r18           SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17

OPTIONAL,    -- Need N

pathlossReferenceRS-ToReleaseList-r18          SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-Id-r17

OPTIONAL,    -- Need N

unifiedTCI-StateType-r18                       ENUMERATED {separate, joint}                          OPTIONAL,    -- Need R

...

}

In NZP-CSI-RS-Resource IE, three LTM specific fields with condition LTM exist:

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present in an LTM-Candidate IE. Otherwise, the field is absent. |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

Let's consider a scenario where the network first configures an NZP-CSI-RS resource within LTM-Candidate IE with fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix, and then modifies the same NZP-CSI-RS resource so that these fields are absent. Since there is no need code in condition LTM, the specification does not capture the expected UE behaviour in this scenario (i.e., whether the UE is expected to release or maintain the previously configured values of these fields).

When NZP-CSI-RS-Resource IE is used within LTM-Candidate IE, fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix are optionally present without a need code, which leaves the UE behaviour unspecified in case the fields are absent.

In this scenario, we think the expected UE behaviour is to maintain the previous values.

The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case the fields are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE (TP in Annex B).

## 2.3 Usage of LTM Reference Configuration

The LTM reference configuration (signalled with ltm-ReferenceConfiguration field), if used by the network and supported by the UE, provides a baseline configuration for a group of non-complete LTM candidate configurations. For each LTM candidate with a non-complete LTM candidate configuration, the final configuration which the UE applies upon the LTM cell switch is formed by the UE by applying the LTM candidate configuration (signalled with ltm-CandidateConfig field) on top of the LTM reference configuration using normal RRC delta signalling rules. This is captured in 38.331 clause 5.3.5.18.7 as shown below.

38.331, clause 5.3.5.18.7 1> if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3: 2> consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure; 2> if measConfig is included within ltm-ReferenceConfiguration in ltm-Config; 3> perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig: NOTE 1: When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause. 1> if the LTM cell switch is triggered by an indication from lower layers: 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3; 1> else (LTM cell switch triggered upon cell selection performed while timer T311 was running): 2> apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3; 1> release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete). NOTE 2: When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

Since both LTM reference configuration and LTM candidate configuration are RRCReconfiguration messages and normal RRC delta signalling rules are used when the UE applies the LTM candidate configuration on top of the LTM reference configuration, the 38.331 specification leaves extreme flexibility for the network to partition the UE configuration between the LTM reference configuration and the LTM candidate configuration. Three examples of such extremely flexible partitioning, allowed by the 38.331 specification, are shown below.

Example 1:

- In the LTM reference configuration, the network configures one SCell with sCellToAddModList

- In the LTM candidate configuration, the network also has sCellToAddModList with which the network adds one additional BWP on the same SCell

Example 2:

- In the LTM reference configuration, the network configures physicalCellGroupConfig where two optional 'Need R' fields are present

- In the LTM candidate configuration, the network also configures physicalCellGroupConfig where the first optional 'Need R' field which was present in the LTM reference configuration is absent, and the second optional 'Need R' field which was present in the LTM reference configuration is present with a new value

Example 3:

- In the LTM reference configuration, the network configures masterCellGroup -> spCellConfig -> reconfigurationWithSync, but servingCellConfigCommon is absent

- In the LTM candidate configuration, the network configures masterCellGroup -> spCellConfig -> reconfigurationWithSync -> servingCellConfigCommon

The examples above illustrate such partitionings of the UE configuration between the LTM reference configuration and the LTM candidate configuration which utilize the flexibility to the maximum. Such extremely flexible partitionings may not have a practical need, yet the specification allows such partitionings and the UE which supports LTM reference configuration is required to implement support for all such partitioning possibilities.

The current 38.331 specification allows such partitionings of the UE configuration between the LTM reference configuration and the LTM candidate configuration which may not have practical needs, but which are required to be supported by the UE which supports the LTM reference configuration mechanism.

The way how the network in practice partitions the UE configuration between the LTM reference configuration and the LTM candidate configuration depends on particular network implementation, but may also depend on which parts of the UE configuration are managed by the gNB-CU and which parts are defined by the target gNB-DU (i.e., information transter across the F1 interface for LTM configuration management).

The gNB-CU/gNB-DU split for LTM configuration management and the interaction over F1 interface may set practical limits on how the network may partition the UE configuration between the LTM reference configuration and the LTM candidate configuration.

Given the Observation 3 and the Observation 4, it would make sense to discuss the usage of the LTM reference configuration in more detail, focusing on aspects which are relevant for the UE handling of the LTM reference configuration. Such aspects include the following:

- Whether certain part of the UE configuration can be included in the LTM reference configuration at all.

- Whether certain part of the UE configuration can be included at the same in the LTM reference configuration and in the LTM candidate configuration.

- Whether both LTM reference configuration and LTM candidate configuration can configure the same entities (such as same SCell, same radio bearer, etc.).

Overall target of the discussion should be to identity the partitionings of the UE configuration between the LTM reference configuration and the LTM candidate configuration which are not possible to occur.

Based on above discussion, we propose the following:

RAN2 to discuss usage of the LTM reference configuration using question list in Annex C.

Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present in the LTM reference configuration.

Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration.

Based on the outcome of the discussion (P3), RAN2 to agree on whether the fields which can be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration, if any, can configure the same entities (same radio bearer, same SCell, etc.).

RAN2 to discuss whether the agreements (from P4, P5, and P6), if any, should be captured in 38.331.

## 2.4 RS Alignment between LTM TCI State and CFRA Resource

In RACH-based LTM, the UE may receive an early TCI state activation indication before the cell switch and may not apply the PDCCH-ordered early RACH procedure. In this case, the UE may receive the LTM Cell Switch Command MAC CE with both the TCI state for the target cell and the CFRA information for the random access procedure.

In the current specification, there is no limitation on whether the RS of the indicated (joint) TCI state and the SSB indicated in the CFRA information should be identical or not. Whether or not to allow different configurations for these two RSs in the same LTM Cell Switch Command MAC CE needs further discussion.

If these two indicated RSs are different, the UE needs to track and apply the CFRA SSB for the RACH procedure first. According to the RAN1 specification, after RACH, the UE is expected to switch to the indicated target TCI state for the following UL transmission, rather than using the CFRA SSB.

For RACH-based LTM, the UE uses the SSB indicated in the CFRA information for the RACH procedure. After RACH, the UE is expected to use the indicated target TCI state for the following UL transmission.

If the RS type for the indicated joint TCI state is also SSB, we believe there is no motivation to configure these two RSs differently in the LTM Cell Switch Command MAC CE. Such configuration provides unnecessary flexibility and overcomplicates both network and UE design.

Additionally, this introduces extra workload in RAN4 as the UE may need to track CFRA beams if they are not indicated in the early TCI state activation indication. This may introduce extra latency, and the current RAN4 specifications do not define delay requirements for this misalignment (assuming that the SSB in the indicated TCI state and CFRA resource are always aligned).

The UE may need to track extra CFRA beams if they are different from the indicated joint TCI state and not provided in the early TCI state activation indication. This introduces extra latency and increase the workload on delay requirements for this scenario.

Currently, RAN4 does not support TRS measurement before the LTM cell switch, which means the UE will not track TRS in the early TCI state activation procedure. In that case, if the indicated TCI state is TRS, the network may configure the indicated TRS to have the same QCL source to the SSB that the UE activated in the early TCI indication for RS synchronization. To avoid extra latency and workload as mentioned above, the QCL source of this TCI state should also have the same SSB index as the CFRA resource.

Therefore, we propose to align these two RSs in these two cases to avoid over-design.

Within LTM Cell Switch Command MAC CE, the indicated joint TCI state in the TCI state ID field is the same or QCL to the SSB indicated in the SS/PBCH index field of the contention-free Random Access Resources (TP in Annex D).

## 2.5 Ambiguity of UE capability pdcch-RACH-AffectedBandsList-r18

In the latest TS 38.306, the UE capability pdcch-RACH-AffectedBandsList-r18 is defined as whether the UE may cause an interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission, with the granularity of a feature set. For this UE capability, the UE reports a list with the length of the bands indicated in appliedFreqBandListFilter.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pdcch-RACH-AffectedBandsList-r18 Indicates whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission towards target bands. Each "source-target" pair indicates the band pair between the target band for RACH transmission and band under UE's current band combination. The target bands only consist of the bands indicated in appliedFreqBandListFilter. They are listed in the same order as in appliedFreqBandListFilter and the first entry correspond to the first entry on appliedFreqBandListFilter and so on. A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18. | FS | No | N/A | N/A |

In the latest TS 38.331, the related field has an ENUMERATED with two values {noInterruption, interruption}.

FeatureSetDownlink-v1830 ::=        SEQUENCE {

-- R4 39-4: Interruption on DL slot(s) due to PDCCH- ordered RACH transmission

pdcch-RACH-AffectedBandsList-r18    SEQUENCE (SIZE (1..maxBandsMRDC)) OF ENUMERATED {noInterruption, interruption}        OPTIONAL,

Irrelevant IE has been omitted

}

However, if the UE does not support LTM in some bands indicated in appliedFreqBandListFilter, it is unclear whether the UE should report noInterruption or interruption for those elements. Actually, whether the UE can support PDCCH-ordered RACH transmission in a band is indicated by the pre-requisite UE capability rach-EarlyTA-Measurement-r18, which is signalled per band. But it is still unclear how the UE should report those unsupported bands with ENUMERATED {noInterruption, interruption}.

Therefore, we suggest RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM.

For UE capability pdcch-RACH-AffectedBandsList-r18, RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM. There are two options:
Option 1: For those bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM (which can be further indicated by rach-EarlyTA-Measurement-r18), RAN2 to clarify that it is up to UE implementation to select noInterruption or interruption for that element of pdcch-RACH-AffectedBandsList-r18, and such indication does not represent anything. Add a note to TS 38.306 to capture this clarification.
Option 2: Add a new value notSupported to the ENUMERATED field of pdcch-RACH-AffectedBandsList-r18.

# 3 	Conclusion

Based on the discussion in the previous sections, we propose the following:

Missing early UL sync parameters for TDD LTM candidate

Proposal 1 	The network always signals fields frequencyBandList and absoluteFrequencyPointA when FrequencyInfoUL IE is used within EarlyUL-SyncConfig IE (TP in Annex A).

Unspecified UE behaviour for NZP-CSI-RS-Resource IE

Proposal 2	The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case the fields are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE (TP in Annex B).

Usage of LTM reference configuration

Proposal 3	RAN2 to discuss usage of the LTM reference configuration using question list in Annex C.

Proposal 4	Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present in the LTM reference configuration.

Proposal 5	Based on the outcome of the discussion (P3), RAN2 to agree on which fields can and which fields cannot be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration.

Proposal 6	Based on the outcome of the discussion (P3), RAN2 to agree on whether the fields which can be present at the same time in the LTM reference configuration and in a non-complete LTM candidate configuration, if any, can configure the same entities (same radio bearer, same SCell, etc.).

Proposal 7	RAN2 to discuss whether the agreements (from P4, P5, and P6), if any, should be captured in 38.331.

RS alignment between LTM TCI state and CFRA resource

Proposal 8	Within LTM Cell Switch Command MAC CE, the indicated joint TCI state in the TCI state ID field is the same or QCL to the SSB indicated in the SS/PBCH index field of the contention-free Random Access Resources (TP in Annex D).

Ambiguity of UE capability pdcch-RACH-AffectedBandsList-r18

Proposal 9	For UE capability pdcch-RACH-AffectedBandsList-r18, RAN2 to discuss the issue of UE reporting this UE capability for bands for which the UE does not support PDCCH-ordered RACH for LTM. There are two options:
Option 1: For those bands indicated in appliedFreqBandListFilter where the UE does not support PDCCH-ordered RACH towards target bands for LTM (which can be further indicated by rach-EarlyTA-Measurement-r18), RAN2 to clarify that it is up to UE implementation to select noInterruption or interruption for that element of pdcch-RACH-AffectedBandsList-r18, and such indication does not represent anything. Add a note to TS 38.306 to capture this clarification.
Option 2: Add a new value notSupported to the ENUMERATED field of pdcch-RACH-AffectedBandsList-r18.

# 4 	References

N/A

# Annex A – TP for Proposal 1

3GPP TSG-RAN WG2 #129	R2-25xxxxx

Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | XXXX | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to need code in FrequencyInfoUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ASN.1 does not contain frequency band indicator or absolute frequency of point A for a TDD LTM candidate for early UL sync purposes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected need code for fields frequencyBandList and absoluteFrequencyPointA in FrequencyInfoUL IE. The fields are mandatory present when FrequencyInfoUL is used within EarlyUL-SyncConfig IE, regardless of the duplex mode of the LTM candidate. Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: LTM early UL sync Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE does not have necessary parameters for early UL sync on TDD LTM candidate. 2. If the UE is implemented according to the CR and the network is not, the UE does not have necessary parameters for early UL sync on TDD LTM candidate. This CR is considered mandatory to support the impacted functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is unable to send RA preamble for early UL sync purposes on TDD LTM candidate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL-OrEarlyUL-Sync

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL-OrEarlyUL-Sync

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...,

[[

additionalSpectrumEmission-v1760    AdditionalSpectrumEmission-v1760                        OPTIONAL    -- Need S

]],

[[

additionalSpectrumEmissionAerial-r18   AdditionalSpectrumEmission-r18                       OPTIONAL   -- Need S

]]

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2). Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList.

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If both additionalSpectrumEmission (without suffix) and additionalSpectrumEmission-v1760 are absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], tables 6.2.3.1-1A, 6.2A.3.1.1-2 and 6.2A.3.1.2-2, TS 38.101-2 [39], tables 6.2.3.1-2 and 6.2A.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1A). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured and if signalled, the same vaue in additionalSpectrumEmission-v1760 for all uplink carrier(s) of the same band with UL configured, except for additionalSpectrumEmission value corresponding to NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for these uplink carriers, and NS_01 for the remaining uplink carrier(s) of band n77. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

additionalSpectrumEmissionAerial The additional spectrum emission requirements to be applied by the aerial UE on this uplink (see TS 38.101-1 [15], clause 6.2K). If the field is absent, the aerial UE uses value indicated by the field additionalSpectrumEmission/ additionalSpectrumEmission-v1760.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. Value in dBm. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emission requirements as specified in TS 38.106 [79].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL-OrEarlyUL-Sync | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), if this FrequencyInfoUL is for a supplementary uplink (SUL), or if this FrequencyInfoUL is configured within EarlyUL-SyncConfig. It is absent, Need R, otherwise. |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

# Annex B – TP for Proposal 2

3GPP TSG-RAN WG2 #129	R2-25xxxxx

Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing need code in NZP-CSI-RS-Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE behaviour is not specified in case the network first configures an NZP-CSI-RS resource within LTM-Candidate IE with fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix, and then modifies the same NZP-CSI-RS resource so that these fields are absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing need code (Need M) for the first part of the condition LTM within NZP-CSI-RS-Resource IE. The UE maintains fields subCarrierSpacing, absoluteFrequencyPointA, and cyclicPrefix in case they are absent in NZP-CSI-RS-Resource IE used within LTM-Candidate IE. Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: LTM Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE might release the fields whereas the network expects the UE to maintain them. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour remains unspecified, thus interoperability issues may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### –	NZP-CSI-RS-Resource

The IE NZP-CSI-RS-Resource is used to configure Non-Zero-Power (NZP) CSI-RS transmitted in the cell where the IE is included, which the UE may be configured to measure on (see TS 38.214 [19], clause 5.2.2.3.1). A change of configuration between periodic, semi-persistent or aperiodic for an NZP-CSI-RS-Resource is not supported without a release and add.

NZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-NZP-CSI-RS-RESOURCE-START

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

-- TAG-NZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

NZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214 [19], clause 5.2.2.3.1). Network always configures the UE with a value for this field for periodic and semi-persistent NZP-CSI-RS-Resource (as indicated in CSI-ResourceConfig).

powerControlOffset Power offset of PDSCH EPRE to NZP CSI-RS EPRE. Value in dB (see TS 38.214 [19], clauses 5.2.2.3.1 and 4.1). The UE shall ignore this field in case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE.

powerControlOffsetSS Power offset of NZP CSI-RS EPRE to SS/PBCH block EPRE. Value in dB (see TS 38.214 [19], clause 5.2.2.3.1).

qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or dl-OrJoint-TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1). In case NZP-CSI-RS-Resources is received as part of an LTM-Candidate IE, it refers to the TCI state identifier in CandidateTCI-State and is defined in ltm-DL-OrJointTCI-StateToAddModList within the LTM-Candidate IE.

resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource.

scramblingID Scrambling ID (see TS 38.214 [19], clause 5.2.2.3.1).

| Conditional Presence | Explanation |
| --- | --- |
| LTM | The field is optionally present, Need M, in an LTM-Candidate IE. Otherwise, the field is absent. |
| Periodic | The field is optionally present, Need M, for periodic NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent NZP-CSI-RS-Resources (as indicated in CSI-ResourceConfig). The field is absent otherwise. |

# Annex C – Question list for Proposal 3

Question 1: Can radioBearerConfig be present in ltm-ReferenceConfiguration?

NOTE: If answer to Question 1 is 'No', following questions Q2-Q3 are not applicable.

Question 2: Can radioBearerConfig be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 2 is 'No', following question Q3 is not applicable.

Question 3: Can radioBearerConfig in ltm-ReferenceConfiguration and in non-complete ltm-CandidateConfig and configure the same radio bearer (same srb-Identity/drb-Identity)?

Question 4: Can measConfig to be present in ltm-ReferenceConfiguration?

NOTE: If answer to Question 4 is 'No', following questions Q5-Q6 are not applicable.

Question 5: Can measConfig be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 5 is 'No', following question Q6 is not applicable.

Question 6: Can measConfig in ltm-ReferenceConfiguration and in non-complete ltm-CandidateConfig configure the same measurement object/reporting configuration/measurement identity (same measObjectId/reportConfigId/measId)?

Question 7: Can otherConfig/otherConfig-v1540/otherConfig-v1610/otherConfig-v1700/otherConfig-v1800/otherConfig-v1830 be present in ltm-ReferenceConfiguration?

NOTE: If answer to Question 7 is 'No', following question Q8 is not applicable.

Question 8: Can otherConfig/otherConfig-v1540/otherConfig-v1610/otherConfig-v1700/otherConfig-v1800/otherConfig-v1830 be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

Question 9: Can masterCellGroup/secondaryCellGroup be present in ltm-ReferenceConfiguration?

NOTE: If answer to Question 9 is 'No', following questions Q10-Q22 are not applicable.

Question 10: Can rlc-BearerToAddModList be present in ltm-ReferenceConfiguration?

Question 11: Can mac-CellGroupConfig be present in ltm-ReferenceConfiguration?

Question 12: Can physicalCellGroupConfig be present in ltm-ReferenceConfiguration?

Question 13: Can spCellConfig be present in ltm-ReferenceConfiguration?

Question 14: Can scellToAddModList be present in ltm-ReferenceConfiguration?

Question 15: Can masterCellGroup/secondaryCellGroup be present in both non-complete ltm-CandidateConfig and ltm-ReferenceConfiguration at the same time?

NOTE: If answer to Question 15 is 'No' and/or answer to Question 10 is 'No', following questions Q16-Q17 are not applicable.

Question 16: Can rlc-BearerToAddModList be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 16 is 'No', following question Q17 is not applicable.

Question 17: Can rlc-BearerToAddModList in ltm-ReferenceConfiguration and in non-complete ltm-CandidateConfig configure the same RLC bearer (same logicalChannelIdentity)?

NOTE: If answer to Question 15 is 'No' and/or answer to Question 11 is 'No', following question Q18 is not applicable.

Question 18: Can mac-CellGroupConfig be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 15 is 'No' and/or answer to Question 12 is 'No', following question Q19 is not applicable.

Question 19: Can physicalCellGroupConfig be present in both ltm-ReferenceConfiguration  and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 15 is 'No' and/or answer to Question 13 is 'No', following question Q20 is not applicable.

Question 20: Can spCellConfig be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 15 is 'No' and/or answer to Question 14 is 'No', following questions Q21-Q22 are not applicable.

Question 21: Can scellToAddModList be present in both ltm-ReferenceConfiguration and non-complete ltm-CandidateConfig at the same time?

NOTE: If answer to Question 21 is 'No', following question Q22 is not applicable.

Question 22: Can scellToAddModList in ltm-ReferenceConfiguration and in non-complete ltm-CandidateConfig configure the same SCell (same sCellIndex)?

# Annex D – TP for Proposal 8

3GPP TSG-RAN WG2 #129	R2-25xxxxx

Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | XXXX | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RS alignment between LTM TCI state and CFRA resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-02-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current specification does not limit whether the RS of the indicated (joint) TCI state and the SSB for CFRA information in the LTM cell switch command should be identical. If both RS types are SSB, there is no motivation to configure two SSBs differently in the same LTM cell switch command. If the two indicated SSBs are not the same, the UE needs to track and apply the CFRA SSB for the RACH procedure first, and then switch to the indicated target TCI state for the subsequent UL transmission. If the CFRA SSB is not indicated in the early TCI state activation indication, the tracking on these CFRA beams introduce extra latency and RAN4 workload on delay requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note that if the indicated TCI state provides an SSB index as a QCL source, the SS/PBCH index field of the contention-free Random Access Resources, if present, is the same as the SSB index of the indicated TCI state. Impact Analysis Impacted 5G architecture options: NR SA and NR-DC Impacted functionality: LTM Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, UE may fail to correctly track and apply the CFRA SSB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The tracking of CFRA beams may introduce extra latency and RAN4 workload on delay requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.75 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.3.75	LTM Cell Switch Command MAC CE

The LTM Cell Switch Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields (Figure 6.1.3.75-1):

-	R: Reserved bit, set to 0;

-	Target Configuration ID: This field indicates the index of candidate target configuration to apply for LTM cell switch, corresponding to ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Timing Advance Command: This field indicates whether the TA is valid for the LTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID field). If the value of this field is set to FFF, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell; otherwise, this field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6], and that the UE can skip the Random Access procedure for this LTM cell switch. If tag-Id-ptr is configured for the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM target cell and tag-Id-ptr is set to value n1, this field indicates the TA for the TAG indicated by the tag2-Id of the LTM target cell; otherwise, this field indicates the TA for the TAG indicated by the tag-id of the LTM target cell. The length of the field is 12 bits;

-	TCI state ID: This field indicates and activates the TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The TCI state is identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList as specified in TS 38.331 [5]. If the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is joint, this field is for joint TCI state, otherwise, this field is for downlink TCI state. The length of the field is 7 bits;

-	UL TCI state ID: This field indicates and activates the uplink TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). The UL TCI state is identified by TCI-UL-StateId in ltm-UL-TCI-StateToAddModList as specified in TS 38.331 [5]. The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. The length of the field is 6 bits;

-	C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent.

-	S/U: This field indicates which UL carrier to transmit the PRACH of the contention-free Random Access Resources. If the value of this field is set to 1, SUL is used; otherwise, NUL is used. The length of the field is 1 bit;

-	Random Access Preamble index: This field indicates the Random Access Preamble index of the contention-free Random Access Resources. This field should not be set to 0b000000. The length of the field is 6 bits;

-	SS/PBCH index: This field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission of the contention-free Random Access Resources. The length of the field is 6 bits;

-	PRACH Mask index: This field indicates the RACH occasion(s) associated with the SS/PBCH indicated by 'SS/PBCH index' for the PRACH transmission of the contention-free Random Access Resources. It indicates a subset of RACH occasion(s) from the rach-ConfigDedicated for the UL carrier (indicated by S/U field), (if provided, otherwise it indicates a subset of RACH occasion(s) from the rach-ConfigCommon for the UL carrier (indicated by S/U field) in the UL BWP configuration of firstActiveUplinkBWP-Id as specified in TS 38.331 [5]. When the repetition number field is not set to 0, the UE ignores this field. The length of the field is 4 bits;

-	Repetition number: This field indicates the Msg1 repetition number to be applied to the contention-free Random Access. If this field is set to 0, Msg1 repetition number does not apply. If this field is set to 1, the Msg1 repetition number is 2. If this field is set to 2, the Msg1 repetition number is 4. If this field is set to 3, the Msg1 repetition number is 8. The length of the field is 2 bits.

NOTE 1:	Void

Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

NOTE 2:	If UE receives the LTM Cell Switch Command MAC CE with a Target Configuration ID value not matching any configured ltm-CandidateId minus 1, as specified in TS 38.331 [5], the procedure of handling LTM Cell Switch Command MAC CE in clause 5.18.35 does not apply.

NOTE 3:	When the TCI state ID field provides an SSB index as a QCL source, the SS/PBCH index field of the contention-free Random Access Resources, if present, is the same as the SSB index of the indicated TCI state.