3GPP TSG-RAN WG2 Meeting #129		R2-2500179

Athens, Greece, 17th – 21st Feb. 2025

Source:	China Telecom

Title:	Discussion on asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In last meeting, RAN2 has reached the following agreements.

New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID.

Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB.

In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

In this contribution, we will discuss potential RAN2 specific impacts of asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

  - Pathloss offset update

In the last meeting, It is agreed to introduce a new MAC CE for updating the PL offset for asymmetric DL sTRP/UL mTRP. And based on RAN1 agreements, network can configure the PL offset by RRC. This could potentially lead to an issue where the UE receives both a MAC CE and a RRC Reconfiguration message for updating/reconfiguring the PL offset in a short period of time. At this moment, which configuration should the UE apply? We propose two options for this issue. The first option is time-based method that UE determines and applies the latest update based on the signaling reception time. The second option is validity-based method that UE can decide which offset to apply within the validity period of the update.

Proposal 1: RAN2 to discuss how to address the conflict between MAC CE and RRC when updating PL offset, following options can be considered:

- Option 1: time-based method that UE determines and applies the latest update based on the signaling reception time. Details are FFS.

- Option 2: validity-based method that UE can decide which offset to apply within the validity period of the update. Details are FFS.

  - New MAC CE design

RAN2 has agreed to introduce a new MAC CE for updating the PL offset for asymmetric DL sTRP/UL mTRP and the absolute value range for the PL offset is [-12, 60] dB, with a step size of 4 dB. And the PL offset value can be updated for any configured TCI states with an RRC –configured PL offset. Therefore, the new MAC CE can have a variable size consisting of following fields:

- Serving cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits.

- BWP ID: This field indicates a BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field. The length of the BWP ID field is 2 bits.

- Associated TCI state ID: This field indicates the PL offset updated associated with which TCI state.

- Absolute PL offset value: This field indicates the PL offset. The length of this field is 5 bits.

- Proposal 2: The new MAC CE can have a variable size consisting of following fields:

- Serving cell ID

- BWP ID

- TCI state ID

- Absolute PL offset value

- Conclusion

In this contribution, we discuss the RAN2 aspects on MAC CE impact for asymmetric DL sTRP/UL mTRP scenarios. Based on the discussion, we have the following observations and proposals.

Proposal 1: RAN2 to discuss how to address the conflict between MAC CE and RRC when updating PL offset, following options can be considered:

- Option 1: time-based method that UE determines and applies the latest update based on the signaling reception time. Details are FFS.

- Option 2: validity-based method that UE can decide which offset to apply within the validity period of the update. Details are FFS.

- Proposal 2: The new MAC CE can have a variable size consisting of following fields:

- Serving cell ID

- BWP ID

- TCI state ID

- Absolute PL offset value