3GPP TSG-RAN WG2 Meeting #129	R2-2500192

Athens, Greece, February 2025

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Multi-hop U2N Discovery

For the discovery transmission at the intermediate relay UE, whether Uu and PC5 threshold conditions should be defined is discussed.

For Uu threshold condition, in RAN2 #128, it is agreed that the lower bound of Uu RSRP is not needed and upper bound Uu RSRP can be configured by the network. Two FFS points are left for further discussion.

Agreements: A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound.

The first FFS point on the UE can operate as a last relay UE acts as an intermediate relay UE towards a different cell, it depends on the control plane scenario discussion. The motivation for supporting this case, i.e., a UE instead of acting as the last relay to its serving cell acts as an intermediate relay to another cell, needs to be further clarified. If the serving cell doesn’t support U2N Relay function, no U2N relay operation should be allowed in the cell; If the serving cell supports U2N Relay function, the UE should act as a single-hop U2N Relay/last relay directly to minimise the number of hops.

The motivation of the case that a UE can act as last relay (e.g., Uu RSRP is good) acts as intermediate relay towards other cell should be further clarified.

It is confirmed that the cross-path topology should not be supported for the intermediate relay UE acting as a remote UE:

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

For the IDLE/INACTIVE intermediate relay UE, although it has no RRC connection towards the network, it relies on the parent relay for SIB/paging reception, which is also acting-as-remote-UE behavior. The cross-cell topology introduces 2 paths for SIB/Paging reception, which is not supported in the current NR system.

IDLE/INACTIVE intermediate relay UE also acts as remote UE considering Paging/SIB reception.

Not support the case that a UE can operate as a last relay UE according to the restrictions configured by the network acting as an intermediate relay UE towards a different cell.

For the second FFS point on the details of the upper bound of Uu RSRP, we understand there are the following 3 options:

      - a new/separate parameter for acting as intermediate relay UE.

      - reuse the same upper bound parameter as acting as remote UE.

      - reuse the same lower bound parameter as acting as last relay UE.

For the 3 options, option 2 seems more reasonable and easier since:

-  it doesn’t require a new parameter.

- for L2 multi-hop U2N relay, the intermediate relay must receive paging and SI from the last relay, which means each intermediate relay UE functions simultaneously as a remote UE, making it straightforward to verify the same conditions as the remote UE.

- as shown in the following specification, the Uu threshold condition is used together with a hysteresis parameter, if reusing the lower bound of the relay UE, the redefinition of parameter is needed.

#### 5.8.14.2 NR sidelink U2N Relay UE threshold conditions A UE capable of NR sidelink U2N Relay UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRelay by hystMaxRelay if configured; and 2> if threshLowRelay is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshLowRelay by hystMinRelay if configured: 3> consider the threshold conditions to be met (entry); 1> else: 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRelay if configured; or 2> if the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshLowRelay if configured; 3> consider the threshold conditions not to be met (leave); … #### 5.8.15.2 NR Sidelink U2N Remote UE threshold conditions A UE capable of NR sidelink U2N Remote UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or 2> if the UE has no serving cell: 3> consider the threshold conditions to be met (entry);

Reusing the Uu RSRP upper bound of remote UE for the intermediate relay UE can: 1) save the effort for a new parameter; 2) be in line with the fact that intermediate relay UE for L2 case also acts as remote UE; 3) save the redefinition of the lower bound parameters for acting as U2N Relay UE.

For the upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, reuse the same upper bound of Uu RSRP parameter for acting as remote UE.

The next question is how for the UE to operate if the parameter is not configured. In R17 U2N Relay, if the related parameter is not configured, remote UE considers the condition to be met.

#### 5.8.15.2 NR Sidelink U2N Remote UE threshold conditions A UE capable of NR sidelink U2N Remote UE operation shall: 1> if the threshold conditions specified in this clause were previously not met: 2> if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or 2> if the UE has no serving cell: 3> consider the threshold conditions to be met (entry);

In R17, if threshHighRemote is not configured, the remote UE considers the threshold conditions to be met.

By following R17 principle and the agreement in the last RAN2 meeting, the UE capable of U2N intermediate Relay UE operation and cannot act as last relay UE/ U2N Relay UE considers the threshold condition to be met if the Uu RSRP upper bound is not configured in the serving cell.

If upper bound of Uu RSRP is not configured by the network, the UE acting as U2N intermediate Relay UE operation considers the threshold condition to be met.

Then for the PC5 threshold condition, for the PC5 link quality with the last relay, for Model-A discovery, as replied by SA2, the link between the intermediate relay and last relay should be established before discovery forwarding by the intermediate relay UE.

Regarding the following question about Model A discovery from RAN2, SA2 would like to provide the answer as below. - Question from RAN2: Whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message. - Answer from SA2: Yes.

For Model-A discovery, SA2 confirms that the intermediate relay UE needs to establish PC5 link with the relay from which the discovery announcement message is received before forwarding the discovery announcement message.

Therefore, the PC5 between the intermediate relay UE and the last relay UE is stable and the RSRP threshold condition is not required for discovery forwarding.

No PC5 RSRP threshold condition is needed for the Model-A discovery forwarding at the intermediate relay UE.

For Model-B discovery, it is discussed in SA2 that the PC5 link establishment is not required. However, they are more considering the L3 case, the situation is different for L2 multi-hop U2N Relay, as discussed in observation 2, the intermediate relay UE has 2 roles at the same time:

- To the downstream link, it acts as an intermediate relay UE that provides relay service to the child UE(s);

- To the upstream link, it acts as a remote UE that relies on the parent UE (s) to access the network for paging, SI, or data forwarding.

For L2 Multi-hop U2N Relay, the intermediate relay UE also acts as a remote for SI/Paging/data reception/transmission.

Therefore, the intermediate relay which is also acting as a remote UE evaluates the link quality towards the parent node and decides whether to select/connect to the parent UE.

The intermediate relay UE which also acts as remote UE at the same time evaluates the link quality towards the parent node via relay (re)selection procedure.

Since the link quality is evaluated by the intermediate relay UE via relay (re)selection criteria, no additional discovery transmission condition is needed. If the link quality is bad, the intermediate relay UE should trigger relay reselection procedure.

In addition, the discovery type (e.g., whether it is Model-A or Model-B) does not matter from AS layer perspective, they are both carried via SL-SRB4. So it is preferred to have unified approach for Model-A and Model-B as in R17 single-hop U2N Relay.

In R17 single-hop U2N Relay, there is no different handling of Model-A and Model-B discovery, i.e., both carried via SL-SRB4.

RAN2 to discuss for Model-B discovery, no PC5 RSRP threshold condition is needed for the discovery response message forwarding at the intermediate relay UE .

For the PC5 link between the remote UE and the intermediate relay UE, it can be evaluated by the remote UE based on relay (re)selection criteria.

The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Therefore, there is no need for a double evaluation of the PC5 link quality at the intermediate relay upon Model-B solicitation message reception.

For Model-B Solicitation message forwarding by the intermediate relay, no PC5 threshold configuration is needed.

There is also a discussion about whether the last relay needs to check the SD-RSRP of the received Model-B solicitation message to decide whether to respond. As discussed in Proposal 2, the link quality between the intermediate relay and the last relay is evaluated by the intermediate relay via relay (re)selection, i.e., if the link quality between the intermediate relay and the last relay is bad, the intermediate relay should perform relay reselection to another last relay. Therefore, as in single-hop U2N Relay, there is no PC5 condition to be checked at the last relay UE, i.e., the PC5 threshold configuration is not needed for Model-B response at the last relay UE.

For Model-B Response transmission by the last relay, no PC5 threshold configuration is needed.

Relay (re)selection

For the relay selection and reselection trigger condition, the following trigger condition is defined in R17 single-hop U2N relay and R18 U2U relay case:

The U2N Remote UE triggers U2N Relay selection in following cases: - Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold; - Indicated by upper layer of the U2N Remote UE. The U2N Remote UE may trigger U2N Relay reselection in following cases: - PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold; - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling; - When U2N Remote UE receives a PC5-S link release message from U2N Relay UE; - When U2N Remote UE detects PC5 RLF; - Indicated by upper layer. The U2U Remote UE may trigger U2U Relay selection in the following cases: - When the SL-RSRP or SD-RSRP between U2U Remote UEs is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication to trigger U2U relay selection from the upper layer of the UE. The U2U Remote UE may trigger U2U Relay reselection in the following cases: - When the SL-RSRP or SD-RSRP of the current U2U Relay UE is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication from the upper layer due to detecting PC5 RLF; - When L2 U2U Remote UE receives an indication from the upper layer due to receiving the PC5 RLF indication from the L2 U2U Relay UE; - When U2U Remote UE receives a PC5-S link release message from U2U Relay UE; - When U2U Remote UE receives an indication to trigger U2U relay reselection from the upper layer of the UE.

For multi-hop relay selection trigger condition, it should be the same as single-hop relay, i.e., indicated by the upper layer and direct link quality is below a configured threshold.

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For Relay reselection, in R17 single-hop U2N Relay and R18 U2U Relay, the trigger condition can be summarized into four aspects:

- Upper layer triggered.

- Link quality with the current relay below a configured threshold.

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release.

- The link between the relay UE and other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger conditions:

- Upper layer triggered.

- Link quality with the current relay below a configured threshold.

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release.

- The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In the R19 multi-hop U2N Relay, we can adopt a similar approach by considering relay reselection triggers from the above aspects. For triggered by the upper-layer, degradation in link quality with the currently connected relay, or if the link failure between the remote UE and the directly connected relay (due to Radio Link Failure or PC5-S release). These three aspects align with the conditions for single-hop U2N Relay and can also apply to multi-hop U2N Relay.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

- PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold.

- When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE.

- When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE.

- Indicated by upper layer.

To address issues related to the other link, for the Uu link between the relay UE and the network. In situations such as cell reselection, handover, Uu Radio Link Failure (RLF), or failures in Uu RRC connection establishment or resumption, it is reasonable for the remote UE to initiate relay reselection, as group handover is not supported.

Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

In the case of the PC5 link between two multi-hop U2N Relay UEs, similar to the Uu link, if the PC5 radio link between the two multi-hop U2N Relay UEs fails, the relay UE that is closer to the remote UE should inform the remote UE of the broken link. This way, the remote UE will be aware that the multi-hop relay link has been disrupted.

The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link breaks and perform relay reselection.

The unclear part is how for the multi-hop U2N Relay UE to indicate such information to the remote UE given that legacy Notification message is a per-hop PC5 RRC message. This depends on the control plane design which can be further discussed by RAN2.

How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

- cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network.

- PC5 RLF/release between the multi-hop U2N Relay UE(s).

For the relay (re)selection at the intermediate relay UE, there is a discussion on whether the relay (re)selection condition is required.

Firstly, as discussed in observation 2 and 6, the intermediate relay UE also acts as remote UE considering it needs to rely on the last relay for Paging/SIB monitoring in IDLE/INACTIVE case, and for its own data forwarding in CONNECTED case. Which means the intermediate relay acts as remote UE needs to perform relay reselection, which is already concluded by SA2 in TS 23.304.

##### 6.4.3.8.3 Relay reselection and mobility support […] The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

The intermediate relay UE also acts as remote UE for Paging/SIB and data forwarding, it performs relay reselection same as remote UE.

Then, in case the link is broken, i.e., not workable, there is no need to keep the link between the intermediate relay UE and the serving relay, i.e., the intermediate relay can decide to release/switch to another link.

There is no need to keep the link between the intermediate relay UE and its serving relay if the link is broken (RLF, link release, RRC failure), the intermediate relay can decide to release/switch to another link in such cases.

Therefore, it is needed for the intermediate relay UE to perform relay reselection, the same trigger condition of relay (re)selection defined for the remote UE can be used. Additionally, once relay (re)selection has occurred, the PC5 link between the intermediate relay UE and the source last relay is released. Consequently, it is necessary for the intermediate relay to send a notification message to the remote UE.

RAN2 to discuss intermediate relay UE performs relay (re)selection with the same trigger condition defined for the remote UE. Upon relay reselection, it sends notification to the remote UE to indicate PC5 link release with its parent relay.

Conclusion

We have the following observations:

Observation 1	The motivation of the case that a UE can act as last relay (e.g., Uu RSRP is good) acts as intermediate relay towards other cell should be further clarified.

Observation 2	IDLE/INACTIVE intermediate relay UE also acts as remote UE considering Paging/SIB reception.

Observation 3	Reusing the Uu RSRP upper bound of remote UE for the intermediate relay UE can: 1) save the effort for a new parameter; 2) be in line with the fact that intermediate relay UE for L2 case also acts as remote UE; 3) save the redefinition of the lower bound parameters for acting as U2N Relay UE.

Observation 4	In R17, if threshHighRemote is not configured, the remote UE considers the threshold conditions to be met.

Observation 5	For Model-A discovery, SA2 confirms that the intermediate relay UE needs to establish PC5 link with the relay from which the discovery announcement message is received before forwarding the discovery announcement message.

Observation 6	For L2 Multi-hop U2N Relay, the intermediate relay UE also acts as a remote for SI/Paging/data reception/transmission.

Observation 7	The intermediate relay UE which also acts as remote UE at the same time evaluates the link quality towards the parent node via relay (re)selection procedure.

Observation 8	In R17 single-hop U2N Relay, there is no different handling of Model-A and Model-B discovery, i.e., both carried via SL-SRB4.

Observation 9	The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Observation 10	In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger conditions:

-	Upper layer triggered.

-	Link quality with the current relay below a configured threshold.

-	The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release.

-	The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

Observation 11	Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

Observation 12	The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link breaks and perform relay reselection.

Observation 13	How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Observation 14	The intermediate relay UE also acts as remote UE for Paging/SIB and data forwarding, it performs relay reselection same as remote UE.

Observation 15	There is no need to keep the link between the intermediate relay UE and its serving relay if the link is broken (RLF, link release, RRC failure), the intermediate relay can decide to release/switch to another link in such cases.

We have the following proposals:

Proposal 1	Not support the case that a UE can operate as a last relay UE according to the restrictions configured by the network acting as an intermediate relay UE towards a different cell.

Proposal 2	For the upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, reuse the same upper bound of Uu RSRP parameter for acting as remote UE.

Proposal 3	If upper bound of Uu RSRP is not configured by the network, the UE acting as U2N intermediate Relay UE operation considers the threshold condition to be met.

Proposal 4	No PC5 RSRP threshold condition is needed for the Model-A discovery forwarding at the intermediate relay UE.

Proposal 5	RAN2 to discuss for Model-B discovery, no PC5 RSRP threshold condition is needed for the discovery response message forwarding at the intermediate relay UE .

Proposal 6	For Model-B Solicitation message forwarding by the intermediate relay, no PC5 threshold configuration is needed.

Proposal 7	For Model-B Response transmission by the last relay, no PC5 threshold configuration is needed.

Proposal 8	For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

Proposal 9	For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE.

-	When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE.

-	Indicated by upper layer.

Proposal 10	Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network.

-	PC5 RLF/release between the multi-hop U2N Relay UE(s).

Proposal 11	RAN2 to discuss intermediate relay UE performs relay (re)selection with the same trigger condition defined for the remote UE. Upon relay reselection, it sends notification to the remote UE to indicate PC5 link release with its parent relay.