3GPP TSG-RAN WG2 #129	R2-2500195

Athens, Greece, February 2025

Agenda Item:	8.7.5

Source:	OPPO

Title:	Discussion on RLC re-transmission related enhancements

Document for:	Discussion, Agreement

Introduction

For the RLC retransmission related enhancements, RAN2 has made some down selection of the enhancement options for both timely RLC retransmission and avoiding unnecessary retransmission. This paper will further discuss details for each aspect.

Discussion

Unnecessary retransmissions

To avoid unnecessary retransmissions, the following agreements are made:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In the agreed combined approach, the Tx side and Rx side stops transmission and reception independently. At the Rx side, it is agreed that the abandonment of SDU is based on a local timer. However, it remains unclear how the Tx side decides the retransmission of SDU is unnecessary. According to the current specification, e SDU discarding at the RLC Tx side is based on the indication from the PDCP entity. PDCP decides whether a packet is outdated based on discardTimer or discardTimerForLowImportance. It is straightforward to follow the current principle that the SDU discard at Tx RLC is relies on the indication from PDCP layer.

TS 38.322 ## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU. TS 38.323 - 5.3 SDU discard When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall: - if pdu-SetDiscard is configured: - discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs; NOTE 1: PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set. - else: - discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

RAN2 confirm SDU discard at TX side is based on the indication from PDCP as the current specification.

For the indication from Tx to Rx side, RAN2 has the following agreements in RAN2 #128 meeting:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

Based on the agreements, there are 2 unclear aspects to be further clarified:

- What specific problem can the Tx to Rx indication resolve that the local timer cannot?

- What will the indication look like?

Regarding the problems that cannot be addressed by the local timer, the main issues arise from the unsynchronized discard decisions made by the Tx and Rx sides:

- If Tx side decides not to transmit the SDUs, while the Rx side continues to wait, there will be: 1) Window stalling caused by the Rx_timer delay. The Tx side may experience stalling if all remaining packets in the Tx window are discarded (e.g., in the PDU set case) but the Rx_timer is still running. 2) The handling of NACK feedback for the discarded SDU on the Tx side needs to be discussed.

- If Tx side intends to transmit the SDUs while the Rx side decides to abandon, there will be:1) data loss due to the premature discarding at the Rx side; 2) misalignment between HARQ and ARQ procedure, i.e., MAC feedback NACK so the DL HARQ re-transmission is still being scheduled while RLC feedback ACK, which is confusing to the network.

Based on the Rx-timer discarding at the Rx side, if Tx side decides SDU discarding first, there may be: 1) window stalling at the Tx side during Rx_timer running; 2) additional specification impact at the Tx RLC of NACK feedback to the discarded SDUs.

Based on the Rx-timer discarding at the Rx side, if Rx side decides SDU discarding first, there may be: 1) data loss; 2) misalignment between MAC HARQ and RLC ARQ, i.e., HARQ NACK and ARQ ACK for the same packet.

To address the issues discussed in Observation 1/2, a simple Tx to Rx side indication can be sent via data PDU with no Data field.

- If Tx decides SDUs discarding first, it just sends discarded data PDUs with only PDU header, which has no impacts to the current Rx behaviour and can solve the Tx window stalling/NACK handling issue.

- The data PDU with no Data field can also prevent the premature data discarding at the Rx side. By sending this PDU, the Rx timer can be restarted, helping to avoid unnecessary retransmission discards..

A simple Tx to Rx indication via a data PDU without a Data field adds no complexity to the current procedure and effectively addresses all issues discussed in Observations 1 and 2.

RAN2 to discuss use data PDU with no Data field, i.e., only header with SN number to indicate the discard information from Tx to Rx side.

Timely RLC retransmission(s)

For timely retransmissions, it is agreed that both autonomous retransmission and polling enhancement can be supported based on network configuration.

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

The further details of both polling based enhancement and autonomous retransmission needs to be discussed.

For polling enhancement, the current polling from UE for the UL traffic is triggered based on the following conditions:

-	If PDU_WITHOUT_POLL >= pollPDU;

-	If BYTE_WITHOUT_POLL >= pollByte:

-	Upon expiry of t-PollRetransmit

So timely polling can be achieved by the following options

Option-1: Introducing new polling trigger condition besides the above ones, e.g., the UE can trigger polling if the delay budget of the packet is below a threshold.

Option-2: Reuse the above existing polling trigger conditions but have smaller value configuration for small remaining delay budget data, i.e., UE can use the configuration with smaller value if there is small remaining delay budget data.

For the polling enhancement in RLC AM, RAN2 discuss the following two options:

- Option 1: Define new polling trigger condition: UE triggers polling if the remaining delay budget of the packet is below a threshold.

- Option 2: Based on legacy spec, rely on smaller value for small remaining delay budget data.

For blind retransmission, as indicated in the WID, the objective is for the XR data with a small delay budget, which means the delay budget is the key point, so the trigger of retransmission without feedback should be based on the remaining delay budget.

Currently, the RLC AM retransmission without status report can be performed if t-PollRetransmit expires.

- 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

The RLC SDU can be retransmitted without feedback if t-PollRetransmit expires.

With proper t-PollRetransmit configuration, for example, network to configure the t-PollRetransmit value by consideration of the delay budget of the RLC channel, the timely retransmission can be achieved. So there are two options for autonomous retransmission trigger as well:

Option-1: Relying on the legacy t-PollRetransmit expiry triggered retransmission with proper t-PollRetransmit configuration (e.g., based on delay budget);

Option-2: Define new trigger condition based on configured remaining delay threshold.

For autonomous retransmission without feedback, RAN2 to discuss the following trigger condition options:

- Option-1: Rely on the legacy t-PollRetransmit expiry to trigger retransmission, with proper t-PollRetransmit configuration based on the delay budget.

- Option-2: define new trigger condition based on the remaining delay budget, e.g., based on a configured remaining delay threshold.

The main concern for blind retransmission is about the impact on system capacity especially considering a congestion system. One possible way to mitigate the capacity issue is to rely on the network to judge whether/how many blind retransmissions should be performed since the network has a better knowledge of the overall radio resource status.

Network has a better knowledge of the overall radio resource status.

If the RLC retransmission without feedback is supported, it is suggested that RAN2 to discuss to rely on the network to enable and control whether/how the retransmission without feedback is performed.

For autonomous retransmission without feedback, if new trigger condition is introduced, RAN2 to discuss to rely on network to enable/disable UL retransmission without feedback and to configure the maximum blind retx number.

At last, as agreed by RAN2 in last meeting, it is possible to configure both Polling enhancement and autonomous retransmission at the same time, how to understand this case relates to the following two options been discussed in RAN2.

Compromise solutions: - If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet. - Have both autonomous and polling enhancement.

The main difference between option-1 and option-2 is whether there is a coupling between polling enhancement and autonomous retransmission, i.e., in option-1, the autonomous should be the last call after polling enhancement cannot ensure the delay budget. This motivation can be achieved by Option 2 and network implementation for proper configuration to trigger autonomous retransmission after polling.

When both Polling enhancement and autonomous retransmission are configured, network implementation on proper configuration can achieve autonomous retransmission been triggered after polling.

If both Polling enhancement and autonomous retransmission with new trigger condition are configured, they operate independently at the UE side.

Conclusion

We have the following observations:

Observation 1	Based on the Rx-timer discarding at the Rx side, if Tx side decides SDU discarding first, there may be: 1) window stalling at the Tx side during Rx_timer running; 2) additional specification impact at the Tx RLC of NACK feedback to the discarded SDUs.

Observation 2	Based on the Rx-timer discarding at the Rx side, if Rx side decides SDU discarding first, there may be: 1) data loss; 2) misalignment between MAC HARQ and RLC ARQ, i.e., HARQ NACK and ARQ ACK for the same packet.

Observation 3	A simple Tx to Rx indication via a data PDU without a Data field adds no complexity to the current procedure and effectively addresses all issues discussed in Observations 1 and 2.

Observation 4	The RLC SDU can be retransmitted without feedback if t-PollRetransmit expires.

Observation 5	Network has a better knowledge of the overall radio resource status.

Observation 6	When both Polling enhancement and autonomous retransmission are configured, network implementation on proper configuration can achieve autonomous retransmission been triggered after polling.

We have the following proposals:

Proposal 1	RAN2 confirm SDU discard at TX side is based on the indication from PDCP as the current specification.

Proposal 2	RAN2 to discuss use data PDU with no Data field, i.e., only header with SN number to indicate the discard information from Tx to Rx side.

Proposal 3	For the polling enhancement in RLC AM, RAN2 discuss the following two options:

Option 1: Define new polling trigger condition: UE triggers polling if the remaining delay budget of the packet is below a threshold.

Option 2: Based on legacy spec, rely on smaller value for small remaining delay budget data.

Proposal 4	For autonomous retransmission without feedback, RAN2 to discuss the following trigger condition options:

Option-1: Rely on the legacy t-PollRetransmit expiry to trigger retransmission, with proper t-PollRetransmit configuration based on the delay budget.

Option-2: define new trigger condition based on the remaining delay budget, e.g., based on a configured remaining delay threshold.

Proposal 5	For autonomous retransmission without feedback, if new trigger condition is introduced, RAN2 to discuss to rely on network to enable/disable UL retransmission without feedback and to configure the maximum blind retx number.

Proposal 6	If both Polling enhancement and autonomous retransmission with new trigger condition are configured, they operate independently at the UE side.