3GPP TSG-RAN WG2 Meeting #129	R2-2500198

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item:	8.7.4.2

Source: 	OPPO

Title:  	Discussion on DSR enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on DSR enhancements for scheduling.

# Discussion on DSR enhancements

In RAN2#127bis and RAN2#128 meetings, the following agreements were made. Here we further discuss the leftover issues.

RAN2#127bis Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

RAN2#128 Agreements on DSR enhancements - Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

For the new DSR MAC CE design, it is agreed that a one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. It is still FFS on the choice between “a certain/further pair”.

For one bit indicating a certain pair, it is actually a bitmap on presence of each pair for each LCG. It is hard to determine a fixed length of the bitmap at this moment, since the number of pairs depends on the maximum number of reporting thresholds allowed to configure for each LCG.

Therefore, we tend to the later way, i.e., one bit indicating further pair. It would be flexible on the number of pairs, since this one-bit indication is piggyback within each pair. For the format of new DSR MAC CE, as shown in the figure below, the format of legacy DSR MAC CE could be taken as baseline. And R bit in the legacy DSR MAC CE could be used for this one-bit indication, so no additional signalling overhead is introduced compared to the way of bitmap. To differentiate the pairs from each LCG, in the last pair of each LCG it always needs to indicate “no further pair existing for this LCG” as an end maker.

- RAN2 confirms that a one-bit indication may indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

Another issue is the co-existence with legacy DSR MAC CE. We understand that for it is not necessary for network to configure both old DSR and new DSR at the same time. If networks configure them at the same time, considering the new DSR MAC CE is also compatible to legacy DSR MAC CE, it is straightforward that in case there is at least one LCG configured with new DSR, UE always uses a new DSR MAC CE for all LCGs.

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

Although in RAN2#127bis meeting we have agreed that the legacy definition of delay-critical data is not changed, according to the RAN2#128 agreement above, there is no constraint on how NW to configure the triggering and reporting thresholds for Rel-19 DSR, which allows both delay-critical data and non-delay-critical data to be captured in Rel-19 DSR. Besides, according to the RAN2#128 agreement above, UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for Rel-19 DSR enhancement.

- For Rel-19 DSR, both delay-critical data and non-delay critical data could be captured in Rel-19 DSR MAC CE, due to 1) the flexible configuration of triggering and reporting thresholds, and 2) non-delay critical data ahead of delay critical data in the buffer.

In legacy, the DSR is used to report the information of the delay-critical data whose remaining time is smaller than the triggering threshold. Thus, the current specs (e.g. MAC spec) use the concept of the delay-critical data to describe data volume calculation for DSR triggering and the content to be report for a DSR. However, Rel-19 DSR, considering the situation observed above, solo usage of the concept of the delay-critical data is insufficient to reflect both delay critical data and non-delay critical data, and RAN2 needs to consider how to represent the data volume for a DSR reporting to include both critical and non-critical data. For instance, we don’t need to mention the concept of delay-critical data in the MAC spec, instead, when we describe the Buffer Size field for Rel-19 DSR MAC CE, we can say this field indicates the total amount of UL data for one portion of s specific LCG (no matter the corresponding data is the critical one or not). Alternately, if the concept of delay-critical data is still needed in MAC spec, except for delay-critical data, the concept of non-delay-critical data is also needed for MAC spec.

- RAN2 discusses whether the concept of delay-critical data is still used to represent the data volume for a Rel-19 DSR reporting, including the following options:
1) no need to mention the concept of delay-critical data in the MAC spec,
2) except for delay-critical data, the concept of non-delay-critical data is also needed for MAC spec.

Another issue needed to be started to discuss is the capability for DSR enhancement. The DSR enhancement introduced in Rel-19 XR should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- RAN2 confirms that a one-bit indication may indicate whether further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

- UE always uses the new DSR MAC CE for all LCGs in case there is at least one LCG configured with new DSR.

- For Rel-19 DSR, both delay-critical data and non-delay critical data could be captured in Rel-19 DSR MAC CE, due to 1) the flexible configuration of triggering and reporting thresholds, and 2) non-delay critical data ahead of delay critical data in the buffer.

- RAN2 discusses whether the concept of delay-critical data is still used to represent the data volume for a Rel-19 DSR reporting, including the following options:
1) no need to mention the concept of delay-critical data in the MAC spec,
2) except for delay-critical data, the concept of non-delay-critical data is also needed for MAC spec.

- Introduce a new capability on whether the UE supports DSR enhancement for Rel-19 XR, which is an optional capability with signalling.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia