3GPP TSG-RAN WG2 Meeting #129	R2-2500206

Athens, GR, Feb. 17th – 21st, 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	Discussion on RAN2 impacts of IoT-NTN TDD

Document for:	Discussion and Decision

- Introduction

In RAN#105, a new work item was approved to introduce NB-IoT TDD mode exclusively for 1616-1626.5 MHz Mobile Satellite Services MSS band [1]. During the RAN1#119 meeting, some agreements were made for the TDD pattern and the corresponding frame structure [2], as listed below.

Agreement： Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

The WID was revised in RAN#106 [3] to reflect RAN1 progress and narrow down the scope of the solutions.

In this paper, we discuss the RAN2 impacts of this new WID in general.

- Discussion

  - Impact on Access Control

In the WID [3], it was determined that the NB-IoT TDD mode for NTN will operate exclusively in 1616-1626.5 MHz MSS band, where TN deployment is not expected.

- 4.1 Objective of SI or Core part WI or Testing part WI The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

Due to this frequency band being newly allocated and not  part of  the existing operating frequency bands, the pre-R19 UEs cannot  access network in this frequency band. So there is no need to enhance the Access Control mechanism to prevent the legacy UEs from camping on.

Observation 1: Pre-R19 UEs cannot access the network in 1616-1626.5 MHz MSS allocated band so there is no need to enhance the Access Control mechanism for legacy UEs.

For R19 UEs which support NB-IoT NTN TDD mode, they will search for the 1616-1626.5 MHz MSS band if needed. Alternatively, the R19 UEs can choose to access the TN network. If the R19 UE detects a frequency within this band before detecting other frequencies during cell search, it may try to camp on the NTN eNB using NB-IoT TDD mode. In this case, it is up to NW configuration whether to allow the UEs to access using the legacy access control indication cellBarred and there is no need to broadcast cellBarred-NTN in an NB-IoT TDD NTN cell.

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

  - Impact on Random Access

Upon completion of the cell search procedure, the UE achieves DL synchronization with the cell. After camping on the cell, the UE must then obtain UL synchronization and be allocated UL resources before it can initiate UL transmissions. UL synchronization is accomplished through a Random Access procedure, as shown in Figure 1.

Figure 1. Random Access procedure initialization

The implementation of the Random Access procedure initialization depends on the frame structure. Alterations of the frame structure can disrupt the predefined UL and DL message transmission timings in TDD. For instance, from the UE’s perspective, Msg1/Msg3, which are supposed to be transmitted in a UL slot, may not be sent if the current slot is a DL slot. Similarly, Msg2/Msg4, which are intended to be received in a DL slot, may not be received if the current slot is a UL slot.

Hence, it is necessary to further analyse the timings of the messages exchanged during the Random Access procedure to determine whether the configuration of Random Access should be adjusted to adapt to the new frame structure.

- In Msg1, the UE sends an RA preamble. The UE needs to obtain the resource configuration for sending the RA preamble based on the PRACH configuration from the SIB1 message, which should be determined by RAN1.

- In Msg 2, the eNB sends an RA response. The RA Response window starts at the subframe that contains the end of the last preamble repetition, plus 3 subframes, plus UE-eNB RTT and has length defined by ra-ResponseWindowSize for the corresponding coverage enhancement level.

- In Msg 3 the UE sends RRCSetupRequest.

- In Msg 4 the eNB sends a RRCSetup message to the UE.

UE receives MSG2 in RAR window and MSG4 in Contention Resolution window. The sizes of these two windows are configured by NW.

ra-ResponseWindowSize Duration of the RA response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods and so on. For FDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: ra-ResponseWindowSize = Min (signaled value x PDCCH period, 20.48s).

mac-ContentionResolutionTimer Timer for contention resolution in TS 36.321 [6]. Value in PDCCH periods. Value pp1 corresponds to 1 PDCCH period, pp2 corresponds to 2 PDCCH periods and so on. mac-ContentionResolutionTimer-r15 is only applicable for EDT. UE performing EDT shall use mac-ContentionResolutionTimer-r15, if present. For FDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 10.24s). For TDD: The value considered by the UE is: mac-ContentionResolutionTimer = Min (signaled value x PDCCH period, 20.48s).

Considering the link budget in NB IoT NTN, normally the PDCCH period is long because of the large repetitions to compensate the link budget. And proper NW implementation can guarantee that the Downlink-to-Uplink Guard Period is shorter than the duration of the RAR window and the CR timer. In this case, there is no issue reusing the current design of RAR window and the CR timer. And UE can receive the corresponding response in the next DL slot even when the DL slot is not available in the current cycle

Based on the above reason, we propose

Proposal 2: The legacy design of RAR window and the CR timer can be reused in NB-IoT NTN TDD system with appropriate values configured.

  - Impact on Paging

Currently RAN1 is studying the offset between the 90ms TDD structure and the 10240ms H-SFN and the following options are on the table.

Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

This may have some impact on paging, e.g., PO calculation. But since the final conclusion hasn’t been made in RAN1, we can just wait for their conclusion.

Proposal 3: Discussion on paging can be wait for RAN1’s conclusion on the offset between the 90ms TDD structure and the 10240ms H-SFN.

  - UE Behavior during the Guard Period

In the new frame structure, the UE is not allowed to engage in any uplink or downlink scheduling with the NTN eNB during the guard period. The UE behavior during this period should be clear. For the sake of power saving, it is beneficial for the UE to assume that there is current no coverage. Therefore, the corresponding UE behaviour is similar to the UE's behavior during discontinuous coverage, as highlighted below.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

Proposal 4: UE behavior during the guard period should follow the behavior observed during discontinuous coverage.

- Conclusion

The observations and proposals made in this contribution are summarized below:

Impact on Access Control

Observation 1: Pre-R19 UEs cannot access the network in 1616-1626.5 MHz MSS allocated band so there is no need to enhance the Access Control mechanism for legacy UEs.

Proposal 1: The IE cellBarred in the SIB1-NB message can be reused to control whether a UE supporting NB-IoT NTN TDD mode is allowed to access an NB-IoT TDD NTN cell.

Impact on Random Access

Proposal 2: The legacy design of RAR window and the CR timer can be reused in NB-IoT NTN TDD system with appropriate values configured.

Impact on Paging

Proposal 3: Discussion on paging can be wait for RAN1’s conclusion on the offset between the 90ms TDD structure and the 10240ms H-SFN.

UE Behavior during the Guard Period

Proposal 4: UE behavior during the guard period should follow the behavior observed during discontinuous coverage.

- References

RP-242415, New WID on introduction of IoT-NTN TDD mode

Chair notes RAN1#119 eom1

RP‑243293, Revised WID on introduction of IoT-NTN TDD mode