3GPP TSG-RAN WG2 Meeting #129	R2-2500207

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.2

Source: 	OPPO

Title:  	Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to provide our considerations on open issues for inter-CU LTM.

Discussion

LTM configuration handling

As agreed in previous RAN2 meeting, how to handle the LTM configuration after inter-CU LTM cell switch was discussed and the following agreement was achieved.

Handling of candidate configuration after inter-CU LTM cell switch: - As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

In this section, we would like to further discuss the R19 LTM configuration handling in other cases, e.g., upon RRC state transition, MR-DC release etc. R18 LTM configuration handlings are summarized as in the table below, which can be taken as the baseline for R19 discussions.

| Cases | R18 MCG LTM configuration | R18 SCG LTM configuration |
| --- | --- | --- |
| State transition: 1/ Reception of RRCRelease. 2/ Going to IDLE | Autonomous release | Autonomous release |
| MR-DC release | Kept | Autonomous release |
| Re-establishment: 1/ LTM-based recovery is not configured. 2/ LTM-based recovery is configured but the selected cell is not LTM candidate cell is not selected during T311. 3/ LTM-based recovery fails. | Autonomous release | Autonomous release |
| PCell change or PSCell change/addition | Up to explicit NW indication | Up to explicit NW indication |
| MCG/SCG failure | Up to explicit NW indication | Up to explicit NW indication |

For the listed cases, our considerations on R19 LTM configuration handling are as follows:

- For reception of RRCRelease/going to IDLE, we understand the same behaviour can be simply followed for R19 LTM configuration, i.e., both inter-CU MCG LTM configurations and inter-CU SCG LTM configurations are autonomously released.

- For MR-DC release, the inter-CU SCG LTM configuration also requires to be released due to the detachment from the NW node (i.e., SN) to trigger the SCG LTM procedure. Compared to NW-triggered release, UE autonomous release is preferred as in R18 for signalling efficiency.

- For RRC re-establishment, it is unclear how the R19 LTM configuration can be applied for fast recovery, thus, the discussion of LTM configuration handling in this case can be postponed.

The R19 inter-CU MCG LTM configuration is autonomously released for the following cases

  - Reception of RRCRelease.

  - Go to IDLE.

The R19 inter-CU SCG LTM configuration is autonomously released for the following cases

  - Reception of RRCRelease.

  - Go to IDLE.

  - MR-DC release.

- For PCell change or PSCell change/addition, an RRCReconfiguration message is required for the execution of the procedure, thus the handling of LTM configuration can rely on explicit NW indication.

- For MCG/SCG failure, an RRCReconfiguration message is expected to recover the radio link failure, the handling of LTM configuration can be also up to the NW.

It is up to explicit NW indication on how to handle the R19 inter-CU LTM configurations for the following cases:

  - Normal and conditional PCell change/PSCell change/addition.

  - MCG failure/SCG failure

SCG configuration handling in MCG LTM

In last RAN2 meeting, it was agreed that inter-CU MCG LTM with intra-SN PSCell change is supported.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

It is questionable on how to handling the SCG configuration during MCG LTM execution and we may have the following alternatives:

- Alt1: the same operations as MCG configuration handling, e.g., RLC bearers/entities are remained and the UE determines L2 reset operations by comparing between source SCG configuration and target SCG configuration.

- Alt2: mrdc-ReleaseAndAdd is reused, i.e., MR-DC is released and then added.

When there is no PSCell change, Alt1 is advantageous for ensuring service continuity, but it introduces extra complicity, e.g., the UE needs to check whether there is PSCell change. Conversely, Alt2 is simpler for implementation but may cause additional data interruption. Given that the gain of Alt1 is only evident in limited cases, Alt2 is preferred for sake of simplicity.

mrdc-ReleaseAndAdd is reused for SCG configuration handling during MCG LTM execution.

DRB handling in DC

In last RAN2 meeting, L2 reset operation for inter-CU LTM in DC case was discussed and the following agreements were made.

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

According to the agreements, PDCP handling was concluded in case that the Rel-19 IDs of candidate cell and serving cell are different. Further discussion is needed on RLC handling and our considerations are provided as follows.

- For inter-CU MCG LTM, RLC re-establishment is required due to security key update. If P4 can be agreed, all the SCG bearers are released and new RLC bearers will be established by applying the target cell configuration, i.e., if the DRB is associated with SCG bearer, no extra operation is needed except for following mrdc-ReleaseAndAdd. Consequently, RLC-establishment is only performed if the DRB is associated with MCG bearer that will not be released by the target cell.

For MCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, UE performs RLC re-establishment if the DRB is associated with MCG bearer that maintains unchanged (i.e., configured in both source cell configuration and target cell configuration).

- For inter-CU SCG LTM, RLC handling depends on the whether there is security key update and bearer type change as analysed in Table1.

Table 1 RLC handling in inter-CU SCG LTM

| Bearer type change from row to col | MCG |  | SCG |  | Split |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SN terminated bearer w/o termination point change /DRB with termination point change | MN terminated bearer w/o termination point change | SN terminated bearer w/o termination point change /DRB with termination point change | MN terminated bearer w/o termination point change | SN terminated bearer w/o termination point change /DRB with termination point change | MN terminated bearer w/o termination point change |
| MCG | MCG bearer: Re-establish | MCG bearer: No action | MCG bearer: Release SCG bearer: Establish | MCG bearer: Release SCG bearer: Establish | MCG bearer: Re-establish SCG bearer: Establish | MCG bearer: No action SCG bearer: Establish |
| SCG | MCG bearer: Establish SCG bearer: Release | MCG bearer: Establish SCG bearer: Release | SCG bearer: Re-establish | SCG bearer: Re-establish | MCG bearer: Establish SCG bearer: Re-establish | MCG bearer: Establish SCG bearer: Re-establish |
| Split | MCG bearer: Re-establish SCG bearer: Release | SCG bearer: Release MCG bearer: No action | MCG bearer: Release SCG bearer: Re-establish | MCG bearer: Release SCG bearer: Re-establish | MCG bearer: Re-establish SCG bearer: Re-establish | MCG bearer: No action. SCG bearer: Re-establish |

For SCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, for SN terminated bearer without termination point change and DRB with termination point change:

  - If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated RLC bearer that maintains unchanged if any.

  - If there is no bearer type change, the UE performs RLC re-establishment for the associated RLC bearer(s).

For SCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, for MN terminated bearer without termination point change:

  - If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated SCG bearer that maintains unchanged if any.

  - If there is no bearer type change, the UE performs RLC re-establishment for the associated SCG bearer if any.

Furthermore, current agreements only cover the case that the Rel-19 ID of candidate cell is different from serving cell. It is still unclear on how to perform L2 reset operations when the Rel-19 ID of candidate cell is the same as the serving cell. In this case, the UE does not need to perform security key update triggered by the change of CU and the L2 handling mainly depends on R18 ltm-NoResetID and whether there is termination point/bearer type change.

- MCG LTM:

-  For DRB with termination point change, the UE should perform PDCP re-establishment. If the DRB is associated with SCG bearer, all the SCG bearers will be released due to mrdc-ReleaseAndAdd operation as in P4. If the DRB is associated with MCG bearers which will not be released in the target cell, RLC re-establishment is required.

-  For DRB without termination point change, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for MCG bearer.

For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

  - The UE performs PDCP re-establishment.

  - For the associated MCG bearer that maintains unchanged if any, the UE performs RLC re-establishment.

For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

- For the associated MCG bearer that maintains unchanged if any, UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- If it is AM DRB, the UE performs PDCP data recovery when there is RLC bearer released/re-established.

- SCG LTM:

-  For DRB with termination point change, the UE should perform PDCP-re-establishment. For RLC handling, the same behaviour as for SN terminated bearer without termination point change / DRB with termination point change in Table1 can be followed.

-  For DRB without termination point change, most of RLC handling for MN terminated bearer w/o termination point change can be followed except for the involved SCG bearer. Different from inter-CU SCG LTM where RLC re-establishment is required for RLC bearer due to CU change, R19 NoResetID needs to be further check to determine whether to perform RLC re-establishment, i.e., following R18 behaviour. For PDCP, data recovery is required if there is RLC bearer released or re-established.

Based on above discussions, RLC handling in intra-CU SCG LTM is summarized in Table2. The different parts compared to Table1 is blue-highlighted.

Table 2 RLC handling in intra-CU SCG LTM

| Bearer type change from row to col | MCG |  | SCG |  | Split |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DRB with termination point change | DRB w/o termination point change | DRB with termination point change | DRB w/o termination point change | DRB with termination point change | DRB w/o termination point change |
| MCG | MCG bearer: Re-establish | MCG bearer: No action | MCG bearer: Release SCG bearer: Establish | MCG bearer: Release SCG bearer: Establish | MCG bearer: Re-establish SCG bearer: Establish | MCG bearer: No action SCG bearer: Establish |
| SCG | MCG bearer: Establish SCG bearer: Release | MCG bearer: Establish SCG bearer: Release | SCG bearer: Re-establish | SCG bearer: R18 behaviour | MCG bearer: Establish SCG bearer: Re-establish | MCG bearer: Establish SCG bearer: R18 behaviour |
| Split | MCG bearer: Re-establish SCG bearer: Release | SCG bearer: Release MCG bearer: No action | MCG bearer: Release SCG bearer: Re-establish | MCG bearer: Release SCG bearer: R18 behaviour | MCG bearer: Re-establish SCG bearer: Re-establish | MCG bearer: No action. SCG bearer: R18 behaviour |

For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

  - The UE performs PDCP re-establishment.

  - If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated RLC bearer that maintains unchanged if any

  - If there is no bearer type change, the UE performs re-establishment for the associated RLC bearer(s).

For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

  - If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer that maintains unchanged if any.

  - If there is no bearer type change, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer if any.

  - If it is AM DRB, the UE performs PDCP data recovery when there is RLC bearer released/re-established.

SRB handling in DC

For SRB handing in inter-CU SCG LTM, consensus was made to follow SCPAC principle as below agreements.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

Further discussion is needed on how to handle the SRBs in inter-CU MCG LTM. For SRB3, if mrdc-ReleaseAndAdd is applied for SCG handling during MCG LTM execution, there is no need to specify SRB3 handling since it will be released during execution. For SRB1/SRB2, when Rel-19 Set ID of candidate cell is different from serving cell, the UE needs to perform security key update including both master key (i.e. KgNB) and secondary key (i.e. S-KgNB), thus PDCP re-establishment is required. When Rel-19 Set ID of candidate cell is the same as serving cell, the RRC message intended for the source PCell (i.e., RRC messages via SRB1/2) should be discarded due to PCell change.

For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

Inter-CU SCG LTM security

For security update in inter-CU SCG LTM, it was agreed to follow SCPAC according to the agreement made in previous meeting.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

For SCPAC, each SN is configured with a list of sk-counter following the SA3 solution as in R2-2307070 and the UE selects the first unused sk-counter when performing inter-SN SCPAC.

While in current RRC CR, only one sk-counter list is configured for all LTM candidates. The problem with having a single sk-counter list is that the KSN is unavailable for SN until MN receives the RRCReconfigurationComplete message including the candidate cell ID and the selected sk-counter. This operation will delay the LTM completion as the SN can not ascertain the success of the first UL transmission without security information. Therefore, it is proposed to further clarity that the sk-counter list for inter-CU SCG LTM security key update is configured per SN.

The sk-counter list for inter-CU SCG LTM security key update is configured per SN.

Number of candidate cells

According to RRC CR, one left FFS is whether 8 is the number or LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations. Supporting more candidates will lead to higher complexity on UE implementation, e.g. configuration storing and processing, L1 measurement. It would be enough to support at most 8 candidates for all types of LTM, thus it is proposed that the maximum number of LTM plus CLTM candidate cells is 8.

The maximum number of LTM plus CLTM candidate cells is 8, including MCG LTM, SCG LTM, MCG CLTM and SCG CLTM (if supported) .

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	The R19 inter-CU MCG LTM configuration is autonomously released for the following cases

o	Reception of RRCRelease.

o	Go to IDLE.

Proposal 2	The R19 inter-CU SCG LTM configuration is autonomously released for the following cases

o	Reception of RRCRelease.

o	Go to IDLE.

o	MR-DC release.

Proposal 3	It is up to explicit NW indication on how to handle the R19 inter-CU LTM configurations for the following cases:

o	Normal and conditional PCell change/PSCell change/addition.

o	MCG failure/SCG failure

Proposal 4	mrdc-ReleaseAndAdd is reused for SCG configuration handling during MCG LTM execution.

Proposal 5	For MCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, UE performs RLC re-establishment if the DRB is associated with MCG bearer that maintains unchanged (i.e., configured in both source cell configuration and target cell configuration).

Proposal 6	For SCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, for SN terminated bearer without termination point change and DRB with termination point change:

o	If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated RLC bearer that maintains unchanged if any.

o	If there is no bearer type change, the UE performs RLC re-establishment for the associated RLC bearer(s).

Proposal 7	For SCG LTM, when Rel-19 Set ID of candidate cell is different from serving cell, for MN terminated bearer without termination point change:

o	If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to split bearer, RLC re-establishment is required for the associated SCG bearer that maintains unchanged if any.

o	If there is no bearer type change, the UE performs RLC re-establishment for the associated SCG bearer if any.

Proposal 8	For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

o	The UE performs PDCP re-establishment.

o	For the associated MCG bearer that maintains unchanged if any, the UE performs RLC re-establishment.

Proposal 9	For MCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

o	For the associated MCG bearer that maintains unchanged if any, UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Proposal 10	For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB with termination point change:

o	The UE performs PDCP re-establishment.

o	If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to non-split bearer, RLC re-establishment is required for the associated RLC bearer that maintains unchanged if any

o	If there is no bearer type change, the UE performs re-establishment for the associated RLC bearer(s).

Proposal 11	For SCG LTM, when Rel-19 Set ID of candidate cell is the same with serving cell, for DRB without termination point change:

o	If there is bearer type change, the UE performs RLC release and/or establishment. For split bearer changes from/to split bearer, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer that maintains unchanged if any.

o	If there is no bearer type change, the UE compares the ltm-ServingCellNoResetID and the ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18 for the associated SCG bearer if any.

o	If it is AM DRB, the UE performs PDCP data recovery when there is RLC bearer released/re-established.

Proposal 12	For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

Proposal 13	For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

Proposal 14	The sk-counter list for inter-CU SCG LTM security key update is configured per SN.

Proposal 15	The maximum number of LTM plus CLTM candidate cells is 8, including MCG LTM, SCG LTM, MCG CLTM and SCG CLTM (if supported) .