3GPP TSG-RAN WG2 Meeting #129                              	  R2-2500208

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Discussion on event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In previous meeting, RAN2 has discussed the issues related to event-triggered L1 measurement reporting and the following agreements were reached:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we discuss the remaining open issues on event-triggered L1 measurement reporting.

Discussion

SR for MR MAC CE

In earlier meeting, it has agreed that NW can configure a dedicated SR configuration for MR MAC CE transmission. And the legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

There are still open issues related to SR handling which need to be further studied. The first one is SR cancelation condition. If the SR is triggered for L1 event-triggered measurement report, and if a MAC PDU is transmitted and this PDU includes L1 measurement report MAC CE, the triggered SR should be cancelled accordingly.

The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 measurement report MAC CE.

Another issue is about SR overlapping. In current spec, there are some note to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs.

- Case 1: non-BFR SR overlapping

Except for SR for SCell beam failure recovery, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

- Case 2: BFR SR& non-BFR SR

When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

- Case 3 Scell BFR SR & BFD-RS set BFR SR

When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set.

For L1 event triggered measurement report, there may be similar issue. When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, which PUCCH resource is considered as valid.

In general, there are three ways:

- Option 1: left to UE implementation.

- Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

- Option 1: left to UE implementation.

- Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

MR MAC CE delivery

In TS 38.321 section 5.4.3.1.3, the logical channel priority has been defined as following.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

…

As we have agreed to used MAC CE to carry the event triggered measurement report, the LCH priority of the MR MAC CE should also be defined. In our understanding, the MR MAC CE is quite important to assist the NW to make a correct cell switch execution decision, especially for the MR MAC CE triggered by the “cell switch execution” event. To guarantee the MR MAC CE to be delivered promptly, same priority as BFR MAC CE should be baseline.

MR MAC CE have same LCH priority as MAC CE for (Enhanced) BFR.

Regarding the resource which can be used for MR MAC CE transmission, it seems there is no motivation to restrict the resource type for MR MAC CE transmission. To guarantee a real-time measurement report, the MR MAC CE should be delivered to NW as soon as possible. Furthermore, the MR MAC CE can be delivered through both SPcell and scell.

MR MAC CE can be transmitted with any type of grant.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 measurement report MAC CE.

Proposal 2	When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

-	Option 1: left to UE implementation.

-	Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

-	Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

Proposal 3	MR MAC CE have same LCH priority as MAC CE for (Enhanced) BFR.

Proposal 4	MR MAC CE can be transmitted with any type of grant.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx