3GPP TSG-RAN WG2 Meeting#129	R2-2500215

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Simulation results of model generalization for RRM measurement prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

At RAN2#128 meeting [1], RAN2 made the following agreements about model generalization for RRM prediction:

Agreements:

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases,

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

2	Companies can choose which case they compare with and should report it with simulation results.

3	Generalization issues on RRM measurement prediction are prioritized.

4	Start the study with generalization issue with RRM measurement prediction in temporal domain.   Companies can choose to study frequency domain prediction cases and report what they have simulated.

5	Study generalization over UE speeds

6	The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h.  FFS on combinations

7	The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h.  FFS on combinations

8	Between GC#1 and GC#2, RAN2 focus on GC#2

In this contribution, we provide our understanding of the objective of the model generalization study first and then present our simulation results and analyses of model generalization in terms of UE speed, frequency, and cell configuration, respectively.

- Discussion

  - The Objective of Model Generalization Study

In TR 38.843 [2], the definition of model generalization is using one model that is generalizable to different scenarios/configurations/sites. However, how to determine whether one model is generalizable at a specific configuration is not clear. Therefore, we propose:

Proposal 1: RAN2 to discuss how to determine whether one model is generalizable to different configurations, the following two options can be considered:

- Option 1: whether model is generalizable depends on whether the performance can fulfill the prediction accuracy requirement when inference at different configurations.

- Option 2: whether model is generalizable depends on whether the degree of performance degradation is tolerable for a model trained at a different configuration from the baseline (i.e., training and inference at the same configuration).

Currently, we think Option 1 meets the definition of model generalization. Thus option 1 is selected in the following discussion.

Meanwhile, to study the generalization performance of an AI/ML model over various configurations, Case 2 (similar to GC#1 in AI mobility study) in CSI feedback enhancement and Beam management use cases are different:

- In CSI feedback, the model is trained based on dataset from a different configuration with the baseline and then tested at the same configuration with the baseline.

- In Beam management, the model is trained based on dataset from the same configuration with the baseline and then tested at the same configuration with the baseline.

### 6.2.1 Evaluation assumptions, methodology and KPIs (CSI) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#B/Configuration#B, and then the AI/ML model performs inference/test on a dataset from the same Scenario#B/Configuration#B - Case 2: The AI/ML model is trained based on training dataset from a different data set than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on Scenario#B/Configuration#B. - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#B/Configuration#B and a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from Scenario#B/Configuration#B. ### 6.3.1 Evaluation assumptions, methodology and KPIs (Beam) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#B/Configuration#B and a different dataset than Scenario#B/Configuration#B, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#A, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from Scenario#B/Configuration#B.

In summary, two dimensions of model generalization study on GC#1 are illustrated in Table 2.1-1:

Table 2.1-1: Two dimensions of model generalization study on GC#1

|  | Model generalization study Dimension 1 | Model generalization study Dimension 2 |
| --- | --- | --- |
| Training dataset | Training datasets from multiple configurations | Training dataset from a single configuration |
| Testing dataset | Testing dataset from a single configuration | Testing datasets from multiple configurations |

At the RAN2#128 meeting, the agreement on the definition of generalization cases and the combination in the tables of generalization study for speed are not aligned especially for GC#1. To fix the misalignment, the rapporteur updated the generalization case in the tables, which is Dimension 1 in Table 2.1-1.

From our understanding, both dimensions make sense and can provide informative observations. Specifically, Dimension 1 can illustrate the performance degradation of the model trained in a different configuration(s) compared with the model trained with the dataset configured the same as the testing one. Dimension 2 can indicate whether one model trained in a certain configuration is generalizable and can be reused in different configurations or not based on the performance of model testing. On top of that, we think Dimension 2 is more critical as the performance of the model trained in the new configuration is literally not available in the real deployment.

Therefore, we propose:

Proposal 2: The model generalization study for GC#1 can consider the following two dimensions:

- Dimension 1: Multiple models trained in different configurations are tested in the same configuration, to study whether the specific configuration has an impact on the generalization performance;

- Dimension 2: One model trained in a single configuration is tested in different configurations, to study whether the model is generalizable in different configurations.

In this understanding, we provide the analysis of both Model generalization Dimension 1 and Dimension 2 for GC#1 in the following sections.

  - Generalization study for UE speed

    - FR1 intra-frequency temporal domain

Based on our previous simulation results, for the FR1 intra-frequency temporal domain, sub-use case 1 achieves better performance than sub-use cases 2 and 3. Thus we carry out the generalization simulation toward sub-use case 1 for FR1. Some key simulation parameters are shown in the following table:

Table 2.2-1: Key parameters of generalization study for UE speed in FR1

| Parameter | Values |
| --- | --- |
| Sub-use case | 1 |
| L1/L3 filtering | Non-sliding |
| OW length | 10*200=2000 ms |
| PW length | 200 ms |
| MRRT | 50% |
| Pattern |  |

The simulation results are illustrated in Table 2.2-2.

Table 2.2-2: Simulation results of generalization study for UE speed in FR1

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | 30 km/h | 60 km/h | 90 km/h | Average of [30,60,90] |
| --- | --- | --- | --- | --- |
| 30 km/h | 0.442 | 0.683 | 0.888 | 0.671 |
| 60 km/h | 0.448 | 0.676 | 0.877 | 0.667 |
| 90 km/h | 0.453 | 0.680 | 0.875 | 0.670 |
| [30,60,90] km/h | 0.445 | 0.676 | 0.873 | 0.665 |

Dimension 1 of GC#1: model trained in different configurations and tested in the same configuration

The updated evaluation combinations for FR1 generalization study on UE speed are illustrated in Table 2.2-3.

Table 2.2-3: Generalization cases of Dimension 1 for UE speed in FR1

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | test @ Dataset: 30km/h | test @ Dataset: 60km/h | test @ Dataset: 90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1_1 |  | Yes |  | Yes |  |  |
| GC#1_2 |  |  | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1_1 | Yes |  |  |  | Yes |  |
| GC#1_2 |  |  | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1_1 | Yes |  |  |  |  | Yes |
| GC#1_2 |  | Yes |  |  |  | Yes |

To analyze the performances of each generalization case, Table 2.2-4 illustrates the prediction accuracy of baseline, GC#1_1 and GC#1_2 in Dimension 1.

Table 2.2-4: Simulation results of GC#1 for UE speed in Dimension 1

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1_1 | GC#1_2 |
| --- | --- | --- | --- |
| Testing @ 30 km/h | 0.442 (Training @ 30 km/h) | 0.448 (Training @ 60km/h) | 0.453 (Training @ 90km/h) |
| Testing @ 60 km/h | 0.676 (Training @ 60 km/h) | 0.683 (Training @ 30 km/h) | 0.680 (Training @ 90 km/h) |
| Testing @ 90 km/h | 0.875 (Testing @ 90 km/h) | 0.888 (Testing @ 30 km/h) | 0.877 (Testing @ 60 km/h) |

Based on the simulation results, we have the following observations:

- For the model testing at UE speed of 30km/h, the prediction accuracy of GC#1 degrades compared with that of the baseline, with a degradation of 0.006 ~0.011 dB.

- For the model testing at UE speed of 60km/h, the prediction accuracy of GC#1 degrades compared with that of the baseline, with a degradation of 0.004 ~0.007 dB.

- For the model testing at UE speed of 90km/h, the prediction accuracy of GC#1 degrades compared with that of the baseline, with a degradation of 0.002 ~0.013 dB.

In summary, the model trained from the dataset with different UE speeds performs basically at the same degree as the baseline, in which the model is trained and tested by the dataset with the same UE speed.

Dimension 2 of GC#1: model trained in a single configuration and tested in different configurations

Table 2.2-5 follows the generalization cases defined for UE speed in FR1 agreed at the RAN2#128 meeting [1].

Table 2.2-5: Generalization cases of Dimension 2 for UE speed in FR1

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | test @ Dataset: 30km/h | test @ Dataset: 60km/h | test @ Dataset: 90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1_1 | Yes |  |  |  | Yes |  |
| GC#1_2 | Yes |  |  |  |  | Yes |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1_1 |  | Yes |  | Yes |  |  |
| GC#1_2 |  | Yes |  |  |  | Yes |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1_1 |  |  | Yes |  | Yes |  |
| GC#1_2 |  |  | Yes | Yes |  |  |

To analyze the performance of each generalization case, Table 2.2-6 illustrates the prediction accuracy of baseline, GC#1_1 and GC#1_2 in dimension 2.

Table 2.2-6: Simulation results of GC#1 for UE speed in Dimension 2

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1_1 | GC#1_2 |
| --- | --- | --- | --- |
| Training @ 30 km/h | 0.442 (Testing @ 30 km/h) | 0.683 (Testing @ 60km/h) | 0.888 (Testing @ 90km/h) |
| Training @ 60 km/h | 0.676 (Testing @ 60 km/h) | 0.448 (Testing @ 30 km/h) | 0.877 (Testing @ 90 km/h) |
| Training @ 90 km/h | 0.875 (Testing @ 90 km/h) | 0.453 (Testing @ 30 km/h) | 0.680 (Testing @ 60 km/h) |

Based on the simulation results, we have the following observations:

- For the model trained at UE speed of 30km/h, the prediction accuracy of GC#1 degrades compared with that of Baseline, with a degradation of 0.241~0.446 dB.

- For the model trained at UE speed of 60km/h, the prediction accuracy of GC#1_1@30km/h improves compared with that of Baseline, with an improvement of 0.228 dB; while the prediction accuracy of GC#1_2@90km/h degrades compared with that of the baseline, with a degradation of 0.201 dB.

- For the model trained at UE speed of 90km/h, the prediction accuracy of GC#1 improves compared with that of Baseline, with an improvement of 0.195~0.422 dB.

GC#2: model trained in mixed configurations and tested in the same configuration

To see the variation tendency of GC#2, the tree diagrams is shown in Figure 2.2-1.

Figure 2.2-1: Simulation results of GC#2 for UE speed

Visually, the model trained by the mixed dataset (i.e., GC#2) also performs basically at the same degree as the baseline, in which the model is trained and tested by the dataset with the same UE speed.

    - FR2 intra-frequency temporal domain

Based on the agreed observation, for the FR2 intra-frequency temporal domain, sub-use case 2 achieves better performance than sub-use cases 1 and 3. Thus we carry out the generalization simulation toward sub-use case 2 for FR2. Some key simulation parameters are shown in the following table:

Table 2.2-7: Key parameters of generalization study for UE speed in FR2

| Parameter | Values |
| --- | --- |
| Sub-use case | 2 |
| L1/L3 filtering | Sliding |
| OW length | 5*80=400 ms |
| PW length | 5*80=400 ms |
| Pattern |  |

The simulation results are illustrated in Table 2.2-8.

Table 2.2-8: Simulation results of generalization study for UE speed in FR2

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | 60 km/h | 90 km/h | 120 km/h | Average of [60,90,120] |
| --- | --- | --- | --- | --- |
| 60 km/h | 1.097 | 1.522 | 1.824 | 1.481 |
| 90 km/h | 1.105 | 1.479 | 1.750 | 1.445 |
| 120 km/h | 1.121 | 1.471 | 1.703 | 1.431 |
| [60,90,120] km/h | 1.094 | 1.486 | 1.739 | 1.436 |

We have similar observations for FR2:

- For GC#1 dimension 1, the model trained from the dataset with different UE speeds performs basically at the same degree as the baseline, in which the model is trained and tested by the dataset with the same UE speed.

- For GC#1 dimension 2:

- For the model trained at UE speed of 60km/h, the prediction accuracy of GC#1 degrades compared with that of the baseline, with a degradation of 0.425~0.727 dB.

- For the model trained at UE speed of 90km/h, the prediction accuracy of GC#1_1@60km/h improves compared with that of the baseline, with an improvement of 0.374 dB; while the prediction accuracy of GC#1_2@120km/h degrades compared with that of the baseline, with a degradation of 0.271 dB.

- For the model trained at UE speed of 120km/h, the prediction accuracy of GC#1 improves compared with that of the baseline, with an improvement of 0.253~0.645 dB.

- For GC#2, the model trained by the mixed dataset also performs basically at the same degree with the baseline, in which the model is trained and tested by the dataset with the same UE speed.

    - Analysis of UE speed

As indicated in section 2.2.2, the variation in generalization performance for FR1 and FR2 are quite similar. For simplicity, this section provides the analysis for FR2.

For GC#1 dimension 1 and GC#2, the tree diagram of simulation results is depicted in Figure 2.2-2.

Figure 2.2-2: Simulation results of GC#1 dimension 1 and GC#2 for UE speed in FR2

It can be observed that the model trained from the dataset with different UE speeds or mixed UE speeds performs basically at the same degree as the baseline, in which the model is trained and tested by the dataset with the same UE speed.

Observation 1: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain UE speed, the model trained with different UE speed (GC#1) or mixed UE speeds (GC#2) is comparable in prediction accuracy to the model trained in this certain UE speed (Baseline).

Proposal 3: For intra-frequency temporal domain prediction in FR1/FR2, UE speed has a minor impact on the generalization performance.

For GC#1 dimension 2, the tree diagram of simulation results is depicted in Figure 2.2-3.

Figure 2.2-3: Simulation results of GC#1 dimension 2 for UE speed in FR2

It can be observed that when trained at a low UE speed, the model performance will decrease when inference at a higher UE speed. Oppositely, the model trained at a high UE speed can perform better when inference at a lower UE speed. Based on observation 1, this is due to higher UE speeds resulting in larger prediction errors for the intra-frequency temporal domain rather than generalization impact.

Observation 2: For intra-frequency temporal domain prediction in FR1/FR2, the model trained at a high UE speed shows better prediction accuracy when tested at a lower UE speed.

Observation 3: For intra-frequency temporal domain prediction in FR1/FR2, the model trained at a low UE speed shows worse prediction accuracy when tested at a higher UE speed.

  - Generalization Study for Frequency

Some key simulation parameters for inter-frequency prediction are shown in the following table:

Table 2.3-1: Key parameters of generalization study for frequency

| Parameter | Values |
| --- | --- |
| L1/L3 filtering | Sliding |
| OW length | 50*40=2000 ms |
| PW length | 40 ms |
| UE speed | 30 km/h |
| Pattern |  |

The simulation results are illustrated in Table 2.3-2.

Table 2.3-2: Simulation results of generalization study for frequency

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | 2G -> 4G | 4G -> 2G |
| --- | --- | --- |
| 2G -> 4G | 0.70 | 6.38 |
| 4G -> 2G | 6.20 | 0.76 |
| Mixed data | 0.83 | 0.83 |
| Mixed data (input labeled with frequency) | 0.73 | 0.77 |

Following the generalization cases defined for UE speed, we additionally study generalization cases over frequency at different dimensions for GC#1.

Dimension 1 of GC#1: model trained in different configurations and tested in the same configuration

The generalization cases of Dimension 1 for inter-frequency is described in Table 2.3-3.

Table 2.3-3: Generalization cases of Dimension 1 for inter-frequency

|  | Training @Dataset: 2G->4G | Training @Dataset: 4G->2G | test @Dataset 2G->4G | test @Dataset 4G->2G |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 |  | Yes | Yes |  |
| Baseline |  | Yes |  | Yes |
| GC#1 | Yes |  |  | Yes |

Table 2.3-4 illustrates the prediction accuracy of baseline, GC#1 in Dimension 1.

Table 2.3-4: Simulation results of GC#1 for prediction direction in Dimension 1

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Testing @ 2G->4G | 0.70 (Training @ 2G->4G) | 6.20 (Training @ 4G->2G) |
| Testing @ 4G->2G | 0.76 (Training @ 4G->2G) | 6.38 (Training @ 2G->4G) |

Based on the simulation results, we have the following observations:

- The performance of models in GC#1 degrades a lot from the baseline if the prediction direction of the training dataset is opposite to the baseline, with an average degradation of 5.56 dB in L3 cell RSRP difference.

Observation 4: The change of prediction direction (GC#1) for inter-frequency RRM prediction will result in significant performance degradation.

Proposal 4: For inter-frequency prediction, prediction direction has a significant impact on the generalization performance.

Dimension 2 of GC#1: model trained in a single configuration and tested in different configurations

The generalization cases of Dimension 2 for inter-frequency is described in Table 2.3-5.

Table 2.3-5: Generalization cases of Dimension 2 for inter-frequency

|  | Training @Dataset: 2G->4G | Training @Dataset: 4G->2G | test @Dataset 2G->4G | test @Dataset 4G->2G |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 | Yes |  |  | Yes |
| Baseline |  | Yes |  | Yes |
| GC#1 |  | Yes | Yes |  |

Table 2.3-6 illustrates the prediction accuracy of baseline, GC#1 in Dimension 2.

Table 2.3-6: Simulation results of GC#1 for prediction direction in Dimension 2

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Training @ 2G->4G | 0.70 (Testing @ 2G->4G) | 6.38 (Testing @ 4G->2G) |
| Training @ 4G->2G | 0.76 (Testing @ 4G->2G) | 6.20 (Testing @ 2G->4G) |

Based on the simulation results, we have the following observations:

- The performance of models in GC#1 degrades a lot from the baseline if the prediction direction changes.

Therefore, we propose:

Proposal 5: For inter-frequency prediction, the model is not generalizable when the prediction direction (i.e., low-to-high or high-to-low) is different from the training one(i.e., high-to-low or low-to-high).

GC#2: model trained in mixed configurations and tested in the same configuration

Based on Table 2.3-2, it can be observed that the model performance of GC#2 (when trained with a mixed dataset) is slightly worse than that of Baseline, at 0.13 dB and 0.07 dB reduction for low-to-high frequency prediction and high-to-low frequency prediction. In addition, if the input can be labeled with frequency, then the performance is comparable with the baseline (0.01/0.03 dB reduction)

Observation 5: For inter-frequency prediction, the model trained with a mixed dataset (GC#2) can achieve comparable performance with Baseline, especially when the input data is labeled with frequency.

Proposal 6: For inter-frequency prediction, AI/ML model trained with a mixed dataset of different prediction directions can improve generalization performance.

  - Generalization Study for Cell Configuration

In regard to the generalization aspects mentioned in [Post128][018][AI Mob], two sets of cell configurations for the generalization study are shown in Table 2.4-1.

Table 2.4-1: Cell configurations for Baseline and GC#1/2

| Parameter | Cell Configuration #A | Cell Configuration #B |
| --- | --- | --- |
| UMa/UMi | UMi | UMa |
| ISD（Inter-Site Distance） | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

    - FR1 intra-frequency temporal domain

The key simulation parameters in Table 2.2-1 are reused.

The simulation results of FR1 for Case B are illustrated in Table 2.4-2.

Table 2.4-2: Simulation results of generalization study for Cell configuration in FR1

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | UMi | UMa |
| --- | --- | --- |
| UMi | 0.558 | 0.498 |
| UMa | 0.577 | 0.424 |
| Mixed data | 0.559 | 0.428 |

    - FR2 intra-frequency temporal domain

The key simulation parameters in Table 2.2-7 are reused.

The simulation results of FR2 for Case A are illustrated in Table 2.4-3.

Table 2.4-3: Simulation results of generalization study for Cell configuration in FR2

(L3 cell-level prediction error in dB)

| Testing Dataset Training Dataset | UMi | UMa |
| --- | --- | --- |
| UMi | 1.127 | 0.915 |
| UMa | 1.187 | 0.889 |
| Mixed data | 1.151 | 0.878 |

    - Analysis of Cell Configuration

Based on the results in FR1 and FR2 respectively in Table 2.4-2 and Table 2.4-3, it can be observed that the variation in generalization performance between GC and Baseline are similar. Since the PW length of FR2 for Case A is longer leading to decreased prediction accuracy, to better to illustrate the impact of configuration on generalization, FR2 is selected for analysis.

In this section, we still illustrate the generalization cases of Dimension 1 and Dimension 2 towards different cell configurations for GC#1.

Dimension 1 of GC#1: model trained in different configurations and tested in the same configuration

The generalization cases of Dimension 1 for cell configuration is described in Table 2.4-4.

Table 2.4-4: Generalization cases of Dimension 1 for cell configuration

|  | Training @Dataset: UMi | Training @Dataset: UMa | testing Dataset @UMi | Testing Dataset @UMa |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 |  | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |
| Baseline |  | Yes |  | Yes |
| GC#1 | Yes |  |  | Yes |
| GC#2 | Yes | Yes |  | Yes |

Table 2.4-5 illustrates the prediction accuracy of baseline, GC#1 in Dimension 1.

Table 2.4-5: Simulation results of GC#1 for Cell configuration in Dimension 1

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Testing @ UMi | 1.127 (Training @ UMi) | 1.187 (Training @ UMa) |
| Testing @ UMa | 0.889 (Training @ UMa) | 0.915 (Training @ UMi) |

Based on the simulation results in FR2, we have the following observations:

- For the model testing in UMi, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.06 dB.

- For the model testing in UMa, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.026 dB.

GC#2: model trained in mixed configurations and tested in the same configuration

Based on Table 2.4-3, it can be observed that:

- For the model testing in UMi, the model performance of GC#2 (when trained with a mixed dataset) is slightly worse than that of Baseline, at 0.024 dB.

- For the model testing in UMa, the model performance of GC#2 (when trained with a mixed dataset) is slightly better than that of Baseline, at 0.011 dB.

Based on the result of GC#1 Dimension 1 and GC#2, we have the following observations and proposals:

Observation 6: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain cell configuration (UMa or UMi), the model trained from the dataset at different cell configuration (UMi or Uma) (GC#1) or mixed cell configurations (GC#2) can achieve comparable performance with the Baseline.

Proposal 7: For intra-frequency temporal domain prediction in FR1/FR2, cell configuration has a minor impact on the generalization performance.

Dimension 2 of GC#1: model trained in a single configuration and tested in different configurations

The generalization cases of Dimension 2 for cell configuration is described in Table 2.4-6.

Table 2.4-6: Generalization cases of Dimension 2 for cell configuration

|  | Training @Dataset: UMi | Training @Dataset: UMa | Testing @Dataset: UMi | Testing @Dataset: UMa |
| --- | --- | --- | --- | --- |
| Baseline | Yes |  | Yes |  |
| GC#1 | Yes |  |  | Yes |
| Baseline |  | Yes |  | Yes |
| GC#1 |  | Yes | Yes |  |

Table 2.4-7 illustrates the prediction accuracy of baseline, GC#1 in Dimension 2.

Table 2.4-7: Simulation results of GC#1 for prediction direction in Dimension 2

(L3 cell-level prediction error in dB)

|  | Baseline | GC#1 |
| --- | --- | --- |
| Training @ UMi | 1.127 (Testing @ UMi) | 0.915 (Testing @ UMa) |
| Training @ UMa | 0.889 (Testing @ UMa) | 1.187 (Testing @ UMi) |

Based on the result of GC#1 Dimension 2, we have the following observations and proposals:

- For the model trained in UMi, the prediction accuracy of GC#1 increases compared with that of Baseline, at 0.212 dB.

- For the model trained in UMa, the prediction accuracy of GC#1 degrades slightly compared with that of Baseline, with a degradation of 0.298 dB.

From our understanding, this is due to UMi scenario is more complicated than UMa, resulting in larger prediction errors for the intra-frequency temporal domain.

Based on the result, we propose:

Observation 7: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario.

Observation 8: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMa scenario shows worse prediction accuracy when tested in UMi scenario.

- Conclusion

Based on the above analysis, we make the following observations and proposals:

Objective of Model Generalization

Proposal 1: RAN2 to discuss how to determine whether one model is generalizable to different configurations, the following two options can be considered:

- Option 1: whether model is generalizable depends on whether the performance can fulfill the prediction accuracy requirement when inference at different configurations.

- Option 2: whether model is generalizable depends on whether the degree of performance degradation is tolerable for a model trained at a different configuration from the baseline (i.e., training and inference at the same configuration).

Proposal 2: The model generalization study for GC#1 can consider the following two dimensions:

- Dimension 1: Multiple models trained in different configurations are tested in the same configuration, to study whether the specific configuration has an impact on the generalization performance;

- Dimension 2: One model trained in a single configuration is tested in different configurations, to study whether the model is generalizable in different configurations.

Generalization study for UE speed

Observation 1: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain UE speed, the model trained with different UE speed (GC#1) or mixed UE speeds (GC#2) is comparable in prediction accuracy to the model trained in this certain UE speed (Baseline).

Observation 2: For intra-frequency temporal domain prediction in FR1/FR2, the model trained at a high UE speed shows better prediction accuracy when tested at a lower UE speed.

Observation 3: For intra-frequency temporal domain prediction in FR1/FR2, the model trained at a low UE speed shows worse prediction accuracy when tested at a higher UE speed.

Proposal 3: For intra-frequency temporal domain prediction in FR1/FR2, UE speed has a minor impact on the generalization performance.

Generalization Study for Frequency

Observation 4: The change of prediction direction (GC#1) for inter-frequency RRM prediction will result in significant performance degradation.

Observation 5: For inter-frequency prediction, the model trained with a mixed dataset (GC#2) can achieve comparable performance with Baseline, especially when the input data is labeled with frequency.

Proposal 4: For inter-frequency prediction, prediction direction has a significant impact on the generalization performance.

Proposal 5: For inter-frequency prediction, the model is not generalizable when the prediction direction (i.e., low-to-high or high-to-low) is different from the training one(i.e., high-to-low or low-to-high).

Proposal 6: For inter-frequency prediction, AI/ML model trained with a mixed dataset of different prediction directions can improve generalization performance.

Generalization Study for Cell configuration

Observation 6: For intra-frequency temporal domain prediction in FR1/FR2, when tested in a certain cell configuration (UMa or UMi), the model trained from the dataset at different cell configuration (UMi or Uma) (GC#1) or mixed cell configurations (GC#2) can achieve comparable performance with the Baseline.

Observation 7: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMi scenario shows better prediction accuracy when tested in UMa scenario.

Observation 8: For intra-frequency temporal domain prediction in FR1/FR2, the model trained in UMa scenario shows worse prediction accuracy when tested in UMi scenario.

Proposal 7: For intra-frequency temporal domain prediction in FR1/FR2, cell configuration has a minor impact on the generalization performance.

- Reference

- R2_128_ChairNotes

- TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)