3GPP TSG-RAN WG2 Meeting #129	                                                                                   R2-2500222

Athens, Greece, Feb. 17th – 21st, 2025

Source:                 CATT

Title:	                L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this paper, we further explore the open issues regarding L1 event-triggered measurement reporting.

The organization of this contribution is as follows. In Section 2, we discuss the following aspects.

- Measurement report MAC CE

- Support of CSI-RS measurements for LTM

Our proposals are summarized in Section 3.

Discussion

Measurement report MAC CE

LCP priority of the measurement report  MAC CE

The priority of the L1 measurement report MAC CE is a FFS in the MAC running CR.

Editor’s NOTE: FFS the priority order of event triggered L1 measurement report MAC CE.

In the legacy MAC spec, UL MAC CEes have different LCP priorities as follows,

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; - MAC CE for Sidelink Configured Grant Confirmation; - MAC CE for LBT failure; - MAC CE for SL LBT failure according to clause 5.31.2; ……

Considering the MR is used for mobility management, the MAC entity shall prioritize the MAC CE in a higher order than data from any Logical Channel. But it should be lower than the MAC CE used for serving cell channel quality management e.g. BFR MAC CE.

Proposal 1: The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'. And it should have a lower LCP than BFR MAC CE.

In the MAC running CR, there is a FFS on L1 measurement report MAC CE can only be transmitted in SpCell.

Editor’s NOTE: FFS whether the event trigger L1 measurement report MAC CE can only be transmitted in SpCell.

We see no motivation to have this restriction.

Proposal 2: Remove the restriction in the MAC running CR that the transmission of the event - triggered L1 measurement report MAC CE must occur only on the SpCell.

Whether to use a single MR MAC CE when multiple events trigger simultaneously

When multiple events happen at the same time, reporting the relevant measurement results in one MR MAC CE is more resource-efficient than using separate MAC CEs.

Thus, it is proposed that:

Proposal 3: A single MR MAC CE is used when multiple events trigger simultaneously.

Whether to report the measurement results of beams meeting the leaving condition

In legacy L3 measurement report, when configured by the NW, UE will trigger the Measurement report when one cell meets the leaving condition. The measurement report includes the measurement results of the cells fulfilled the entering condition. The similar mechanism could be reused for L1 event measurement report.

Proposal 4: When a beam meets the leaving condition and the network configures the triggering of a Measurement Report (MR), the reported MR includes the measurement results of the beams that meet the entering condition as well as those of the other beams that do not meet the entering condition if configured.

Support of CSI-RS measurements for LTM

For the enhancement on supporting of the CSI-RS measurements for LTM procedures [1],

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1]

RAN1 has assumed that semi-persistent CSI-RS is supported, as follows,

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

RAN1 also sent LS [2] to RAN3 for the purpose of confirming the feasibility. Subsequently, RAN3 has affirmed the feasibility within the RAN3 reply LS [3].

In order to support semi-persistent CSI-RS, it is essential to design a solution that can dynamically activate or deactivate the semi-persistent CSI-RS for candidate LTM cell. In legacy for serving cell, this functionality is accomplished via MAC CE (SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE). Therefore, despite that in the LS, RAN1 merely requests RAN3 to assess the feasibility and potential workload on RAN3, there are also impacts on the RAN2 specifications. For instance, a MAC CE is needed for the activation and deactivation of the semi-persistent CSI-RS for LTM candidate cell.

Observation 1: To support the semi-persistent CSI-RS of a candidate cell, RAN2 impact is expected regarding the activation/deactivation of the LTM semi-persistent CSI-RS.

With regard to the MAC CE design for the activation and deactivation of LTM SP CSI-RS, two options are possible:

- Option 1: Extend the existing MAC CE

Under this option, the existing MAC CE (namely, SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE) is extended to be utilized for the activation and deactivation of LTM SP CSI-RS. Nevertheless, within the leagcy SP CSI-RS activation/deactivation MAC CE, there exist certain fields (such as Serving Cell ID and BWP ID) that are not applicable to LTM SP CSI-RS. This would render the behavior of the UE rather complex (for instance, determining when to disregard which fields).

- Option 2: New MAC CE

In this alternative, a new MAC CE is defined specifically for the activation and deactivation of LTM SP CSI-RS. RAN2 to consider the following information into the new MAC CE:

- A/D Indication to indicate whether to activate or deactivate indicated SP CSI-RS resource set

- Candidate cell ID

- SP CSI-RS resource set ID

- TCI state for each CSI-RS resource

And provide LS to RAN1 to ask them if any other information is needed.

Evidently, from the perspective of UE behavior, Option 2 is more straightforward and thus preferable.

Consequently, the following proposals are put forward:

Proposal 5: A new MAC CE is defined to enable the activation and deactivation of the semi - persistent CSI - RS for LTM candidate cells.

Proposal 6: The new MAC CE include the following information:

- Indication of activating or deactivating of the indicated SP CSI-RS resource set

- Candidate cell ID

- SP CSI-RS resource set ID

- TCI state for each CSI-RS resource

Since the SP CSI-RS was introduced by RAN1, the solution for its activation and deactivation will have an impact on RAN1. We need to convey this agreement to RAN1.

Proposal 7: RAN2 sends LS to RAN1 to inform the agreement above.

Conclusion

According to the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: To support the semi-persistent CSI-RS of a candidate cell, RAN2 impact is expected regarding the activation/deactivation of the LTM semi-persistent CSI-RS.

Measurement report MAC CE

Proposal 1: The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'. And it should have a lower LCP than BFR MAC CE.

Proposal 2: Remove the restriction in the MAC running CR that the transmission of the event - triggered L1 measurement report MAC CE must occur only on the SpCell.

Proposal 3: A single MR MAC CE is used when multiple events trigger simultaneously.

Proposal 4: When a beam meets the leaving condition and the network configures the triggering of a Measurement Report (MR), the reported MR includes the measurement results of the beams that meet the entering condition as well as those of the other beams that do not meet the entering condition if configured.

Support of CSI-RS measurements for LTM

Proposal 5: A new MAC CE is defined to enable the activation and deactivation of the semi - persistent CSI - RS for LTM candidate cells.

Proposal 6: The new MAC CE include the following information:

- Indication of activating or deactivating of the indicated SP CSI-RS resource set

- Candidate cell ID

- SP CSI-RS resource set ID

- TCI state for each CSI-RS resource

Proposal 7: RAN2 sends LS to RAN1 to inform the agreement above.

Reference

- Revised Work Item: NR mobility enhancements Phase 4, RP-242356

- LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, R1-2409283

- Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells,R3-247911