3GPP TSG-RAN WG2 Meeting #129                                                                                   R2-2500223

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on Conditional Intra-CU LTM

Agenda Item:    8.6.4

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues on the conditional intra-CU LTM [1]. In detail, the following aspects are covered by our discussion,

- Conditional LTM preparation

- Conditional LTM early sync

- Conditional LTM evaluation

- Conditional LTM execution

The detailed discussion is provided in Section 2, and followed by Section 3 that summarizes the contribution of this paper.

Discussion

- Conditional LTM preparation

The initialization of conditional LTM preparation

The Rel-19 conditional LTM only supports intra-CU scenario. For the preparation of conditional LTM, similar as NW-triggered LTM, it should be the CU to initiate the conditional LTM preparation based on the L3 measurement.

Proposal 1: It is the CU to initiate the conditional LTM preparation.

The co-existence between conditional LTM and NW-triggered LTM

Regarding the co-existence of conditional LTM and NW-triggered LTM, from the signalling point of view, it is captured in the RRC running CR as follows,

LTM-Config-r18 ::= SEQUENCE { … ..., [[ ltm-ServingCellNoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-SK-Counters-r19 SetupRelease {SK-CounterList-r19} OPTIONAL, -- Need M cltm-ServingCellExecutionConditions-r19 SetupRelease {CLTM-ExecutionConditions-r19} OPTIONAL -- Need M ]] } LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, … ..., [[ ltm-NoSecurityChangeID-r19 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M cltm-ExecutionConditions-r19 SetupRelease {CLTM-ExecutionConditions-r19} OPTIONAL -- Need M Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations Editor’s Note: How the execution conditions for CLTM are configured is pending the progresses on the event-triggered L1 measurements objective. ]] } … CLTM-ExecutionConditions-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF ExecutionConditions-r19 ExecutionConditions-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not. ... }

As per the current signalling framework, whether the current serving cell (including the case when the candidate cell becomes the serving cell) can perform conditional LTM is indicated to UE by providing the corresponding execution condition. That means the candidate for conditional LTM is a subset of the candidate for NW triggered LTM. Besides, it seems no reason to prevent the NW to trigger LTM to a candidate cell configured for conditional LTM.

So the candidate cell configured for NW triggered LTM could also be configured for conditional LTM. And the candidate cell configured for CLTM could be used for NW triggered LTM execution too.

Proposal 2: Support the co-existence between conditional LTM and network based LTM, as implemented in the RRC running CR, i.e., a LTM candidate cell is a conditional LTM candidate cell if execution condition is provided for it, and NW could trigger LTM execution to a conditional LTM candidate cell.

Now only intra-CU conditional LTM in non-DC is supported, but the inter-CU LTM is supported in Rel-19 for network triggered LTM. How to support the co-existence between conditional LTM and network triggered LTM in Rel-19 should be discussed.

Obviously, if only intra-CU LTM candidate cell in non-DC is configured, any candidate cell can be configured for conditional LTM no matter which candidate cell is as serving cell.

If inter-CU LTM candidate cell in non-DC is configured, the candidate cells may belong to multiple CUs (i.e., mixture of intra-CU and inter-CU LTM candidates). UE may firstly perform intra-CU conditional LTM or inter-CU LTM triggered by the NW, and vice versa.

However, for conditional LTM, according to the following agreements,

To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

The execution condition is provided by the current serving cell for its corresponding candidate cells. Thus, it could be NW implementation to ensure that only the candidate cells belonging to the same CU with the current serving cell are provided with the execution condition.

For example, UE is in serving cell 0 belonging to CU1, and is configured with candidate cell 1, candidate cell 2 belonging to CU1, and candidate cell 3, candidate cell 4 belonging to CU2.

| CU | Current serving cell | Which cells can be configured as conditional LTM candidates |
| --- | --- | --- |
| CU1 | Serving cell 0 | Cell 1 and Cell 2 |
| CU1 | Cell 1 | Cell 0 and Cell 2 |
| CU1 | Cell 2 | Cell0 and Cell 1 |
| CU2 | Cell 3 | Cell 4 |
| CU2 | Cell 4 | Cell 3 |

If serving cell is Cell 0, UE only evaluates the candidate cell which is associated with the corresponding execution condition, i.e., Cell 1 and Cell 2, hence it could ensure the conditional LTM is within intra-CU.

Based on above discussion, the NW could ensure the conditional LTM is performed within one CU. If the candidate cells within one CU support the mixture of DC and non-DC candidates, the CU could be aware of whether the candidate cell is configured with DC, so it could limit the candidate cell for conditional LTM is within the non-DC candidates.

Therefore, it is proposed that,

Proposal 3: For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

Whether L1 and L3 execution conditions can be configured simultaneously

As captured in the current running CR, there is an FFS whether L1 and L3 execution conditions can be configured simultaneously for one conditional LTM candidate.

Editor’s Note: FFS whether L1 and L3 execution conditions can be configured simultaneously or not.

Firstly, L1 execution condition or L3 execution condition would be sufficient to trigger the conditional LTM, and it has no motivation to configure both simultaneously

Observation 1: There is no motivation to configure both L1 and L3 execution conditions simultaneously.

Besides, as agreed before,

For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

If one conditional LTM candidate is provided with both L1 and L3 execution conditions, additional cross-layer interaction is needed to ensure that both the L1 and L3 execution conditions are satisfied, which will complicate the UE behaviour.

Observation 2: More spec impacts (e.g., complicated UE behaviour on cross-layer interaction) are introduced if both L1 and L3 execution conditions are configured simultaneously.

So we prefer not support configuring L1 and L3 execution conditions simultaneously for one conditional LTM candidate.

Proposal 4: Do not support to configure L1 and L3 execution conditions simultaneously for one conditional LTM candidate.

How to configure the L1 execution condition for conditional LTM

In RAN2#127bis, it was agreed that,

Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

Regarding whether to reuse events specified for the event-triggered L1 measurements, it is kept as a FFS in the current running CR.

Editor’s Note: FFS whether we re-use events specified for the event-triggered L1 measurements or we specify events specifically for CLTM.

Given that the behaviour of the UE varies depending on whether the event is triggered for event - triggered L1 measurements or CLTM, for example, measurement reporting is not carried out when the event is triggered for CLTM. Thus, separate events for CLTM are necessary.

Proposal 5: Introduce condEventLTM3 and condEventLTM5 as the events specifically designated for conditional LTM.

- Conditional LTM early sync

UE behaviors to the new TA timer

Regarding the maintenance of early TA, it was agreed to use a new timer,

Candidate cell TA is maintained by a new timer.

Thus, upon receiving the early TA MAC CE, UE should start the TA timer to monitor the validation of the early TA. The NW may trigger UE to perform the early TA acquisition again and send the early TA to UE via the early TA MAC CE no matter the timer is running or not. Therefore, UE should re-start the new timer once UE acquires the new early TA sent by the NW.

Proposal 6: UE starts or re-starts the new TA timer upon receiving the early TA MAC CE.

Then another issue is how to handle the new timer upon the execution of conditional LTM.

In Rel-18 LTM, the value of early TA is delivered to UE via LTM cell switch command. Upon receiving the TA in the LTM cell switch command, the UE applies the TA value and starts the TAT associated with the corresponding PTAG. Therefore, the maintenance of the TA can be aligned among the NW and UE.

When considering conditional LTM, as suggested by our Proposal 7, UE will start the new timer upon receiving the early TA MAC CE. Therefore, upon the execution of conditional LTM, the new timer has been running for some time duration, but the target cell is not aware of such time duration. Thus how the NW and UE align the maintenance of the TA should be further considered. One way is to stop the new timer for early TA, and then start the TAT of the TAG configured by the target RRCReconfiguration associated with this early TA. By this way, the Rel-18 handling on the maintenance of TA in the current serving cell after LTM cell switch can be reused.

Besides the early TA associated with the target cell, the UE may maintain early TA for other candidate cells. If the UE keep new timer running for the other early TA, the new serving cell has no knowledge about when these TA value will be assumed to be invalid by the UE, so one simple way is to stop all running new timer for early TA upon cell switch.

Therefore, it is proposed that,

Proposal 7: Upon the handover execution (e.g., conditional LTM, NW triggered LTM execution, legacy NW triggered handover, CHO), UE stops the new timer associated with the early TA.

- Conditional LTM evaluation

The beam used for evaluation

In order to avoid ping-pong, some companies proposed to evaluate multiple beams for conditional LTM, i.e. when at least N beams fulfilled the execution condition for one candidate cell, the UE could trigger the conditional LTM execution. However, it has already agreed to support TTT for the event configuration, thus we can assume the beam quality of the candidate cell could be stable. Hence it is not necessary to evaluate multiple beams for one triggering event. The UE evaluates the execution condition for each candidate cell/beam, and when at least one of the candidate cell/beam fulfills the associated execution condition, the UE should trigger the LTM cell switch procedure.

Proposal 8: For L1 based execution condition, UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition.

The execution condition evaluation  in mTRP case

Another scenario that needs further consideration for the execution condition evaluation is the serving cell or candidate cell is configured with mTRP.

For the candidate cell configured with mTRP, since UE should evaluate the execution condition per candidate beam (as suggested by our Proposal 8), UE will trigger LTM cell switch if any beam of the candidate cell satisfies the execution condition. So there is no further issue found.

For the serving cell configured with mTRP, two TCI states may be activated simultaneously. Considering the condEventLTM3/5 need to evaluate the serving beam, which beam to be used for evaluation should be discussed.

For mTRP, it may be the intra-cell mTRP or inter-cell mTRP. For inter-cell mTRP, it should be the beam associated with the serving cell but not the beam associated with additional PCI to be used for evaluation. This is because the beam associated with the serving cell is used by UE to receive the paging or system information. This TRP could be the special TRP. To unify the solution, for intra-cell mTRP, we can also use one beam for evaluation. Then which beam to be used for evaluation should be further discussed. For intra-cell mTRP, it can be up to NW implementation to send the paging or system information by a certain TRP or both TRP. Similar as inter-cell mTRP, UE should evaluate this beam for cell switch purpose. Thus, it should be the NW to configure UE about which beam to be used for evaluation.

Proposal 9: The NW configures which activated TCI of the serving cell to be used for conditional LTM evaluation if the serving cell is configured with mTRP.

- Conditional LTM execution

Target beam determination for RACH-less conditional LTM

In the NW-based LTM, the UE is indicated with the TCI state in the LTM cell switch command MAC CE. For RACH-less LTM, the UE will use the indicated TCI state to receive the dynamic grant or to select the type-1 configured grant.

It was agreed in last meeting that the TCI activation is based on the Candidate Cell TCI States Activation/Deactivation MAC CE sent by NW.

For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration

Considering the NW could activate the candidate TCI states, we can assume that the NW ensures the UE could perform RACH-less cell switch upon these TCI states. Hence once the selected TCI state is one of the activated TCI contained in Candidate Cell TCI States Activation/Deactivation MAC CE, the UE could perform RACH-less conditional LTM.

For conditional RACH-less LTM, how to determine the selected cell and beam is based on UE. As mentioned above, for L1 based execution condition, UE evaluates the execution condition for each candidate cell/beam, and determines to trigger the cell switch upon at least one of the candidate cell/beam fulfills the associated execution condition. Thus, the candidate cell/beam which fulfills the execution condition should be the target cell/beam.

In the running CR, there is a FFS on how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution.

If more than one candidate cell/beam fulfills the execution condition, it is beneficial if UE prioritizes the candidate beam that had performed early TCI activation/early TA. By this way, UE can perform RACH-less cell switch to the target cell.

Proposal 10: For L1 based execution condition, the target beam is the beam which fulfills the associated execution condition. If more than one candidate beam fulfills the execution condition, UE prioritizes the candidate beam that had performed early TCI activation/early TA acquisition and performs RACH-less switch to the target cell.

For L3 based execution condition, if only one cell fulfills the execution condition, this cell is the target cell. If at least one of the TCI states of the target cell was activated by Candidate Cell TCI States Activation/Deactivation MAC CE, the UE should choose one of the TCI states included in the Candidate Cell TCI States Activation/Deactivation MAC CE as the target beam. If more than one cell fulfills the execution condition, and at least one of this cells has the activated TCI state by the Candidate Cell TCI States Activation/Deactivation MAC CE, UE should choose the cell with activated TCI states by the Candidate Cell TCI States Activation/Deactivation MAC CE as the target cell and select one of these activated TCI states as target beam. If no TCI state is activated before cell switch, it is up to UE to choose one candidate cell and one beam to access.

Proposal 11a: For L3 based execution condition, the target cell is the cell which fulfills the associated execution condition. If more than one candidate cell fulfills the execution condition, UE prioritizes the candidate cell that had performed early TCI activation/early TA acquisition. Otherwise, it is up to UE implementation to choose the target cell.

Proposal 11b: In the target cell, UE should prioritize the candidate beam that had performed early TCI activation/early TA to be target beam and performs the RACH-less cell switch to the target cell. Otherwise, it is up to UE implementation to choose the target beam.

Whether to support RACH-less C-LTM with DG

As agreed in last meeting, the NW can trigger candidate TCI activation/deactivation or PDCCH ordered early TA based on the L1/L3 measurement reporting by UE,

The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

That means the NW has the potential beam information selected by UE to be used in the target cell in RACH-less conditional LTM. Therefore, to facilitate the efficient scheduling by the target cell, the source cell can send such assistance information to the target cell for the CG/DG scheduling. Hence it is feasible for target cell to start DG scheduling upon reception of the assistance information from the source cell.

Regarding the detailed assistance information, it can be the candidate TCI state(s) pre-activated by the NW or up to NW implementation to send some good-enough candidate beams based on the L1/L3 measurement reporting by UE.

Proposal 12a: RACH-less conditional LTM with DG is supported.

Proposal 12b: The source cell can send the candidate beam assistance information to the target cell to facilitate the CG/DG scheduling by the target cell.

Conclusion

Based on the analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: There is no motivation to configure both L1 and L3 execution conditions simultaneously.

Observation 2: More spec impacts (e.g., complicated UE behaviour on cross-layer interaction) are introduced if both L1 and L3 execution conditions are configured simultaneously.

Conditional LTM preparation

Proposal 1: It is the CU to initiate the conditional LTM preparation.

Proposal 2: Support the co-existence between conditional LTM and network based LTM, as implemented in the RRC running CR, i.e., a LTM candidate cell is a conditional LTM candidate cell if execution condition is provided for it, and NW could trigger LTM execution to a conditional LTM candidate cell.

Proposal 3: For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

Proposal 4: Do not support to configure L1 and L3 execution conditions simultaneously for one conditional LTM candidate.

Proposal 5: Introduce condEventLTM3 and condEventLTM5 as the events specifically designated for conditional LTM.

Conditional LTM early sync

Proposal 6: UE starts or re-starts the new TA timer upon receiving the early TA MAC CE.

Proposal 7: Upon the handover execution (e.g., conditional LTM, NW triggered LTM execution, legacy NW triggered handover, CHO), UE stops the new timer associated with the early TA.

Conditional LTM evaluation

Proposal 8: For L1 based execution condition, UE evaluates the execution condition for each candidate beam and triggers the LTM cell switch procedure when at least one beam fulfills the associated execution condition.

Proposal 9: The NW configures which activated TCI of the serving cell to be used for conditional LTM evaluation if the serving cell is configured with mTRP.

Conditional LTM execution

Proposal 10: For L1 based execution condition, the target beam is the beam which fulfills the associated execution condition. If more than one candidate beam fulfills the execution condition, UE prioritizes the candidate beam that had performed early TCI activation/early TA acquisition and performs RACH-less switch to the target cell.

Proposal 11a: For L3 based execution condition, the target cell is the cell which fulfills the associated execution condition. If more than one candidate cell fulfills the execution condition, UE prioritizes the candidate cell that had performed early TCI activation/early TA acquisition. Otherwise, it is up to UE implementation to choose the target cell.

Proposal 11b: In the target cell, UE should prioritize the candidate beam that had performed early TCI activation/early TA to be target beam and performs the RACH-less cell switch to the target cell. Otherwise, it is up to UE implementation to choose the target beam.

Proposal 12a: RACH-less conditional LTM with DG is supported.

Proposal 12b: The source cell can send the candidate beam assistance information to the target cell to facilitate the CG/DG scheduling by the target cell.

Reference

[1] Draft_RAN2_128_Meeting_Report_v1