3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500226

Athens, Greece, 17th – 21st Feb, 2025

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on case 2. The contribution is organized as follows.

In section 2, we discuss the following aspects,

- The procedure for cell selection in RLF case and other cases

- Validity of WUS configuration

- The details in Msg.1 based RA procedure

- SIB1 transmission for emergency case

- Unnecessary Paging for legacy UE

- WUS configuration content

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - The procedure for cell selection

For RLF case

RAN#106 RP-243297: On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

Based on above RAN#106 meeting conclusion for UE in RRC Connected mode, if RLF is declared, the UE could request the OD-SIB1 transmission if target NES Cell is selected for the RRC reestablishment procedure.  The details could be further discussion in stage 3.

Proposal 1: For RLF case, UE could request the OD-SIB1 transmission on the selected cell if the selected cell is OD-SIB1 enabled NES cell and UE has acquired the valid WUS configuration of the selected cell, the details could be FFS in stage 3.

Furthermore, RAN2 needs to further discuss whether and how the UE in RRC Connected mode to acquire the WUS configuration in NES Cell.

For UE from Idle/Inactive mode to RRC Connected mode in NES Cell, the network is not sure whether the new agreed SIBx for WUS configuration is valid or not at UE side. There are two options for this issue. The option1 is, the UE could request on SIBx by enhanced dedicated SIB request message, such as to enhance DedicatedSIBRequest message, to obtain the valid WUS configuration. The option 2 is, the enhanced request message for SIBx is not supported, when RLF is declared, the UE could request the OD-SIB1 transmission if SIBx is valid in RRC reestablishment procedure, otherwise, the UE will follow the legacy RRC reestablishment procedure to try to read the OD-SIB1 if available, and consider it as barred if OD-SIB1could not be read out. In option 2, the network will try its best to actively transmit the WUS configuration to UE once UE moves in RRC Connected mode, which could reduce the impact from invalid WUS configuration. RAN2 needs to decide which option is applied for UE in RRC connected mode.

Proposal 2: Two options as below needs to be down-selected on WUS configuration for UE in RRC connected mode：

- Option 1： UE relies on network to provide the latest SIBx via RRC message in RRC connected mode, i.e., when RLF is declared, UE could request the OD-SIB1 transmission if UE has stored valid WUS configuration of the selected OD-SIB1 enabled NES cell, otherwise, UE will follow the legacy UE’s procedure;

- Option 2： UE in RRC connected mode could request the SIBx including WUS configuration by an enhanced RRC dedicated message.

For other cases

Based on current discussion, R19 NES UE is allowed to perform cell reselection and camp on NES Cell. However, R19 NES UE may power on and perform cell selection at a NES Cell which is transmitting the OD-SIB1 triggered by other UEs or broadcasting periodic SIB1 message, the R19 NES UE behavior should be defined if it could camp on NES Cell. In our view, even it is a corner case and the UE cannot trigger the WUS, but the NES UE could read out the OD-SIB1 triggered by other UEs, so the UE is able to camp on the NES Cell if allowed.

Proposal 3: In cell selection cases other than RLF case, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

  - Validity of WUS configuration

RAN2#128 meeting agreement: UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

In last meeting, there is an FFS issue on WUS configuration validity. A new SIBx has been introduced to include WUS configuration. Since the legacy SIB could be configured as valid within a SI validity area, it natural to configure the new SIBx validity for a SI validity area.  Then, UE could check the stored SIBx is valid or not based on systemInformationAreaID as in legacy procedure. UE considers WUS configuration valid in directly succeeding cell reselection from cell where UE camped and stored the valid SIBx for WUS configuration

Besides, considering the RLF handling, the UE needs to perform the cell selection to find a suitable cell for RRC re-establishment procedure. It is possible that the selected cell is NES Cell (also including the cell with RLF occurring), based on RAN#106 meeting conclusion, UE will consider the WUS configuration valid in the selected cell if it has the valid WUS configuration of the selected cell in the cell with RLF occurring, and trigger the OD-SIB1 requiring procedure.

Proposal 4: The new SIB for WUS configuration could be valid within a SI validity area.

Proposal 5: UE considers WUS configuration valid in directly succeeding cell reselection from cell where UE has valid WUS configuration for the reselected cell, or in the cell selected for RRC re-establishment procedure if UE has the valid WUS configuration of the selected cell in the cell with RLF occurring.

  - The details in Msg.1 based RA procedure

In RAN1#119 meeting, RAN1 have agreed that the window starting time for on-demand SIB1 is based on the RAR window, the  duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

RAN1#119 meeting Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

One remaining issue is how the UE obtain the time information of OD-SIB1 transmission. The latest RAN1 agreements on OD-SIB1 monitoring is above, it seems that the related configuration on OD-SIB monitoring will be included in WUS configuration, but the details are FFS and still under RAN1 discussion, So, it is suggested to wait RAN1 conclusion before we start any discussion on the starting time of PDCCH monitoring occasions for OD-SIB1.

Proposal 6: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

  - SIB1 transmission for emergency case

According to legacy procedure, if UE is ETWS capable or CMAS capable and receives the etwsAndCmasIndication bit in the Short Message, it will re-acquire the SIB1 immediately.

Observation 1: In legacy, UE needs to acquire the SIB1 immediately when the etwsAndCmasIndication bit of Short Message is set, which is beneficial for latency.

In this case, the network will change to normal mode other than NES mode, or the network will broadcast the SIB periodically as agreed in last meeting, the NES UE could receive the SIB1 immediately without any delay. Then, there is on issue is explored, it could be considered as network implementation.

Proposal 7a: In emergency cases, it is network implementation to switch to normal mode or broadcast the SIB periodically.

Proposal 7b: UE acquires the SIB1 immediately on OD-SIB1 enabled NES cell without sending WUS request when the etwsAndCmasIndication bit of Short Message is set.

  - Unnecessary Paging for legacy UE

RAN2#127bis meeting agreement: 9. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. RAN2#128 meeting agreement: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

RAN2 has agreed several methods as above to make legacy UE not camp on OD-SIB1 Cell, then the paging for legacy UE seems not necessary at NES Cell. It is also network power saving if the paging for legacy UE is not transmitted at NES Cell. Hence, considering the legacy UE is not allowed to camp on NES Cell, also from the view of network power saving, it is proposed the NES Cell will not transmit the legacy UE paging message.

Proposal 8: The paging message for legacy UE is not transmitted at OD-SIB1 Cell.

  - WUS configuration content

Obviously, the WUS configuration needs to include the RA preamble, the RA resource, the parameters used for RA procedure, such as the maximum number of Random Access Preamble transmission preambleTransMax, the parameters on power ramping, the RAR window parameter ra-ResponseWindow and so on, some of them has been discussed and agreed by RAN1#118bis and RAN1#119 meeting as in below table.

RAN2#119 meeting Agreement The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration. Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time RAN2#118bis meeting Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

We could wait on RAN1 more progress and then discuss the details in WUS configuration.

Proposal 9: For the WUS configuration parameters of a NES cell, RAN2 waits further RAN1 progress.

Since Cell A/NES Cell may assist more than one neighbor NES Cells, the neighbor NES Cells have better not use the same WUS configuration to avoid the WUS transmission conflict. So, it can be seen that the WUS configuration format in Cell A/NES Cell could be a list including one or more WUS configuration, each WUS configuration is associated to one or more cells, each WUS configuration is cell level rather than validity area level.

Proposal 10: The WUS configuration in Cell A/NES Cell consists of a list including one or more WUS configuration, and each WUS configuration can associated to one or more cells.

Conclusion

According to the analysis in section 2, our proposals are summarized as follows:

The procedure for cell selection

Proposal 1: For RLF case, UE could request the OD-SIB1 transmission on the selected cell if the selected cell is OD-SIB1 enabled NES cell and UE has acquired the valid WUS configuration of the selected cell, the details could be FFS in stage 3.

Proposal 2: Two options as below needs to be down-selected on WUS configuration for UE in RRC connected mode：

- Option 1： UE relies on network to provide the latest SIBx via RRC message in RRC connected mode, i.e., when RLF is declared, UE could request the OD-SIB1 transmission if UE has stored valid WUS configuration of the selected OD-SIB1 enabled NES cell, otherwise, UE will follow the legacy UE’s procedure;

- Option 2： UE in RRC connected mode could request the SIBx including WUS configuration by an enhanced RRC dedicated message.

Proposal 3: In cell selection cases other than RLF case, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

Validity of WUS configuration

Proposal 4: The new SIB for WUS configuration could be valid within a SI validity area.

Proposal 5: UE considers WUS configuration valid in directly succeeding cell reselection from cell where UE has valid WUS configuration for the reselected cell, or in the cell selected for RRC re-establishment procedure if UE has the valid WUS configuration of the selected cell in the cell with RLF occurring.

The details in Msg.1 based RA procedure

Proposal 6: RAN2 waits RAN1 conclusion on how to determine the window starting time of the PDCCH monitoring occasions for on demand SIB1.

SIB1 transmission for emergency case

Observation 1: In legacy, UE needs to acquire the SIB1 immediately when the etwsAndCmasIndication bit of Short Message is set, which is beneficial for latency.

Proposal 7a: In emergency cases, it is network implementation to switch to normal mode or broadcast the SIB periodically.

Proposal 7b: UE acquires the SIB1 immediately on OD-SIB1 enabled NES cell without sending WUS request when the etwsAndCmasIndication bit of Short Message is set.

Unnecessary Paging for legacy UE

Proposal 8: The paging message for legacy UE is not transmitted at OD-SIB1 Cell.

WUS configuration content

Proposal 9: For the WUS configuration parameters of a NES cell, RAN2 waits further RAN1 progress.

Proposal 10: The WUS configuration in Cell A/NES Cell consists of a list including one or more WUS configuration, and each WUS configuration can associated to one or more cells.

Reference

[1] Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES, Samsung, RAN2#128 meeting.