3GPP TSG-RAN WG2 Meeting #129                                          R2-2500231

Athens, Greece, Feb. 17th – 21st, 2025

Source:        CATT, Qualcomm Inc., CMCC

Title:             Clarification on the content of event Hy measurement report

Agenda Item:	7.0.2.3

Document for:	Discussion and Decision

Introduction

As specified in TS 38.300 as below, it is stated that “RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured”. But after checking, this description is not accurate, and actually misleading from testing perspective, as it may lead to the confusion that the measurement results of both serving cells and neighbour cells are always included in a measurement report triggered by event H1/H2. In this paper, we further clarify the content of eventHy measurement report.

### 16.18.3 Altitude-based Reporting for Aerial UE Communication An Aerial UE can be configured with altitude-dependent, event-based measurement reporting (i.e., eventH1 and eventH2 as defined in TS 38.331 [12]). An Aerial UE sends a measurement report when its altitude becomes higher or lower than configured threshold. The UE includes its altitude and location information in the measurement report if configured to do so by NG-RAN. RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured. The Aerial UE can also be configured to trigger measurement reporting only when both an altitude-dependent condition and an RSRP/RSRQ/SINR-based condition are met (i.e., eventA3H1, eventA3H2, eventA4H1, eventA4H2, eventA5H1 and eventA5H2 in TS 38.331 [12], commonly denoted as eventAxHy). For the content of eventAxHy measurement report, the same rules as described above for eventH1 and eventH2 apply.

Discussion

2.1 Measurement results of serving cell(s)

According to the UE behavior specified in 5.5.5.1 of TS 38.331, after a measurement reporting procedure is triggered, “for each serving cell configured with servingCellMO” the measResultServingCell will be included, the detail is dependent on the rsType configuration.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows: 1> set the measId to the measurement identity that triggered the measurement reporting; 1> for each serving cell configured with servingCellMO: 2> if the reportConfig associated with the measId that triggered the measurement reporting includes rsType: 3> if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report; 2> else: 3> if SSB based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on SSB; 3> else if CSI-RS based serving cell measurements are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on CSI-RS;

Regarding whether a serving cell is configured with servingCellMO, the following restriction has been specified in TS 38.331. And it’s mandatory for the SpCell assuming a NR UE always has a measConfig, but optional for SCells.

ServingCellConfig ::=               SEQUENCE {

<Unnecessary part omitted>

servingCellMO              MeasObjectId         OPTIONAL,   -- Cond MeasObject

<Unnecessary part omitted>

}

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. For (e)RedCap UEs, this field is optionally present, Need M. |

Therefore, considering for a SpCell at least SSB based measurements are always available, the measResultServingCell for a SpCell should be always included in a measurement report. But for SCells, when servingCellMO is configured and measurements results are available the measResultServingCell for a SCell can be included in a measurement report.

Observation 1: In a measurement report, the measResultServingCell for a SpCell is always included.

Observation 2: In a measurement report, the measResultServingCell for a SCell can be included in a measurement report when servingCellMO is configured and measurements results are available.

2.2 Measurement results of neighbor cell(s)

In a measurement report, for each configured servingCellMO other than the measObjectId corresponding with the measId that triggered the measurement report, the measResultBestNeighCell can be included when the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas.

1> if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas: 2> for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting: 3> if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig: 4> set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR; 4> if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> for each best non-serving cell included in the measurement report: 6> include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

Observation 3: In a measurement report, for each configured servingCellMO other than the measObjectId corresponding with the measId that triggered the measurement report, the measResultBestNeighCell can be included when the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas.

Regarding whether to include the measurement results of other neighbor cells in a measurement report, it has also been captured in 5.5.5.1 of TS 38.331 as below. It seems that the yellow highlighted “5> else” is applicable for event H1/H2 (then the measurement results of some neighbouring cells can be included in the measurement report), but after detailed checking, since there is no “applicable neighbouring cell” defined for event H1/H2 the measResultNeighCells will not be included in a measurement report triggered by event H1/H2.

1> if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report: 2> if the reportType is set to eventTriggered or periodical: 3> if the measurement report concerns the candidate L2 U2N Relay UE: <Unnecessary part omitted> 3> else: 4> set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following: 5> if the reportType is set to eventTriggered and eventId is not set to eventD1 or eventD2 or eventH1 or eventH2: 6> include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

Observation 4: In a measurement report triggered by event H1/H2, the measResultNeighCells will not be included since there is no “applicable neighbouring cell” defined for event H1/H2.

Based on the analysis above, after a measurement reporting procedure is triggered by event H1/H2, in the corresponding measurement report:

- the measResultServingCell for a SpCell is always included.

- the measResultServingCell for a SCell can be included in a measurement report when servingCellMO is configured and measurements results are available.

- for each configured servingCellMO other than the measObjectId corresponding with the measId that triggered the measurement report, the measResultBestNeighCell can be included when the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas.

- the measResultNeighCells will not be included since there is no “applicable neighbouring cell” defined for event H1/H2.

Proposal 1: RAN2 to confirm in a measurement report triggered by event H1/H2:

- The measResultServingCell for a SpCell is always included.

- The measResultServingCell for a SCell may be included.

- For each servingCellMO, the measResultBestNeighCell may be included.

- For other neighbour cells, the measResultNeighCells is not included since there is no “applicable neighbouring cell” defined for event H1/H2.

2.3 Correction CR for TS 38.300

If Proposal 1 can be agreed in RAN2, it also means the description “RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured” captured in TS 38.300 is not correct. As it’s only true for a SpCell, and for other serving cells and neighbor cells corresponding to the same servingCellMO it’s optional to include their measurement results in a measurement report. For all other neighbor cells, the measResultNeighCells of them will not be included.

Proposal 2: remove “RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured” in 16.18.3 of TS 38.300.

Conclusion

According to above discussion on the content of measurement report triggered by event H1/H2, we have the following observations:

Observation 1: In a measurement report, the measResultServingCell for a SpCell is always included.

Observation 2: In a measurement report, the measResultServingCell for a SCell can be included in a measurement report when servingCellMO is configured and measurements results are available.

Observation 3: In a measurement report, for each configured servingCellMO other than the measObjectId corresponding with the measId that triggered the measurement report, the measResultBestNeighCell can be included when the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas.

Observation 4: In a measurement report triggered by event H1/H2, the measResultNeighCells will not be included since there is no “applicable neighbouring cell” defined for event H1/H2.

And we propose:

Proposal 1: RAN2 to confirm in a measurement report triggered by event H1/H2:

- The measResultServingCell for a SpCell is always included.

- The measResultServingCell for a SCell may be included.

- For each servingCellMO, the measResultBestNeighCell may be included.

- For other neighbour cells, the measResultNeighCells is not included since there is no “applicable neighbouring cell” defined for event H1/H2.

Proposal 2: remove “RSRP/RSRQ/SINR measurement results are always reported when height reporting is configured” in 16.18.3 of TS 38.300.

Reference

- TS 38.300 v18.4.0.

- TS 38.331 v18.4.0