3GPP TSG-RAN WG2 Meeting #129                                                     R2-2500234

Athens, GR, Feb 17th – 21st, 2025

Source:	CATT

Title:	MRO Enhancements for LTM

Agenda Item:	8.10.2.1

Document for:	Discussion and Decision

1. Introduction

In RAN2#128 meeting, MRO enhancements for LTM was discussed and the following agreements were achieved [1].

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

In this contribution, we further discuss the issues about MRO enhancements for LTM.

2. Discussion

2.1 Enhance RLF report for LTM MRO

In LTM, if the UE can acquire the valid TA value (included in LTM cell switch command or measured by UE itself) when receiving the LTM cell switch command, UE will perform RACH-less LTM execution. For RACH-less LTM, both dynamic grant and configured grant can be used, and the UE will apply the TCI state indicated in the LTM cell switch command for dynamic grant reception and configured grant resource selection. If RACH-less LTM fails, it may be because the TCI state indicated in LTM cell switch command is inappropriate, then the UE needs to log the TCI state ID indicated in the LTM cell switch command in RLF report to help the network to optimize and select a suitable TCI state to avoid the RACH-less LTM failure.

Proposal 1: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

2.2 Enhance SHR for LTM MRO

For successful LTM execution but it’s still close to failure, whether legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM should be discussed. From our perspective, the T310 threshold and T304 threshold trigger conditions can be reused for LTM, but for T312 threshold trigger condition, considering that T312 is configured in MeasObjectNR, i.e., L3 RRM framework, it is not sure whether it is applicable for DU triggered handover.

Proposal 2: The legacy SHR trigger conditions T310 threshold and T304 threshold can be applied to LTM.

SHR should be enhanced to record the successful LTM information. Similar as L3 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L3 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

2.3 Enhance RACH report for LTM

RAN2 has agreed to enhance RACH report for MCG LTM SON, raPurpose should be enhanced to cover successful RACH-based LTM execution, the detail can be discussed further, e.g., whether legacy raPurpose can be reused or a new raPurpose for LTM execution needs to be introduced.

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

For the TA value in LTM cell switch command (network-based TA acquisition), the early RACH procedure is needed. The source cell needs to send the PDCCH order command to UE, then the UE sends the preamble towards the indicated candidate cell. After the candidate cell acquires the TA value when receives the preamble from UE, it sends the TA value to the source cell and source cell includes the TA value in the LTM cell switch command to UE. The RACH procedure can be triggered multiple times towards same or different candidate cells before receiving LTM cell switch command. A new raPurpose can be introduced to indicate the network-based early TA acquisition.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Based on current spec, the UE may include up to 8 RACH report in RACH report list. If the RACH report is enhanced to support logging the RACH information of network-based early TA acquisition, the RACH report entry will reach max number (i.e., maxRAReport) soon as the frequent network-based early TA acquisition, this will affect the legacy RACH information logging, e.g., RACH information of handover as the UE will stop the RACH report logging if the entry number reaches max number, which will cause the legacy RACH information loss and this is not expected as the legacy RACH information is important for RACH optimization.

5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall: 1> if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or 1> if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report: 2> clear the information included in VarRA-Report; 1> if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport: 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report: 3> append the following contents associated to the random-access procedure as a new entry in the VarRA-Report: <omit part>

RAN2 needs to discuss how to log the RACH information of network-based early TA acquisition and avoid the impact on the legacy RACH information logging. Some solutions can be considered to address the issue, e.g., one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before one LTM execution or one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before LTM configuration release or log the RACH information of network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

Proposal 6: RAN2 to discuss how to differentiate RACH information logging of network-based early TA acquisition from the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of all network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

2.4 Other issues

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and may cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 7: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

In Rel-18 mobility WID, L3 measurement based LTM is supported, i.e., the CU can request the DU to send MAC CE to trigger LTM execution based on UE layer 3 measurements. If L3 measurement based LTM execution failure/near failure, it should be the source CU to analyze and optimize the failure/near failure instead of source DU. However, considering that the source CU cannot know whether it is L3 measurement based LTM or L1 measurement based LTM execution when the source CU receives the RLF report/SHR, the source CU can send the RLF report/SHR to source DU by F1 interface to optimize the failure/near failure, but, the handover decision mechanism of the source DU may not be problematic. In order to address the issue, the indication information can be needed to be included in RLF report/SHR to indicate L1 measurement based LTM or L3 measurement based LTM to avoid the error optimization occurred.

Proposal 8: Indication information is needed in RLF report/SHR to indicate L1 measurement based LTM or L3 measurement based LTM.

3. Conclusion

According to the discussion in section 2, we propose:

Enhance RLF report for LTM MRO

Proposal 1: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Enhance SHR for LTM MRO

Proposal 2: The legacy SHR trigger conditions T310 threshold and T304 threshold can be applied to LTM.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Enhance RACH report for LTM

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 6: RAN2 to discuss how to differentiate RACH information logging of network-based early TA acquisition from the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of all network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

Other issues

Proposal 7: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 8: Indication information is needed in RLF report/SHR to indicate L1 measurement based LTM or L3 measurement based LTM.

4. Reference

- R2-2500002, RAN2#128 Meeting Report